### Lake Kampeska Surface Water Hydrology

#### By John R. Little, Hydrologist, PE

Prepared in cooperation with the

### Lake Kampeska Water Project District

##### Watertown, South Dakota

April 2000

CONTENTS

#### Introduction and Scope								5

Water Level Data									5

Water Temperature Data								6

Flood of 1916		7

Facts											8

Newspaper articles documenting flood data						15

References										99

ILLUSTRATIONS

#### Figure 1.  Map showing study area and data collection site				4

#### Figure 2.  Schematic showing a cross-section of the control weir			11

Figure 3.  Graph showing elevation vs. capacity relationship			12

Figure 4.  Graph showing Big Sioux River at Inlet/Outlet elevation

vs. discharge relationship						13

Figure 5.  Graph showing water level data, 1928-99					16

Figure 6.  Graph showing water level data, 1928-38					17

Figure 7.  Graph showing water level data, 1942-71					18

Figure 8.  Graph showing water level data, 1972-99					19

Figure 9.  Graph showing water level data, 1997					20

Figure 10.  Graph showing water temperature data, 1980-99				21

TABLES

Table 1. Flood frequency probabilities						10

Table 2. Ice out and freeze up dates							14

Table 4. Water level data, 1881-1999	and water temperature data, 1980-99		22

#### LAKE KAMPESKA SURFACE WATER HYDROLOGY

Introduction and Scope

Lake Kampeska is a highly developed natural glacial lake of 4,817 acres located in Watertown in northeastern South Dakota.  The lake is connected to the Big Sioux River by a short man made channel which serves as its inlet/outlet.  A sheet-pile weir in the Big Sioux River immediately downstream from the inlet/outlet channel controls the elevation of the lake.  The drainage area contiguous to the lake is 28.8 square miles.  The Big Sioux River upstream from the inlet/outlet has a contributing drainage area of 492 square miles; this includes the Still Lake basin, which was noncontributing prior to 1994.  The lake, via the inlet/outlet serves as a buffer, which protects the city from floods; thus, sediment that is carried into the lake by the floodwater settles out before the water returns slowly back to the river.

The water levels of Lake Kampeska have been collected and recorded by the Watertown Utilities Department since 1928.  This report is a compilation of all of the water-level data available from the Utilities Department and some miscellaneous data documenting floods taken from articles in the “Watertown Public Opinion”.  Also included are tables and figures containing flood-frequency probabilities, an elevation/capacity curve, a cross-section of the control structure, and other data describing the hydrology of the lake.

Water Level Data

#### The water-level data compiled in table 4 of this report were collected by employees of the Watertown Utilities Department at the Water Filtration Plant at 336 South Lake Drive.  According to Harlan Hoy, Chief Operator of the Water Treatment Plant (verbal communication, March 27, 1998) the measuring point in the pumping plant has remained unchanged since the building was built in 1888.  The measuring point is the top of a one-inch pipe in the concrete floor of the pump house.  The measuring point is over the cistern that receives water from the intake to the lake (Jeff DeVille, Forman of the Water Treatment Plant, verbal communication, March 9, 1999).  The elevation of the measuring point is 1,721.71 feet above sea level, current datum.

#### L. D. Lyon, who was the Editor of the “Watertown Public Opinion” and had become Superintendent of Waterworks by 1895, collected the data documenting the floods of 1881, 1893 and 1897 (Electricity, Water, Gas, Steam Heat in Watertown South Dakota, P. J. Albrook, August 1975).  The data were found in 1893 and 1897 “Watertown Public Opinion” articles.  In an article in the April 2, 1897 edition, L. D. Lyon, describing the 1897 flood at Lake Kampeska said:  “The records of my office date back to Dec. 1, 1891, since which time we have the heights of the water in the lake the first of every month”.  The author has inquired about these data at the Utilities Department, the City

#### Engineers Office, the Public Library and the Codington County Historical Society without success.

The data from January 1928 through December 1944 and January 1946 through December 1968 are from (Barari, 1971).  They are monthly values and are shown in Table 4 as having been collected on the first of the month at midnight.  There are a few months missing in 1934-38, 1942-44, 1949, 1961, 1963 and 1966.  There are no data available from 1939 through 1941.

The 1945 and 1969 through 1999 data are from the Utilities Department’s raw data sheets.  The 1970-74, 1976-77, and 1979 data are the highest and lowest values for each month and are shown in Table 4 as having been collected at midnight on the first and fifteenth.

The 1969, 1975, 1978 and 1980-99 data are shown in Table 4 as recorded.

All of the water-level data shown in this report have been converted to the datum in use since November 25, 1984.  At this datum the water-surface elevation at “full” is 1,717.8 feet above sea level.

The water-level of Lake Kampeska ranged from 1,710.0 feet above sea level in 1934 to 1,723.7 feet in 1897 and 1997.

Water Temperature Data

The Watertown Utilities Department at the Filtration Plant collected the water-temperature data, 1980 through 1999.  The water-temperature sensor is in the cistern near the water-level measuring point.  The temperature of the water in the lake ranged from 32 degrees Fahrenheit in 1999 to 81 degrees in 1988.

###### Flood of 1916

An article in the March 31, 1916 edition of the Watertown Public Opinion documents a flood in progress.  It says Lake Kampeska has risen 39 inches in the past 15 days and is “practically stationary now”.  Also “many older residents say the lake has never been so high since 1980”.

There is no doubt this was a major event, but because this was the only article describing the flood event in the newspapers which was daily at the time; and there was no information actually documenting the elevation; the author doubts that it was higher than the floods of 1881 or 1897, but it may have been in the neighborhood of the 1893 flood.

Lake Kampeska Facts

Water surface elevation (WSE) at full:  1,717.8 feet above sea level, new datum, South Dakota Department of Water & Natural Resources (DWNR), March 1981.

WSE of Ordinary High Water Mark set by DWNR at 1,718.3 feet, new datum, March 1981.

Datum correction to be applied to Watertown Utilities Department (WUD) data prior to November 25, 1984 is (-2.53 feet); therefore, WSE at full, old datum, is 1,720.3 feet, WUD, 1984.

Datum of WUD gage at water plant is 1,717.8 feet, new datum, at full is zero inches.

Datum of US Geological Survey (USGS) gage at Inlet/Outlet is 1,697.89 feet, USGS.

Elevation, set by DWNR in 1981, of the crest of weir in Big Sioux River just downstream from the Inlet/Outlet is 1,717.8 feet; over time notches have been cut to lower the crest.

WSE, 1881 maximum:  highest stage known; T. N. Babcock, 1943, Watertown Public Opinion (WPO); L. D. Lyon, WPO, 1897.

WSE, 1897 maximum:  1,723.7 feet, April 1, L. D. Lyon, WPO, 1897.

WSE, 1997 maximum:  1,723.7 feet, April 6, WUD; 1,723.67 feet, April 6, USGS.

WSE, 1893 maximum:  1,722.5 feet; April 8, L. D. Lyon, WPO, 1897.

WSE, 1943 maximum:  1,721.3 feet, April 1, WPO, 1943.

WSE, 1969 maximum:  1,721.2 feet, April 10, WUD.

WSE, 1986 maximum:  1,720.9 feet, April 4, WUD.

WSE, 1993 maximum:  1,720.9 feet, July 27, WUD.

WSE, 1979 maximum:  1,720.3 feet, April 15, WUD.

WSE, 1994 maximum:  1,720.1 feet, March 22, WUD; 1,720.18 feet, March 23, USGS.

WSE, 1985 maximum:  1,720.0 feet, March 23, WUD.

WSE, 1995 maximum:  1,719.9 feet, April 20, WUD; 1719.92 feet, April 21, USGS.

WSE, 1984 maximum:  1,719.8 feet, June 21, WUD.

WSE, 1972 maximum:  1,719.7 feet, June 1, WUD.

WSE, 1991 maximum:  1,719.5 feet, August 11, WUD.

WSE, 1952 maximum:  1,719.3 feet, April 10, WUD.

WSE, 1934 minimum:  1,710.0 feet, J. T. Banner & Associates, Inc., 1965, Appendix D, page 81.

Depth, maximum at full:  16 feet, USGS, 1994.

Depth, average at full:  10 feet, DWNR, 1985.

Depth of sediment maximum:  17.9 feet, USGS, 1994.

Lake Kampeska Facts, continued

Shoreline length at full:  13.5 miles, DWNR,1985.

Volume at full:  51,000 acre-feet, Corps of Engineers, 1994.

Area of water surface at full: 4,817 acres, 7.53 square miles (sq mi), Department of Environment & Natural Resources, (DENR) 1994.

Area of basin contiguous to lake:  28.8 sq mi, USGS 1998.

Area of basin, contributing, including Still Lake basin:  492 sq mi, J. R. Little, 1996.

Area of basin, contributing, excluding Still Lake basin:  279 sq mi, J.R. Little, 1996.

Flow estimate, aquifer to lake: two cubic feet per second, 1985 data, J. R. Little, 1995.

Precipitation at Watertown Airport:  22.33 inches per year, 1951-80, US Department of		 Commerce, 1980.

Lake evaporation in Watertown area:  34 inches per year, 1946-55, SDSU Bulletin 582,		 1971.

Table 1.

Lake Kampeska

Flood Frequency Probabilities

Return Interval	  Lake Elevation

(feet above MSL)

10-Year	        1,720.2

50-Year	        1,722.8

100-Year	        1,724.0

500-Year	        1,727.0

US Army Corps of Engineers, “ENGINEERING DIVISION TECHNICAL REPORT”, February 2000

Table 2.

Lake Kampeska

Ice Out and Freeze Up Dates

Ice Out			       Freeze Up

## April 20, 1969			November 18, 1969

April 16, 1970			November 19, 1970

April 10, 1971			November 26, 1971

April 17, 1972			November 17, 1972

March 27, 1973		November 11, 1973

April 10, 1974			November 29, 1974

April 26, 1975			November 21, 1975

April 1, 1976			November 27, 1976

April 8, 1977			November 14, 1977

April 16, 1978			November 17, 1978

April 23, 1979			November 10, 1979

April 8, 1980			November 24, 1980

March 16, 1981		November 24, 1981

April 16, 1982			November 10, 1982

March 30, 1983		November 24, 1983

April 13, 1984			November 18, 1984

April 10, 1985			November 21, 1985

November 12, 1986

March 21, 1987		December 2, 1987

March 28, 1988		November 17, 1988

April 17, 1989			November 16, 1989

March 29, 1990		November 27, 1990

April 1, 1991			November 3, 1991

March 21, 1992		November 13, 1992

April 19, 1993			November 24, 1993

April 15, 1994			December 2, 1994

April 15, 1995			December 2, 1995

April 23, 1996			November 12, 1996

April 29, 1997			November 17, 1997

April 11, 1998			December 19, 1998

March 30, 1999		December 15, 1999

# March 8, 2000

# Watertown Utilities Department data.

Table 2.

Table 4.  Lake Kampeska Water Levels and Water Temperature.

Water	    Water

Levels	Temperature

Date			(feet)     (degrees F)

| spring,1881 | highest |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| April 1, 1893 | 1,719.0 |  |  |  |  |  |  |
| April 8, 1893 | 1,722.5 |  |  |  |  |  |  |
| March 1897 | 1,717.9 |  |  |  |  |  |  |
| March 28,1897 | 1,718.7 |  |  |  |  |  |  |
| April 1,1897 12:00 | 1,723.7 |  |  |  |  |  |  |
| 1/1/28 0:00 | 1,717.7 |  |  |  |  |  |  |
| 2/1/28 0:00 | 1,718.0 |  |  |  |  |  |  |
| 3/1/28 0:00 | 1,718.3 |  |  |  |  |  |  |
| 4/1/28 0:00 | 1,718.0 |  |  |  |  |  |  |
| 5/1/28 0:00 | 1,717.5 |  |  |  |  |  |  |
| 6/1/28 0:00 | 1,717.0 |  |  |  |  |  |  |
| 7/1/28 0:00 | 1,716.8 |  |  |  |  |  |  |
| 8/1/28 0:00 | 1,716.7 |  |  |  |  |  |  |
| 9/1/28 0:00 | 1,716.3 |  |  |  |  |  |  |
| 10/1/28 0:00 | 1,716.2 |  |  |  |  |  |  |
| 11/1/28 0:00 | 1,716.1 |  |  |  |  |  |  |
| 12/1/28 0:00 | 1,716.0 |  |  |  |  |  |  |
| 1/1/29 0:00 | 1,716.0 |  |  |  |  |  |  |
| 2/1/29 0:00 | 1,716.4 |  |  |  |  |  |  |
| 3/1/29 0:00 | 1,718.5 |  |  |  |  |  |  |
| 4/1/29 0:00 | 1,718.3 |  |  |  |  |  |  |
| 5/1/29 0:00 | 1,717.8 |  |  |  |  |  |  |
| 6/1/29 0:00 | 1,717.3 |  |  |  |  |  |  |
| 7/1/29 0:00 | 1,716.6 |  |  |  |  |  |  |
| 8/1/29 0:00 | 1,716.3 |  |  |  |  |  |  |
| 9/1/29 0:00 | 1,715.9 |  |  |  |  |  |  |
| 10/1/29 0:00 | 1,715.8 |  |  |  |  |  |  |
| 11/1/29 0:00 | 1,715.8 |  |  |  |  |  |  |
| 12/1/29 0:00 | 1,715.8 |  |  |  |  |  |  |
| 1/1/30 0:00 | 1,715.8 |  |  |  |  |  |  |
| 2/1/30 0:00 | 1,716.0 |  |  |  |  |  |  |
| 3/1/30 0:00 | 1,716.6 |  |  |  |  |  |  |
| 4/1/30 0:00 | 1,716.6 |  |  |  |  |  |  |
| 5/1/30 0:00 | 1,716.6 |  |  |  |  |  |  |
| 6/1/30 0:00 | 1,716.5 |  |  |  |  |  |  |
| 7/1/30 0:00 | 1,716.3 |  |  |  |  |  |  |
| 8/1/30 0:00 | 1,715.8 |  |  |  |  |  |  |
| 9/1/30 0:00 | 1,715.3 |  |  |  |  |  |  |
| 10/1/30 0:00 | 1,715.0 |  |  |  |  |  |  |
| 11/1/30 0:00 | 1,715.0 |  |  |  |  |  |  |
| 12/1/30 0:00 | 1,715.0 |  |  |  |  |  |  |
| 1/1/31 0:00 | 1,715.0 |  |  |  |  |  |  |
| 2/1/31 0:00 | 1,715.0 |  |  |  |  |  |  |
| 3/1/31 0:00 | 1,714.8 |  |  |  |  |  |  |
| 4/1/31 0:00 | 1,714.5 |  |  |  |  |  |  |
| 5/1/31 0:00 | 1,716.2 |  |  |  |  |  |  |
| 6/1/31 0:00 | 1,716.2 |  |  |  |  |  |  |
| 7/1/31 0:00 | 1,716.1 |  |  |  |  |  |  |
| 8/1/31 0:00 | 1,715.6 |  |  |  |  |  |  |
| 9/1/31 0:00 | 1,715.0 |  |  |  |  |  |  |
| 10/1/31 0:00 | 1,714.7 |  |  |  |  |  |  |
| 11/0/31 0:00 | 1,714.6 |  |  |  |  |  |  |
| 12/1/31 0:00 | 1,714.6 |  |  |  |  |  |  |
| 1/1/32 0:00 | 1,714.7 |  |  |  |  |  |  |
| 2/1/32 0:00 | 1,714.7 |  |  |  |  |  |  |
| 3/1/32 0:00 | 1,715.2 |  |  |  |  |  |  |
| 4/1/32 0:00 | 1,715.4 |  |  |  |  |  |  |
| 5/1/32 0:00 | 1,715.7 |  |  |  |  |  |  |
| 6/1/32 0:00 | 1,715.8 |  |  |  |  |  |  |
| 7/1/32 0:00 | 1,715.9 |  |  |  |  |  |  |
| 8/1/32 0:00 | 1,714.9 |  |  |  |  |  |  |
| 9/1/32 0:00 | 1,715.2 |  |  |  |  |  |  |
| 10/1/32 0:00 | 1,715.0 |  |  |  |  |  |  |
| 11/1/32 0:00 | 1,715.0 |  |  |  |  |  |  |
| 12/1/32 0:00 | 1,715.0 |  |  |  |  |  |  |
| 1/1/33 0:00 | 1,714.5 |  |  |  |  |  |  |
| 2/1/33 0:00 | 1,714.5 |  |  |  |  |  |  |
| 3/1/33 0:00 | 1,714.7 |  |  |  |  |  |  |
| 4/1/33 0:00 | 1,714.5 |  |  |  |  |  |  |
| 5/1/33 0:00 | 1,713.6 |  |  |  |  |  |  |
| 6/1/33 0:00 | 1,714.2 |  |  |  |  |  |  |
| 7/1/33 0:00 | 1,714.0 |  |  |  |  |  |  |
| 8/1/33 0:00 | 1,713.2 |  |  |  |  |  |  |
| 9/1/33 0:00 | 1,713.0 |  |  |  |  |  |  |
| 10/1/33 0:00 | 1,712.5 |  |  |  |  |  |  |
| 11/1/33 0:00 | 1,712.2 |  |  |  |  |  |  |
| 12/1/33 0:00 | 1,712.2 |  |  |  |  |  |  |
| 1/1/34 0:00 | 1,712.2 |  |  |  |  |  |  |
| 2/1/34 0:00 | 1,712.2 |  |  |  |  |  |  |
| 3/1/34 0:00 | 1,712.2 |  |  |  |  |  |  |
| 11/1/34 0:00 | 1,710.4 |  |  |  |  |  |  |
| 12/1/34 0:00 | 1,710.0 |  |  |  |  |  |  |
| 1/1/35 0:00 | 1,710.4 |  |  |  |  |  |  |
| 2/1/35 0:00 | 1,710.4 |  |  |  |  |  |  |
| 3/1/35 0:00 | 1,712.0 |  |  |  |  |  |  |
| 4/1/35 0:00 | 1,712.5 |  |  |  |  |  |  |
| 5/1/35 0:00 | 1,712.6 |  |  |  |  |  |  |
| 6/1/35 0:00 | 1,712.8 |  |  |  |  |  |  |
| 7/1/35 0:00 | 1,713.0 |  |  |  |  |  |  |
| 8/1/35 0:00 | 1,712.5 |  |  |  |  |  |  |
| 9/1/35 0:00 | 1,712.3 |  |  |  |  |  |  |
| 10/1/35 0:00 | 1,711.7 |  |  |  |  |  |  |
| 12/1/35 0:00 | 1,711.5 |  |  |  |  |  |  |
| 1/1/36 0:00 | 1,711.5 |  |  |  |  |  |  |
| 3/1/36 0:00 | 1,711.6 |  |  |  |  |  |  |
| 4/1/36 0:00 | 1,712.5 |  |  |  |  |  |  |
| 5/1/36 0:00 | 1,713.2 |  |  |  |  |  |  |
| 6/1/36 0:00 | 1,713.1 |  |  |  |  |  |  |
| 7/1/36 0:00 | 1,712.6 |  |  |  |  |  |  |
| 8/1/36 0:00 | 1,711.6 |  |  |  |  |  |  |
| 9/1/36 0:00 | 1,711.0 |  |  |  |  |  |  |
| 10/1/36 0:00 | 1,710.6 |  |  |  |  |  |  |
| 11/1/36 0:00 | 1,710.4 |  |  |  |  |  |  |
| 12/1/36 0:00 | 1,710.4 |  |  |  |  |  |  |
| 1/1/37 0:00 | 1,710.4 |  |  |  |  |  |  |
| 3/1/37 0:00 | 1,710.9 |  |  |  |  |  |  |
| 4/1/37 0:00 | 1,714.0 |  |  |  |  |  |  |
| 5/1/37 0:00 | 1,718.1 |  |  |  |  |  |  |
| 6/1/37 0:00 | 1,719.0 |  |  |  |  |  |  |
| 7/1/37 0:00 | 1,718.3 |  |  |  |  |  |  |
| 9/1/37 0:00 | 1,717.6 |  |  |  |  |  |  |
| 10/1/37 0:00 | 1,717.2 |  |  |  |  |  |  |
| 11/1/37 0:00 | 1,717.1 |  |  |  |  |  |  |
| 12/1/37 0:00 | 1,717.1 |  |  |  |  |  |  |
| 1/1/38 0:00 | 1,717.1 |  |  |  |  |  |  |
| 3/1/38 0:00 | 1,717.4 |  |  |  |  |  |  |
| 4/1/38 0:00 | 1,718.6 |  |  |  |  |  |  |
| 5/1/38 0:00 | 1,718.7 |  |  |  |  |  |  |
| 6/1/38 0:00 | 1,718.3 |  |  |  |  |  |  |
| 7/1/38 0:00 | 1,718.3 |  |  |  |  |  |  |
| 8/1/38 0:00 | 1,718.1 |  |  |  |  |  |  |
| 9/1/38 0:00 | 1,717.3 |  |  |  |  |  |  |
| 10/1/38 0:00 | 1,717.2 |  |  |  |  |  |  |
| 11/1/38 0:00 | 1,716.8 |  |  |  |  |  |  |
| 12/1/38 0:00 | 1,716.7 |  |  |  |  |  |  |
| 3/1/42 0:00 | 1,715.9 |  |  |  |  |  |  |
| 4/1/42 0:00 | 1,717.8 |  |  |  |  |  |  |
| 6/1/42 0:00 | 1,718.0 |  |  |  |  |  |  |
| 7/1/42 0:00 | 1,717.9 |  |  |  |  |  |  |
| 10/1/42 0:00 | 1,717.5 |  |  |  |  |  |  |
| 12/1/42 0:00 | 1,717.5 |  |  |  |  |  |  |
| 3/15/43 0:00 | 1,717.5 |  |  |  |  |  |  |
| 3/27/43 0:00 | 1,720.1 |  |  |  |  |  |  |
| 3/29/43 0:00 | 1,720.7 |  |  |  |  |  |  |
| 3/31/43 0:00 | 1,721.3 |  |  |  |  |  |  |
| 4/1/43 0:00 | 1,721.3 |  |  |  |  |  |  |
| 4/3/43 0:00 | 1,721.1 |  |  |  |  |  |  |
| 4/9/43 0:00 | 1,720.3 |  |  |  |  |  |  |
| 8/1/44 0:00 | 1,717.4 |  |  |  |  |  |  |
| 9/1/44 0:00 | 1,717.1 |  |  |  |  |  |  |
| 10/1/44 0:00 | 1,716.8 |  |  |  |  |  |  |
| 11/1/44 0:00 | 1,716.7 |  |  |  |  |  |  |
| 12/1/44 0:00 | 1,717.5 |  |  |  |  |  |  |
| 1/1/45 0:00 | 1,716.5 |  |  |  |  |  |  |
| 2/1/45 0:00 | 1,716.4 |  |  |  |  |  |  |
| 3/1/45 0:00 | 1,716.3 |  |  |  |  |  |  |
| 4/1/45 0:00 | 1,716.8 |  |  |  |  |  |  |
| 5/1/45 0:00 | 1,716.8 |  |  |  |  |  |  |
| 6/1/45 0:00 | 1,718.1 |  |  |  |  |  |  |
| 7/1/45 0:00 | 1,717.4 |  |  |  |  |  |  |
| 8/1/45 0:00 | 1,716.8 |  |  |  |  |  |  |
| 9/1/45 0:00 | 1,716.5 |  |  |  |  |  |  |
| 10/1/45 0:00 | 1,716.3 |  |  |  |  |  |  |
| 11/1/45 0:00 | 1,716.2 |  |  |  |  |  |  |
| 12/1/45 0:00 | 1,716.2 |  |  |  |  |  |  |
| 1/1/46 0:00 | 1,716.2 |  |  |  |  |  |  |
| 2/1/46 0:00 | 1,716.3 |  |  |  |  |  |  |
| 3/1/46 0:00 | 1,716.3 |  |  |  |  |  |  |
| 4/1/46 0:00 | 1,718.7 |  |  |  |  |  |  |
| 5/1/46 0:00 | 1,717.3 |  |  |  |  |  |  |
| 6/1/46 0:00 | 1,717.1 |  |  |  |  |  |  |
| 7/1/46 0:00 | 1,717.2 |  |  |  |  |  |  |
| 8/1/46 0:00 | 1,716.7 |  |  |  |  |  |  |
| 9/1/46 0:00 | 1,716.8 |  |  |  |  |  |  |
| 10/1/46 0:00 | 1,717.0 |  |  |  |  |  |  |
| 11/1/46 0:00 | 1,717.1 |  |  |  |  |  |  |
| 12/1/46 0:00 | 1,716.9 |  |  |  |  |  |  |
| 1/1/47 0:00 | 1,716.5 |  |  |  |  |  |  |
| 2/1/47 0:00 | 1,716.5 |  |  |  |  |  |  |
| 3/1/47 0:00 | 1,716.6 |  |  |  |  |  |  |
| 4/1/47 0:00 | 1,717.2 |  |  |  |  |  |  |
| 5/1/47 0:00 | 1,718.3 |  |  |  |  |  |  |
| 6/1/47 0:00 | 1,717.2 |  |  |  |  |  |  |
| 7/1/47 0:00 | 1,716.7 |  |  |  |  |  |  |
| 8/1/47 0:00 | 1,716.6 |  |  |  |  |  |  |
| 9/1/47 0:00 | 1,716.5 |  |  |  |  |  |  |
| 10/1/47 0:00 | 1,716.3 |  |  |  |  |  |  |
| 11/1/47 0:00 | 1,716.2 |  |  |  |  |  |  |
| 12/1/47 0:00 | 1,716.0 |  |  |  |  |  |  |
| 1/1/48 0:00 | 1,716.0 |  |  |  |  |  |  |
| 2/1/48 0:00 | 1,716.0 |  |  |  |  |  |  |
| 3/1/48 0:00 | 1,716.1 |  |  |  |  |  |  |
| 4/1/48 0:00 | 1,719.0 |  |  |  |  |  |  |
| 4/2/48 0:00 | 1,719.1 |  |  |  |  |  |  |
| 5/1/48 0:00 | 1,717.8 |  |  |  |  |  |  |
| 6/1/48 0:00 | 1,717.2 |  |  |  |  |  |  |
| 7/1/48 0:00 | 1,716.8 |  |  |  |  |  |  |
| 8/1/48 0:00 | 1,716.7 |  |  |  |  |  |  |
| 9/1/48 0:00 | 1,716.5 |  |  |  |  |  |  |
| 10/1/48 0:00 | 1,716.4 |  |  |  |  |  |  |
| 11/1/48 0:00 | 1,716.4 |  |  |  |  |  |  |
| 12/1/48 0:00 | 1,716.3 |  |  |  |  |  |  |
| 1/1/49 0:00 | 1,716.3 |  |  |  |  |  |  |
| 2/1/49 0:00 | 1,716.3 |  |  |  |  |  |  |
| 3/1/49 0:00 | 1,717.3 |  |  |  |  |  |  |
| 4/1/49 0:00 | 1,717.3 |  |  |  |  |  |  |
| 5/1/49 0:00 | 1,717.0 |  |  |  |  |  |  |
| 6/1/49 0:00 | 1,716.8 |  |  |  |  |  |  |
| 7/1/49 0:00 | 1,716.3 |  |  |  |  |  |  |
| 8/1/49 0:00 | 1,715.9 |  |  |  |  |  |  |
| 9/1/49 0:00 | 1,715.8 |  |  |  |  |  |  |
| 10/1/49 0:00 | 1,715.5 |  |  |  |  |  |  |
| 11/1/49 0:00 | 1,715.2 |  |  |  |  |  |  |
| 1/1/50 0:00 | 1,715.5 |  |  |  |  |  |  |
| 2/1/50 0:00 | 1,715.6 |  |  |  |  |  |  |
| 3/1/50 0:00 | 1,716.8 |  |  |  |  |  |  |
| 4/1/50 0:00 | 1,717.5 |  |  |  |  |  |  |
| 5/1/50 0:00 | 1,717.6 |  |  |  |  |  |  |
| 6/1/50 0:00 | 1,716.6 |  |  |  |  |  |  |
| 7/1/50 0:00 | 1,716.6 |  |  |  |  |  |  |
| 8/1/50 0:00 | 1,716.6 |  |  |  |  |  |  |
| 9/1/50 0:00 | 1,716.2 |  |  |  |  |  |  |
| 10/1/50 0:00 | 1,716.3 |  |  |  |  |  |  |
| 11/1/50 0:00 | 1,716.0 |  |  |  |  |  |  |
| 12/1/50 0:00 | 1,716.0 |  |  |  |  |  |  |
| 1/1/51 0:00 | 1,716.0 |  |  |  |  |  |  |
| 2/1/51 0:00 | 1,716.0 |  |  |  |  |  |  |
| 3/1/51 0:00 | 1,717.8 |  |  |  |  |  |  |
| 4/1/51 0:00 | 1,718.6 |  |  |  |  |  |  |
| 5/1/51 0:00 | 1,717.4 |  |  |  |  |  |  |
| 6/1/51 0:00 | 1,717.4 |  |  |  |  |  |  |
| 7/1/51 0:00 | 1,717.4 |  |  |  |  |  |  |
| 8/1/51 0:00 | 1,717.3 |  |  |  |  |  |  |
| 9/1/51 0:00 | 1,717.2 |  |  |  |  |  |  |
| 10/1/51 0:00 | 1,716.9 |  |  |  |  |  |  |
| 11/1/51 0:00 | 1,717.0 |  |  |  |  |  |  |
| 12/1/51 0:00 | 1,717.0 |  |  |  |  |  |  |
| 1/1/52 0:00 | 1,717.1 |  |  |  |  |  |  |
| 2/1/52 0:00 | 1,717.3 |  |  |  |  |  |  |
| 3/1/52 0:00 | 1,717.2 |  |  |  |  |  |  |
| 4/1/52 0:00 | 1,719.3 |  |  |  |  |  |  |
| 4/10/52 0:00 | 1,720.9 |  |  |  |  |  |  |
| 5/1/52 0:00 | 1,717.4 |  |  |  |  |  |  |
| 6/1/52 0:00 | 1,717.1 |  |  |  |  |  |  |
| 7/1/52 0:00 | 1,717.3 |  |  |  |  |  |  |
| 8/1/52 0:00 | 1,716.8 |  |  |  |  |  |  |
| 9/1/52 0:00 | 1,716.4 |  |  |  |  |  |  |
| 10/1/52 0:00 | 1,716.1 |  |  |  |  |  |  |
| 11/1/52 0:00 | 1,715.9 |  |  |  |  |  |  |
| 12/1/52 0:00 | 1,715.8 |  |  |  |  |  |  |
| 1/1/53 0:00 | 1,715.9 |  |  |  |  |  |  |
| 2/1/53 0:00 | 1,716.0 |  |  |  |  |  |  |
| 3/1/53 0:00 | 1,717.3 |  |  |  |  |  |  |
| 4/1/53 0:00 | 1,717.6 |  |  |  |  |  |  |
| 5/1/53 0:00 | 1,717.5 |  |  |  |  |  |  |
| 6/1/53 0:00 | 1,717.9 |  |  |  |  |  |  |
| 7/1/53 0:00 | 1,718.2 |  |  |  |  |  |  |
| 8/1/53 0:00 | 1,717.8 |  |  |  |  |  |  |
| 9/1/53 0:00 | 1,717.0 |  |  |  |  |  |  |
| 10/1/53 0:00 | 1,716.8 |  |  |  |  |  |  |
| 11/1/53 0:00 | 1,716.7 |  |  |  |  |  |  |
| 12/1/53 0:00 | 1,716.8 |  |  |  |  |  |  |
| 1/1/54 0:00 | 1,716.4 |  |  |  |  |  |  |
| 2/1/54 0:00 | 1,716.4 |  |  |  |  |  |  |
| 3/1/54 0:00 | 1,717.3 |  |  |  |  |  |  |
| 4/1/54 0:00 | 1,717.8 |  |  |  |  |  |  |
| 5/1/54 0:00 | 1,717.7 |  |  |  |  |  |  |
| 6/1/54 0:00 | 1,717.8 |  |  |  |  |  |  |
| 7/1/54 0:00 | 1,716.8 |  |  |  |  |  |  |
| 8/1/54 0:00 | 1,716.4 |  |  |  |  |  |  |
| 9/1/54 0:00 | 1,716.3 |  |  |  |  |  |  |
| 10/1/54 0:00 | 1,716.1 |  |  |  |  |  |  |
| 11/1/54 0:00 | 1,716.1 |  |  |  |  |  |  |
| 12/1/54 0:00 | 1,716.1 |  |  |  |  |  |  |
| 1/1/55 0:00 | 1,716.1 |  |  |  |  |  |  |
| 2/1/55 0:00 | 1,716.2 |  |  |  |  |  |  |
| 3/1/55 0:00 | 1,716.7 |  |  |  |  |  |  |
| 4/1/55 0:00 | 1,717.0 |  |  |  |  |  |  |
| 5/1/55 0:00 | 1,716.8 |  |  |  |  |  |  |
| 6/1/55 0:00 | 1,716.7 |  |  |  |  |  |  |
| 7/1/55 0:00 | 1,716.7 |  |  |  |  |  |  |
| 8/1/55 0:00 | 1,716.3 |  |  |  |  |  |  |
| 9/1/55 0:00 | 1,716.1 |  |  |  |  |  |  |
| 10/1/55 0:00 | 1,715.8 |  |  |  |  |  |  |
| 11/1/55 0:00 | 1,715.6 |  |  |  |  |  |  |
| 12/1/55 0:00 | 1,715.6 |  |  |  |  |  |  |
| 1/1/56 0:00 | 1,715.7 |  |  |  |  |  |  |
| 2/1/56 0:00 | 1,715.7 |  |  |  |  |  |  |
| 3/1/56 0:00 | 1,715.8 |  |  |  |  |  |  |
| 4/1/56 0:00 | 1,716.7 |  |  |  |  |  |  |
| 5/1/56 0:00 | 1,717.1 |  |  |  |  |  |  |
| 6/1/56 0:00 | 1,717.3 |  |  |  |  |  |  |
| 7/1/56 0:00 | 1,717.2 |  |  |  |  |  |  |
| 8/1/56 0:00 | 1,717.5 |  |  |  |  |  |  |
| 9/1/56 0:00 | 1,717.0 |  |  |  |  |  |  |
| 10/1/56 0:00 | 1,716.8 |  |  |  |  |  |  |
| 11/1/56 0:00 | 1,716.8 |  |  |  |  |  |  |
| 12/1/56 0:00 | 1,716.8 |  |  |  |  |  |  |
| 1/1/57 0:00 | 1,716.8 |  |  |  |  |  |  |
| 2/1/57 0:00 | 1,716.8 |  |  |  |  |  |  |
| 3/1/57 0:00 | 1,717.4 |  |  |  |  |  |  |
| 4/1/57 0:00 | 1,717.5 |  |  |  |  |  |  |
| 5/1/57 0:00 | 1,717.8 |  |  |  |  |  |  |
| 6/1/57 0:00 | 1,717.8 |  |  |  |  |  |  |
| 7/1/57 0:00 | 1,717.3 |  |  |  |  |  |  |
| 8/1/57 0:00 | 1,716.7 |  |  |  |  |  |  |
| 9/1/57 0:00 | 1,716.8 |  |  |  |  |  |  |
| 10/1/57 0:00 | 1,717.0 |  |  |  |  |  |  |
| 11/1/57 0:00 | 1,717.2 |  |  |  |  |  |  |
| 12/1/57 0:00 | 1,717.2 |  |  |  |  |  |  |
| 1/1/58 0:00 | 1,717.1 |  |  |  |  |  |  |
| 2/1/58 0:00 | 1,717.1 |  |  |  |  |  |  |
| 3/1/58 0:00 | 1,717.2 |  |  |  |  |  |  |
| 4/1/58 0:00 | 1,717.6 |  |  |  |  |  |  |
| 5/1/58 0:00 | 1,717.3 |  |  |  |  |  |  |
| 6/1/58 0:00 | 1,717.1 |  |  |  |  |  |  |
| 7/1/58 0:00 | 1,716.8 |  |  |  |  |  |  |
| 8/1/58 0:00 | 1,716.3 |  |  |  |  |  |  |
| 9/1/58 0:00 | 1,716.0 |  |  |  |  |  |  |
| 10/1/58 0:00 | 1,715.5 |  |  |  |  |  |  |
| 11/1/58 0:00 | 1,715.4 |  |  |  |  |  |  |
| 12/1/58 0:00 | 1,715.4 |  |  |  |  |  |  |
| 1/1/59 0:00 | 1,715.4 |  |  |  |  |  |  |
| 2/1/59 0:00 | 1,715.5 |  |  |  |  |  |  |
| 3/1/59 0:00 | 1,715.5 |  |  |  |  |  |  |
| 4/1/59 0:00 | 1,715.7 |  |  |  |  |  |  |
| 5/1/59 0:00 | 1,715.7 |  |  |  |  |  |  |
| 6/1/59 0:00 | 1,715.7 |  |  |  |  |  |  |
| 7/1/59 0:00 | 1,715.3 |  |  |  |  |  |  |
| 8/1/59 0:00 | 1,714.8 |  |  |  |  |  |  |
| 9/1/59 0:00 | 1,714.3 |  |  |  |  |  |  |
| 10/1/59 0:00 | 1,714.1 |  |  |  |  |  |  |
| 11/1/59 0:00 | 1,714.0 |  |  |  |  |  |  |
| 12/1/59 0:00 | 1,714.1 |  |  |  |  |  |  |
| 1/1/60 0:00 | 1,714.2 |  |  |  |  |  |  |
| 2/1/60 0:00 | 1,714.2 |  |  |  |  |  |  |
| 3/1/60 0:00 | 1,714.8 |  |  |  |  |  |  |
| 4/1/60 0:00 | 1,717.9 |  |  |  |  |  |  |
| 5/1/60 0:00 | 1,717.6 |  |  |  |  |  |  |
| 6/1/60 0:00 | 1,717.6 |  |  |  |  |  |  |
| 7/1/60 0:00 | 1,717.2 |  |  |  |  |  |  |
| 8/1/60 0:00 | 1,716.7 |  |  |  |  |  |  |
| 9/1/60 0:00 | 1,716.4 |  |  |  |  |  |  |
| 10/1/60 0:00 | 1,716.2 |  |  |  |  |  |  |
| 11/1/60 0:00 | 1,716.0 |  |  |  |  |  |  |
| 12/1/60 0:00 | 1,716.0 |  |  |  |  |  |  |
| 2/1/61 0:00 | 1,716.2 |  |  |  |  |  |  |
| 3/1/61 0:00 | 1,716.4 |  |  |  |  |  |  |
| 4/1/61 0:00 | 1,716.5 |  |  |  |  |  |  |
| 5/1/61 0:00 | 1,716.8 |  |  |  |  |  |  |
| 6/1/61 0:00 | 1,716.9 |  |  |  |  |  |  |
| 7/1/61 0:00 | 1,716.6 |  |  |  |  |  |  |
| 8/1/61 0:00 | 1,716.1 |  |  |  |  |  |  |
| 9/1/61 0:00 | 1,715.9 |  |  |  |  |  |  |
| 10/1/61 0:00 | 1,715.8 |  |  |  |  |  |  |
| 11/1/61 0:00 | 1,715.7 |  |  |  |  |  |  |
| 12/1/61 0:00 | 1,715.7 |  |  |  |  |  |  |
| 1/1/62 0:00 | 1,715.7 |  |  |  |  |  |  |
| 2/1/62 0:00 | 1,715.7 |  |  |  |  |  |  |
| 3/1/62 0:00 | 1,717.1 |  |  |  |  |  |  |
| 4/1/62 0:00 | 1,718.1 |  |  |  |  |  |  |
| 5/1/62 0:00 | 1,719.1 |  |  |  |  |  |  |
| 6/1/62 0:00 | 1,719.1 |  |  |  |  |  |  |
| 7/1/62 0:00 | 1,718.5 |  |  |  |  |  |  |
| 8/1/62 0:00 | 1,717.8 |  |  |  |  |  |  |
| 9/1/62 0:00 | 1,716.9 |  |  |  |  |  |  |
| 10/1/62 0:00 | 1,717.1 |  |  |  |  |  |  |
| 11/1/62 0:00 | 1,717.1 |  |  |  |  |  |  |
| 12/1/62 0:00 | 1,717.2 |  |  |  |  |  |  |
| 1/1/63 0:00 | 1,717.2 |  |  |  |  |  |  |
| 2/1/63 0:00 | 1,717.2 |  |  |  |  |  |  |
| 3/1/63 0:00 | 1,717.4 |  |  |  |  |  |  |
| 4/1/63 0:00 | 1,717.9 |  |  |  |  |  |  |
| 5/1/63 0:00 | 1,718.0 |  |  |  |  |  |  |
| 6/1/63 0:00 | 1,717.9 |  |  |  |  |  |  |
| 7/1/63 0:00 | 1,717.8 |  |  |  |  |  |  |
| 9/1/63 0:00 | 1,717.3 |  |  |  |  |  |  |
| 10/1/63 0:00 | 1,717.3 |  |  |  |  |  |  |
| 11/1/63 0:00 | 1,717.3 |  |  |  |  |  |  |
| 12/1/63 0:00 | 1,717.2 |  |  |  |  |  |  |
| 1/1/64 0:00 | 1,717.1 |  |  |  |  |  |  |
| 2/1/64 0:00 | 1,717.0 |  |  |  |  |  |  |
| 3/1/64 0:00 | 1,717.2 |  |  |  |  |  |  |
| 4/1/64 0:00 | 1,717.9 |  |  |  |  |  |  |
| 5/1/64 0:00 | 1,717.9 |  |  |  |  |  |  |
| 6/1/64 0:00 | 1,717.3 |  |  |  |  |  |  |
| 7/1/64 0:00 | 1,717.0 |  |  |  |  |  |  |
| 8/1/64 0:00 | 1,716.4 |  |  |  |  |  |  |
| 9/1/64 0:00 | 1,716.2 |  |  |  |  |  |  |
| 10/1/64 0:00 | 1,716.0 |  |  |  |  |  |  |
| 11/1/64 0:00 | 1,715.7 |  |  |  |  |  |  |
| 12/1/64 0:00 | 1,715.7 |  |  |  |  |  |  |
| 1/1/65 0:00 | 1,715.7 |  |  |  |  |  |  |
| 2/1/65 0:00 | 1,715.7 |  |  |  |  |  |  |
| 3/1/65 0:00 | 1,715.8 |  |  |  |  |  |  |
| 4/1/65 0:00 | 1,718.1 |  |  |  |  |  |  |
| 5/1/65 0:00 | 1,718.7 |  |  |  |  |  |  |
| 6/1/65 0:00 | 1,717.9 |  |  |  |  |  |  |
| 7/1/65 0:00 | 1,717.7 |  |  |  |  |  |  |
| 8/1/65 0:00 | 1,717.1 |  |  |  |  |  |  |
| 9/1/65 0:00 | 1,716.8 |  |  |  |  |  |  |
| 10/1/65 0:00 | 1,717.0 |  |  |  |  |  |  |
| 11/1/65 0:00 | 1,716.9 |  |  |  |  |  |  |
| 12/1/65 0:00 | 1,716.9 |  |  |  |  |  |  |
| 1/1/66 0:00 | 1,716.9 |  |  |  |  |  |  |
| 3/1/66 0:00 | 1,718.1 |  |  |  |  |  |  |
| 4/1/66 0:00 | 1,717.9 |  |  |  |  |  |  |
| 6/1/66 0:00 | 1,717.5 |  |  |  |  |  |  |
| 7/1/66 0:00 | 1,717.2 |  |  |  |  |  |  |
| 8/1/66 0:00 | 1,716.8 |  |  |  |  |  |  |
| 9/1/66 0:00 | 1,716.9 |  |  |  |  |  |  |
| 10/1/66 0:00 | 1,716.7 |  |  |  |  |  |  |
| 1/1/67 0:00 | 1,716.7 |  |  |  |  |  |  |
| 2/1/67 0:00 | 1,716.6 |  |  |  |  |  |  |
| 3/1/67 0:00 | 1,718.1 |  |  |  |  |  |  |
| 4/1/67 0:00 | 1,718.0 |  |  |  |  |  |  |
| 5/1/67 0:00 | 1,717.9 |  |  |  |  |  |  |
| 6/1/67 0:00 | 1,717.6 |  |  |  |  |  |  |
| 7/1/67 0:00 | 1,717.7 |  |  |  |  |  |  |
| 8/1/67 0:00 | 1,716.8 |  |  |  |  |  |  |
| 9/1/67 0:00 | 1,716.4 |  |  |  |  |  |  |
| 10/1/67 0:00 | 1,716.2 |  |  |  |  |  |  |
| 11/1/67 0:00 | 1,716.0 |  |  |  |  |  |  |
| 12/1/67 0:00 | 1,715.9 |  |  |  |  |  |  |
| 1/1/68 0:00 | 1,715.9 |  |  |  |  |  |  |
| 2/1/68 0:00 | 1,716.0 |  |  |  |  |  |  |
| 3/1/68 0:00 | 1,715.9 |  |  |  |  |  |  |
| 4/1/68 0:00 | 1,716.2 |  |  |  |  |  |  |
| 5/1/68 0:00 | 1,716.2 |  |  |  |  |  |  |
| 6/1/68 0:00 | 1,716.3 |  |  |  |  |  |  |
| 7/1/68 0:00 | 1,716.2 |  |  |  |  |  |  |
| 8/1/68 0:00 | 1,716.0 |  |  |  |  |  |  |
| 9/1/68 0:00 | 1,715.7 |  |  |  |  |  |  |
| 10/1/68 0:00 | 1,715.6 |  |  |  |  |  |  |
| 11/1/68 0:00 | 1,715.7 |  |  |  |  |  |  |
| 12/1/68 0:00 | 1,715.8 |  |  |  |  |  |  |
| 3/1/69 0:00 | 1,716.0 |  |  |  |  |  |  |
| 4/5/69 0:00 | 1,716.0 |  |  |  |  |  |  |
| 4/7/69 0:00 | 1,716.0 |  |  |  |  |  |  |
| 4/9/69 0:30 | 1,719.4 |  |  |  |  |  |  |
| 4/9/69 2:30 | 1,719.6 |  |  |  |  |  |  |
| 4/9/69 6:00 | 1,719.9 |  |  |  |  |  |  |
| 4/9/69 15:30 | 1,720.7 |  |  |  |  |  |  |
| 4/9/69 17:30 | 1,720.7 |  |  |  |  |  |  |
| 4/9/69 19:30 | 1,720.8 |  |  |  |  |  |  |
| 4/9/69 21:30 | 1,720.9 |  |  |  |  |  |  |
| 4/9/69 23:30 | 1,721.0 |  |  |  |  |  |  |
| 4/10/69 1:30 | 1,721.1 |  |  |  |  |  |  |
| 4/10/69 0:30 | 1,721.1 |  |  |  |  |  |  |
| 4/10/69 5:30 | 1,721.1 |  |  |  |  |  |  |
| 4/10/69 7:30 | 1,721.2 |  |  |  |  |  |  |
| 4/10/69 9:30 | 1,721.2 |  |  |  |  |  |  |
| 4/10/69 13:45 | 1,721.2 |  |  |  |  |  |  |
| 4/10/69 16:30 | 1,721.2 |  |  |  |  |  |  |
| 4/10/69 22:00 | 1,721.2 |  |  |  |  |  |  |
| 4/11/69 0:30 | 1,721.2 |  |  |  |  |  |  |
| 4/11/69 3:30 | 1,721.2 |  |  |  |  |  |  |
| 4/11/69 7:30 | 1,721.2 |  |  |  |  |  |  |
| 4/12/69 3:00 | 1,721.1 |  |  |  |  |  |  |
| 4/12/69 6:00 | 1,721.0 |  |  |  |  |  |  |
| 4/12/69 14:00 | 1,720.9 |  |  |  |  |  |  |
| 4/13/69 3:00 | 1,720.9 |  |  |  |  |  |  |
| 4/13/69 0:00 | 1,720.8 |  |  |  |  |  |  |
| 4/13/69 6:30 | 1,720.7 |  |  |  |  |  |  |
| 4/14/69 0:30 | 1,720.7 |  |  |  |  |  |  |
| 4/14/69 5:30 | 1,720.6 |  |  |  |  |  |  |
| 4/15/69 0:00 | 1,720.5 |  |  |  |  |  |  |
| 4/15/69 6:15 | 1,720.5 |  |  |  |  |  |  |
| 4/15/69 16:30 | 1,720.3 |  |  |  |  |  |  |
| 4/16/69 8:15 | 1,720.1 |  |  |  |  |  |  |
| 4/17/69 10:30 | 1,719.9 |  |  |  |  |  |  |
| 4/18/69 0:30 | 1,719.8 |  |  |  |  |  |  |
| 4/18/69 16:00 | 1,719.8 |  |  |  |  |  |  |
| 4/19/69 0:00 | 1,719.7 |  |  |  |  |  |  |
| 4/20/69 3:15 | 1,719.6 |  |  |  |  |  |  |
| 4/21/69 3:15 | 1,719.6 |  |  |  |  |  |  |
| 4/22/69 3:30 | 1,719.3 |  |  |  |  |  |  |
| 4/23/69 0:45 | 1,719.2 |  |  |  |  |  |  |
| 4/24/69 3:15 | 1,719.1 |  |  |  |  |  |  |
| 4/26/69 2:00 | 1,719.0 |  |  |  |  |  |  |
| 4/27/69 2:30 | 1,718.9 |  |  |  |  |  |  |
| 4/28/69 0:00 | 1,718.9 |  |  |  |  |  |  |
| 5/1/69 3:00 | 1,718.7 |  |  |  |  |  |  |
| 5/2/69 3:30 | 1,718.6 |  |  |  |  |  |  |
| 5/5/69 1:00 | 1,718.6 |  |  |  |  |  |  |
| 5/6/69 2:30 | 1,718.5 |  |  |  |  |  |  |
| 5/7/69 4:00 | 1,718.5 |  |  |  |  |  |  |
| 5/8/69 0:30 | 1,718.5 |  |  |  |  |  |  |
| 5/13/69 0:00 | 1,718.3 |  |  |  |  |  |  |
| 5/16/69 0:00 | 1,718.2 |  |  |  |  |  |  |
| 5/18/69 8:30 | 1,718.0 |  |  |  |  |  |  |
| 5/20/69 3:00 | 1,717.9 |  |  |  |  |  |  |
| 5/23/69 2:30 | 1,717.8 |  |  |  |  |  |  |
| 5/31/69 7:00 | 1,717.6 |  |  |  |  |  |  |
| 6/1/69 0:00 | 1,717.5 |  |  |  |  |  |  |
| 6/15/69 0:00 | 1,717.1 |  |  |  |  |  |  |
| 7/1/69 0:00 | 1,717.4 |  |  |  |  |  |  |
| 7/15/69 0:00 | 1,717.1 |  |  |  |  |  |  |
| 8/1/69 0:00 | 1,717.4 |  |  |  |  |  |  |
| 8/15/69 0:00 | 1,716.9 |  |  |  |  |  |  |
| 9/1/69 0:00 | 1,716.9 |  |  |  |  |  |  |
| 9/15/69 0:00 | 1,716.7 |  |  |  |  |  |  |
| 10/1/69 0:00 | 1,716.7 |  |  |  |  |  |  |
| 10/15/69 0:00 | 1,716.6 |  |  |  |  |  |  |
| 11/1/69 0:00 | 1,716.6 |  |  |  |  |  |  |
| 11/15/69 0:00 | 1,716.6 |  |  |  |  |  |  |
| 12/1/69 0:00 | 1,716.6 |  |  |  |  |  |  |
| 12/15/69 0:00 | 1,716.9 |  |  |  |  |  |  |
| 1/1/70 0:00 | 1,716.8 |  |  |  |  |  |  |
| 1/15/70 0:00 | 1,716.7 |  |  |  |  |  |  |
| 2/1/70 0:00 | 1,716.7 |  |  |  |  |  |  |
| 2/15/70 0:00 | 1,716.8 |  |  |  |  |  |  |
| 3/1/70 0:00 | 1,716.8 |  |  |  |  |  |  |
| 3/15/70 0:00 | 1,717.4 |  |  |  |  |  |  |
| 4/1/70 0:00 | 1,717.0 |  |  |  |  |  |  |
| 4/15/70 0:00 | 1,718.0 |  |  |  |  |  |  |
| 5/1/70 0:00 | 1,718.0 |  |  |  |  |  |  |
| 5/15/70 0:00 | 1,717.8 |  |  |  |  |  |  |
| 6/1/70 0:00 | 1,718.1 |  |  |  |  |  |  |
| 6/15/70 0:00 | 1,717.6 |  |  |  |  |  |  |
| 7/1/70 0:00 | 1,717.7 |  |  |  |  |  |  |
| 7/15/70 0:00 | 1,717.6 |  |  |  |  |  |  |
| 8/1/70 0:00 | 1,717.1 |  |  |  |  |  |  |
| 8/15/70 0:00 | 1,716.7 |  |  |  |  |  |  |
| 9/1/70 0:00 | 1,716.7 |  |  |  |  |  |  |
| 9/15/70 0:00 | 1,716.5 |  |  |  |  |  |  |
| 10/1/70 0:00 | 1,716.3 |  |  |  |  |  |  |
| 10/15/70 0:00 | 1,716.3 |  |  |  |  |  |  |
| 11/1/70 0:00 | 1,716.4 |  |  |  |  |  |  |
| 11/15/70 0:00 | 1,716.4 |  |  |  |  |  |  |
| 12/1/70 0:00 | 1,716.4 |  |  |  |  |  |  |
| 12/15/70 0:00 | 1,716.4 |  |  |  |  |  |  |
| 1/1/71 0:00 | 1,716.5 |  |  |  |  |  |  |
| 1/15/71 0:00 | 1,716.5 |  |  |  |  |  |  |
| 2/1/71 0:00 | 1,716.5 |  |  |  |  |  |  |
| 2/15/71 0:00 | 1,716.5 |  |  |  |  |  |  |
| 3/1/71 0:00 | 1,717.0 |  |  |  |  |  |  |
| 3/15/71 0:00 | 1,716.6 |  |  |  |  |  |  |
| 4/1/71 0:00 | 1,717.2 |  |  |  |  |  |  |
| 4/15/71 0:00 | 1,717.1 |  |  |  |  |  |  |
| 5/1/71 0:00 | 1,717.1 |  |  |  |  |  |  |
| 5/15/71 0:00 | 1,717.0 |  |  |  |  |  |  |
| 6/1/71 0:00 | 1,717.2 |  |  |  |  |  |  |
| 6/15/71 0:00 | 1,717.1 |  |  |  |  |  |  |
| 7/1/71 0:00 | 1,717.6 |  |  |  |  |  |  |
| 7/15/71 0:00 | 1,717.4 |  |  |  |  |  |  |
| 8/1/71 0:00 | 1,717.3 |  |  |  |  |  |  |
| 8/15/71 0:00 | 1,717.2 |  |  |  |  |  |  |
| 9/1/71 0:00 | 1,717.2 |  |  |  |  |  |  |
| 9/15/71 0:00 | 1,717.1 |  |  |  |  |  |  |
| 10/1/71 0:00 | 1,717.4 |  |  |  |  |  |  |
| 10/15/71 0:00 | 1,717.0 |  |  |  |  |  |  |
| 11/1/71 0:00 | 1,717.7 |  |  |  |  |  |  |
| 11/15/71 0:00 | 1,717.1 |  |  |  |  |  |  |
| 12/1/71 0:00 | 1,717.9 |  |  |  |  |  |  |
| 12/15/71 0:00 | 1,717.9 |  |  |  |  |  |  |
| 1/1/72 0:00 | 1,717.9 |  |  |  |  |  |  |
| 1/15/72 0:00 | 1,717.9 |  |  |  |  |  |  |
| 2/1/72 0:00 | 1,717.9 |  |  |  |  |  |  |
| 2/15/72 0:00 | 1,717.5 |  |  |  |  |  |  |
| 3/1/72 0:00 | 1,717.7 |  |  |  |  |  |  |
| 3/15/72 0:00 | 1,719.7 |  |  |  |  |  |  |
| 4/1/72 0:00 | 1,718.8 |  |  |  |  |  |  |
| 4/15/72 0:00 | 1,718.7 |  |  |  |  |  |  |
| 5/1/72 0:00 | 1,718.7 |  |  |  |  |  |  |
| 5/15/72 0:00 | 1,719.6 |  |  |  |  |  |  |
| 6/1/72 0:00 | 1,719.7 |  |  |  |  |  |  |
| 6/15/72 0:00 | 1,718.5 |  |  |  |  |  |  |
| 7/1/72 0:00 | 1,718.4 |  |  |  |  |  |  |
| 7/15/72 0:00 | 1,718.0 |  |  |  |  |  |  |
| 8/1/72 0:00 | 1,718.2 |  |  |  |  |  |  |
| 8/15/72 0:00 | 1,717.7 |  |  |  |  |  |  |
| 9/1/72 0:00 | 1,717.6 |  |  |  |  |  |  |
| 9/15/72 0:00 | 1,717.0 |  |  |  |  |  |  |
| 10/1/72 0:00 | 1,716.9 |  |  |  |  |  |  |
| 10/15/72 0:00 | 1,716.7 |  |  |  |  |  |  |
| 11/1/72 0:00 | 1,716.7 |  |  |  |  |  |  |
| 11/15/72 0:00 | 1,716.7 |  |  |  |  |  |  |
| 12/1/72 0:00 | 1,716.7 |  |  |  |  |  |  |
| 12/15/72 0:00 | 1,716.6 |  |  |  |  |  |  |
| 1/1/73 0:00 | 1,716.6 |  |  |  |  |  |  |
| 1/15/73 0:00 | 1,716.6 |  |  |  |  |  |  |
| 2/1/73 0:00 | 1,716.6 |  |  |  |  |  |  |
| 2/15/73 0:00 | 1,716.5 |  |  |  |  |  |  |
| 3/1/73 0:00 | 1,716.7 |  |  |  |  |  |  |
| 3/15/73 0:00 | 1,717.7 |  |  |  |  |  |  |
| 4/1/73 0:00 | 1,717.6 |  |  |  |  |  |  |
| 4/15/73 0:00 | 1,717.1 |  |  |  |  |  |  |
| 5/1/73 0:00 | 1,717.2 |  |  |  |  |  |  |
| 5/15/73 0:00 | 1,716.8 |  |  |  |  |  |  |
| 6/1/73 0:00 | 1,716.9 |  |  |  |  |  |  |
| 6/15/73 0:00 | 1,716.2 |  |  |  |  |  |  |
| 7/1/73 0:00 | 1,716.2 |  |  |  |  |  |  |
| 7/15/73 0:00 | 1,715.7 |  |  |  |  |  |  |
| 8/1/73 0:00 | 1,715.7 |  |  |  |  |  |  |
| 8/15/73 0:00 | 1,715.3 |  |  |  |  |  |  |
| 9/1/73 0:00 | 1,715.4 |  |  |  |  |  |  |
| 9/15/73 0:00 | 1,715.2 |  |  |  |  |  |  |
| 10/1/73 0:00 | 1,715.2 |  |  |  |  |  |  |
| 10/15/73 0:00 | 1,715.1 |  |  |  |  |  |  |
| 11/1/73 0:00 | 1,715.1 |  |  |  |  |  |  |
| 11/15/73 0:00 | 1,715.0 |  |  |  |  |  |  |
| 12/1/73 0:00 | 1,715.1 |  |  |  |  |  |  |
| 12/15/73 0:00 | 1,715.0 |  |  |  |  |  |  |
| 1/1/74 0:00 | 1,715.2 |  |  |  |  |  |  |
| 1/15/74 0:00 | 1,715.1 |  |  |  |  |  |  |
| 2/1/74 0:00 | 1,715.2 |  |  |  |  |  |  |
| 2/15/74 0:00 | 1,715.2 |  |  |  |  |  |  |
| 3/1/74 0:00 | 1,715.2 |  |  |  |  |  |  |
| 3/15/74 0:00 | 1,715.5 |  |  |  |  |  |  |
| 4/1/74 0:00 | 1,716.0 |  |  |  |  |  |  |
| 4/15/74 0:00 | 1,715.6 |  |  |  |  |  |  |
| 5/1/74 0:00 | 1,716.0 |  |  |  |  |  |  |
| 5/15/74 0:00 | 1,715.9 |  |  |  |  |  |  |
| 6/1/74 0:00 | 1,716.1 |  |  |  |  |  |  |
| 6/15/74 0:00 | 1,715.7 |  |  |  |  |  |  |
| 7/1/74 0:00 | 1,715.7 |  |  |  |  |  |  |
| 7/15/74 0:00 | 1,715.4 |  |  |  |  |  |  |
| 8/1/74 0:00 | 1,715.4 |  |  |  |  |  |  |
| 8/15/74 0:00 | 1,714.9 |  |  |  |  |  |  |
| 9/1/74 0:00 | 1,714.9 |  |  |  |  |  |  |
| 9/15/74 0:00 | 1,714.4 |  |  |  |  |  |  |
| 10/1/74 0:00 | 1,714.4 |  |  |  |  |  |  |
| 10/15/74 0:00 | 1,714.1 |  |  |  |  |  |  |
| 11/1/74 0:00 | 1,714.2 |  |  |  |  |  |  |
| 11/15/74 0:00 | 1,714.2 |  |  |  |  |  |  |
| 12/1/74 0:00 | 1,714.2 |  |  |  |  |  |  |
| 12/15/74 0:00 | 1,714.1 |  |  |  |  |  |  |
| 1/1/75 0:00 | 1,714.1 |  |  |  |  |  |  |
| 1/15/75 0:00 | 1,714.1 |  |  |  |  |  |  |
| 2/1/75 0:00 | 1,713.9 |  |  |  |  |  |  |
| 2/15/75 0:00 | 1,713.9 |  |  |  |  |  |  |
| 3/1/75 0:00 | 1,714.2 |  |  |  |  |  |  |
| 3/15/75 0:00 | 1,714.4 |  |  |  |  |  |  |
| 4/15/75 0:00 | 1,714.4 |  |  |  |  |  |  |
| 4/17/75 8:00 | 1,714.7 |  |  |  |  |  |  |
| 4/17/75 11:45 | 1,714.7 |  |  |  |  |  |  |
| 4/17/75 20:00 | 1,714.8 |  |  |  |  |  |  |
| 4/18/75 6:00 | 1,714.8 |  |  |  |  |  |  |
| 4/18/75 13:00 | 1,714.9 |  |  |  |  |  |  |
| 4/18/75 18:30 | 1,714.9 |  |  |  |  |  |  |
| 4/18/75 21:30 | 1,714.9 |  |  |  |  |  |  |
| 4/19/75 6:30 | 1,714.9 |  |  |  |  |  |  |
| 4/19/75 10:25 | 1,714.9 |  |  |  |  |  |  |
| 4/19/75 17:30 | 1,715.0 |  |  |  |  |  |  |
| 4/19/75 21:00 | 1,715.0 |  |  |  |  |  |  |
| 4/20/75 7:00 | 1,715.0 |  |  |  |  |  |  |
| 4/20/75 17:00 | 1,715.0 |  |  |  |  |  |  |
| 4/21/75 7:00 | 1,715.1 |  |  |  |  |  |  |
| 4/21/75 6:00 | 1,715.1 |  |  |  |  |  |  |
| 4/22/75 9:00 | 1,715.1 |  |  |  |  |  |  |
| 4/22/75 18:45 | 1,715.2 |  |  |  |  |  |  |
| 4/23/75 9:30 | 1,715.2 |  |  |  |  |  |  |
| 4/23/75 19:30 | 1,715.3 |  |  |  |  |  |  |
| 4/24/75 7:00 | 1,715.3 |  |  |  |  |  |  |
| 4/24/75 16:30 | 1,715.3 |  |  |  |  |  |  |
| 4/25/75 7:00 | 1,715.3 |  |  |  |  |  |  |
| 4/25/75 18:00 | 1,715.3 |  |  |  |  |  |  |
| 4/26/75 8:00 | 1,715.3 |  |  |  |  |  |  |
| 4/26/75 16:30 | 1,715.3 |  |  |  |  |  |  |
| 4/27/75 7:00 | 1,715.3 |  |  |  |  |  |  |
| 4/27/75 20:00 | 1,715.4 |  |  |  |  |  |  |
| 4/28/75 9:00 | 1,715.4 |  |  |  |  |  |  |
| 4/28/75 19:00 | 1,715.4 |  |  |  |  |  |  |
| 4/29/75 7:00 | 1,715.5 |  |  |  |  |  |  |
| 4/29/75 19:00 | 1,715.5 |  |  |  |  |  |  |
| 4/30/75 8:05 | 1,715.6 |  |  |  |  |  |  |
| 4/30/75 17:45 | 1,715.6 |  |  |  |  |  |  |
| 5/1/75 7:00 | 1,715.6 |  |  |  |  |  |  |
| 5/2/75 8:15 | 1,715.6 |  |  |  |  |  |  |
| 5/2/75 12:00 | 1,715.7 |  |  |  |  |  |  |
| 5/2/75 17:45 | 1,715.6 |  |  |  |  |  |  |
| 5/3/75 8:00 | 1,715.6 |  |  |  |  |  |  |
| 5/4/75 7:00 | 1,715.6 |  |  |  |  |  |  |
| 5/5/75 17:00 | 1,715.6 |  |  |  |  |  |  |
| 5/6/75 18:30 | 1,715.6 |  |  |  |  |  |  |
| 5/7/75 7:30 | 1,715.7 |  |  |  |  |  |  |
| 5/9/75 0:00 | 1,715.7 |  |  |  |  |  |  |
| 5/10/75 0:00 | 1,715.7 |  |  |  |  |  |  |
| 5/11/75 0:00 | 1,715.7 |  |  |  |  |  |  |
| 5/12/75 19:00 | 1,715.8 |  |  |  |  |  |  |
| 5/13/75 0:00 | 1,715.7 |  |  |  |  |  |  |
| 5/20/75 0:00 | 1,715.7 |  |  |  |  |  |  |
| 5/26/75 0:00 | 1,715.8 |  |  |  |  |  |  |
| 6/1/75 0:00 | 1,715.8 |  |  |  |  |  |  |
| 6/15/75 0:00 | 1,715.6 |  |  |  |  |  |  |
| 7/1/75 0:00 | 1,715.6 |  |  |  |  |  |  |
| 7/15/75 0:00 | 1,715.2 |  |  |  |  |  |  |
| 8/1/75 0:00 | 1,715.2 |  |  |  |  |  |  |
| 8/15/75 0:00 | 1,714.7 |  |  |  |  |  |  |
| 9/1/75 0:00 | 1,714.7 |  |  |  |  |  |  |
| 9/15/75 0:00 | 1,714.4 |  |  |  |  |  |  |
| 10/1/75 0:00 | 1,714.4 |  |  |  |  |  |  |
| 10/15/75 0:00 | 1,714.1 |  |  |  |  |  |  |
| 11/1/75 0:00 | 1,714.1 |  |  |  |  |  |  |
| 11/15/75 0:00 | 1,714.1 |  |  |  |  |  |  |
| 12/1/75 0:00 | 1,714.1 |  |  |  |  |  |  |
| 12/15/75 0:00 | 1,714.0 |  |  |  |  |  |  |
| 1/1/76 0:00 | 1,714.1 |  |  |  |  |  |  |
| 1/15/76 0:00 | 1,714.0 |  |  |  |  |  |  |
| 2/1/76 0:00 | 1,714.1 |  |  |  |  |  |  |
| 2/15/76 0:00 | 1,714.3 |  |  |  |  |  |  |
| 3/1/76 0:00 | 1,714.2 |  |  |  |  |  |  |
| 3/15/76 0:00 | 1,715.2 |  |  |  |  |  |  |
| 4/1/76 0:00 | 1,715.3 |  |  |  |  |  |  |
| 4/15/76 0:00 | 1,715.2 |  |  |  |  |  |  |
| 5/1/76 0:00 | 1,715.2 |  |  |  |  |  |  |
| 5/15/76 0:00 | 1,714.8 |  |  |  |  |  |  |
| 6/1/76 0:00 | 1,714.8 |  |  |  |  |  |  |
| 6/15/76 0:00 | 1,714.3 |  |  |  |  |  |  |
| 7/1/76 0:00 | 1,714.1 |  |  |  |  |  |  |
| 7/15/76 0:00 | 1,713.8 |  |  |  |  |  |  |
| 8/1/76 0:00 | 1,713.5 |  |  |  |  |  |  |
| 8/15/76 0:00 | 1,713.0 |  |  |  |  |  |  |
| 9/1/76 0:00 | 1,712.8 |  |  |  |  |  |  |
| 9/15/76 0:00 | 1,712.5 |  |  |  |  |  |  |
| 10/1/76 0:00 | 1,712.5 |  |  |  |  |  |  |
| 10/15/76 0:00 | 1,712.3 |  |  |  |  |  |  |
| 11/1/76 0:00 | 1,712.3 |  |  |  |  |  |  |
| 11/15/76 0:00 | 1,712.2 |  |  |  |  |  |  |
| 12/1/76 0:00 | 1,712.2 |  |  |  |  |  |  |
| 12/15/76 0:00 | 1,712.2 |  |  |  |  |  |  |
| 1/1/77 0:00 | 1,712.2 |  |  |  |  |  |  |
| 1/15/77 0:00 | 1,712.2 |  |  |  |  |  |  |
| 2/1/77 0:00 | 1,712.2 |  |  |  |  |  |  |
| 2/15/77 0:00 | 1,712.3 |  |  |  |  |  |  |
| 3/1/77 0:00 | 1,712.7 |  |  |  |  |  |  |
| 3/15/77 0:00 | 1,713.6 |  |  |  |  |  |  |
| 4/1/77 0:00 | 1,714.1 |  |  |  |  |  |  |
| 4/15/77 0:00 | 1,713.6 |  |  |  |  |  |  |
| 5/1/77 0:00 | 1,713.7 |  |  |  |  |  |  |
| 5/15/77 0:00 | 1,714.0 |  |  |  |  |  |  |
| 6/1/77 0:00 | 1,714.3 |  |  |  |  |  |  |
| 6/15/77 0:00 | 1,713.6 |  |  |  |  |  |  |
| 7/1/77 0:00 | 1,714.3 |  |  |  |  |  |  |
| 7/15/77 0:00 | 1,714.0 |  |  |  |  |  |  |
| 8/1/77 0:00 | 1,713.9 |  |  |  |  |  |  |
| 8/15/77 0:00 | 1,713.6 |  |  |  |  |  |  |
| 9/1/77 0:00 | 1,713.8 |  |  |  |  |  |  |
| 9/15/77 0:00 | 1,713.6 |  |  |  |  |  |  |
| 10/1/77 0:00 | 1,713.9 |  |  |  |  |  |  |
| 10/15/77 0:00 | 1,713.7 |  |  |  |  |  |  |
| 11/1/77 0:00 | 1,714.2 |  |  |  |  |  |  |
| 11/15/77 0:00 | 1,713.9 |  |  |  |  |  |  |
| 12/1/77 0:00 | 1,714.3 |  |  |  |  |  |  |
| 12/15/77 0:00 | 1,714.2 |  |  |  |  |  |  |
| 1/1/78 0:00 | 1,714.4 |  |  |  |  |  |  |
| 1/15/78 0:00 | 1,714.3 |  |  |  |  |  |  |
| 2/1/78 0:00 | 1,714.4 |  |  |  |  |  |  |
| 2/15/78 0:00 | 1,714.4 |  |  |  |  |  |  |
| 3/15/78 0:00 | 1,714.4 |  |  |  |  |  |  |
| 3/30/78 6:10 | 1,715.5 |  |  |  |  |  |  |
| 3/30/78 16:00 | 1,715.8 |  |  |  |  |  |  |
| 3/30/78 21:00 | 1,716.0 |  |  |  |  |  |  |
| 3/31/78 6:30 | 1,716.4 |  |  |  |  |  |  |
| 3/31/78 16:00 | 1,717.0 |  |  |  |  |  |  |
| 3/31/78 21:00 | 1,717.3 |  |  |  |  |  |  |
| 4/1/78 6:30 | 1,718.2 |  |  |  |  |  |  |
| 4/1/78 15:00 | 1,718.6 |  |  |  |  |  |  |
| 4/1/78 20:30 | 1,718.8 |  |  |  |  |  |  |
| 4/2/78 6:30 | 1,718.9 |  |  |  |  |  |  |
| 4/2/78 12:00 | 1,719.0 |  |  |  |  |  |  |
| 4/2/78 21:00 | 1,719.2 |  |  |  |  |  |  |
| 4/3/78 6:45 | 1,719.2 |  |  |  |  |  |  |
| 4/3/78 16:05 | 1,719.2 |  |  |  |  |  |  |
| 4/4/78 6:30 | 1,719.2 |  |  |  |  |  |  |
| 4/4/78 16:00 | 1,719.3 |  |  |  |  |  |  |
| 4/5/78 6:45 | 1,719.2 |  |  |  |  |  |  |
| 4/6/78 6:40 | 1,719.3 |  |  |  |  |  |  |
| 4/6/78 19:30 | 1,719.3 |  |  |  |  |  |  |
| 4/7/78 6:45 | 1,719.3 |  |  |  |  |  |  |
| 4/8/78 6:45 | 1,719.3 |  |  |  |  |  |  |
| 4/8/78 19:00 | 1,719.4 |  |  |  |  |  |  |
| 4/9/78 8:00 | 1,719.4 |  |  |  |  |  |  |
| 4/10/78 6:30 | 1,719.4 |  |  |  |  |  |  |
| 4/10/78 16:00 | 1,719.4 |  |  |  |  |  |  |
| 4/11/78 12:00 | 1,719.4 |  |  |  |  |  |  |
| 4/12/78 6:30 | 1,719.4 |  |  |  |  |  |  |
| 4/14/78 6:00 | 1,719.0 |  |  |  |  |  |  |
| 4/15/78 6:15 | 1,719.3 |  |  |  |  |  |  |
| 4/17/78 7:00 | 1,719.2 |  |  |  |  |  |  |
| 4/20/78 7:15 | 1,719.2 |  |  |  |  |  |  |
| 4/21/78 13:00 | 1,719.1 |  |  |  |  |  |  |
| 4/23/78 14:30 | 1,719.2 |  |  |  |  |  |  |
| 4/24/78 6:30 | 1,719.2 |  |  |  |  |  |  |
| 4/26/78 12:00 | 1,719.2 |  |  |  |  |  |  |
| 4/27/78 16:00 | 1,719.2 |  |  |  |  |  |  |
| 4/28/78 9:00 | 1,719.2 |  |  |  |  |  |  |
| 4/29/78 6:30 | 1,719.1 |  |  |  |  |  |  |
| 4/30/78 6:30 | 1,719.1 |  |  |  |  |  |  |
| 5/1/78 6:30 | 1,719.1 |  |  |  |  |  |  |
| 5/2/78 6:30 | 1,719.0 |  |  |  |  |  |  |
| 5/4/78 6:30 | 1,719.0 |  |  |  |  |  |  |
| 5/5/78 7:00 | 1,718.9 |  |  |  |  |  |  |
| 5/6/78 7:30 | 1,718.8 |  |  |  |  |  |  |
| 5/8/78 7:15 | 1,718.9 |  |  |  |  |  |  |
| 5/9/78 7:30 | 1,718.9 |  |  |  |  |  |  |
| 5/10/78 13:00 | 1,718.8 |  |  |  |  |  |  |
| 5/11/78 18:00 | 1,718.9 |  |  |  |  |  |  |
| 5/13/78 19:00 | 1,718.8 |  |  |  |  |  |  |
| 5/16/78 7:00 | 1,718.7 |  |  |  |  |  |  |
| 5/18/78 6:30 | 1,718.6 |  |  |  |  |  |  |
| 5/21/78 0:00 | 1,718.6 |  |  |  |  |  |  |
| 5/29/78 8:30 | 1,718.6 |  |  |  |  |  |  |
| 6/2/78 7:00 | 1,718.4 |  |  |  |  |  |  |
| 6/6/78 6:30 | 1,718.2 |  |  |  |  |  |  |
| 6/18/78 0:00 | 1,718.1 |  |  |  |  |  |  |
| 7/1/78 7:30 | 1,717.9 |  |  |  |  |  |  |
| 7/2/78 7:15 | 1,718.0 |  |  |  |  |  |  |
| 7/3/78 7:00 | 1,718.0 |  |  |  |  |  |  |
| 7/4/78 7:30 | 1,718.0 |  |  |  |  |  |  |
| 7/5/78 6:30 | 1,718.0 |  |  |  |  |  |  |
| 7/6/78 6:45 | 1,718.0 |  |  |  |  |  |  |
| 7/7/78 6:40 | 1,718.0 |  |  |  |  |  |  |
| 7/8/78 8:00 | 1,717.9 |  |  |  |  |  |  |
| 7/10/78 11:00 | 1,717.9 |  |  |  |  |  |  |
| 7/12/78 7:30 | 1,718.0 |  |  |  |  |  |  |
| 7/15/78 7:00 | 1,717.8 |  |  |  |  |  |  |
| 7/20/78 6:30 | 1,717.8 |  |  |  |  |  |  |
| 7/22/78 7:00 | 1,717.8 |  |  |  |  |  |  |
| 7/23/78 7:00 | 1,717.8 |  |  |  |  |  |  |
| 7/24/78 6:15 | 1,717.8 |  |  |  |  |  |  |
| 7/25/78 7:45 | 1,717.8 |  |  |  |  |  |  |
| 7/26/78 7:30 | 1,717.7 |  |  |  |  |  |  |
| 7/27/78 8:10 | 1,717.7 |  |  |  |  |  |  |
| 8/1/78 11:30 | 1,717.5 |  |  |  |  |  |  |
| 8/4/78 12:30 | 1,717.6 |  |  |  |  |  |  |
| 8/5/78 6:30 | 1,717.5 |  |  |  |  |  |  |
| 8/10/78 6:30 | 1,717.4 |  |  |  |  |  |  |
| 8/12/78 6:30 | 1,717.4 |  |  |  |  |  |  |
| 8/13/78 9:30 | 1,717.3 |  |  |  |  |  |  |
| 8/14/78 8:00 | 1,717.3 |  |  |  |  |  |  |
| 8/15/78 7:30 | 1,717.4 |  |  |  |  |  |  |
| 8/16/78 8:00 | 1,717.4 |  |  |  |  |  |  |
| 8/17/78 6:00 | 1,717.4 |  |  |  |  |  |  |
| 8/18/78 6:30 | 1,717.4 |  |  |  |  |  |  |
| 8/19/78 6:20 | 1,717.3 |  |  |  |  |  |  |
| 8/20/78 11:30 | 1,717.4 |  |  |  |  |  |  |
| 8/22/78 9:30 | 1,717.4 |  |  |  |  |  |  |
| 8/25/78 12:30 | 1,717.2 |  |  |  |  |  |  |
| 8/27/78 8:30 | 1,717.2 |  |  |  |  |  |  |
| 8/27/78 4:30 | 1,717.3 |  |  |  |  |  |  |
| 8/28/78 6:30 | 1,717.5 |  |  |  |  |  |  |
| 8/28/78 15:50 | 1,717.7 |  |  |  |  |  |  |
| 8/28/78 20:15 | 1,717.7 |  |  |  |  |  |  |
| 8/29/78 6:30 | 1,717.6 |  |  |  |  |  |  |
| 8/29/78 16:00 | 1,717.8 |  |  |  |  |  |  |
| 8/29/78 20:45 | 1,717.8 |  |  |  |  |  |  |
| 8/30/78 18:30 | 1,717.7 |  |  |  |  |  |  |
| 8/31/78 6:30 | 1,717.8 |  |  |  |  |  |  |
| 9/1/78 6:30 | 1,717.8 |  |  |  |  |  |  |
| 9/2/78 8:00 | 1,717.8 |  |  |  |  |  |  |
| 9/3/78 8:30 | 1,717.8 |  |  |  |  |  |  |
| 9/4/78 7:30 | 1,717.7 |  |  |  |  |  |  |
| 9/6/78 21:00 | 1,717.7 |  |  |  |  |  |  |
| 9/8/78 19:00 | 1,717.6 |  |  |  |  |  |  |
| 9/12/78 13:00 | 1,717.6 |  |  |  |  |  |  |
| 9/15/78 10:00 | 1,717.7 |  |  |  |  |  |  |
| 9/17/78 6:45 | 1,718.0 |  |  |  |  |  |  |
| 9/18/78 13:30 | 1,718.0 |  |  |  |  |  |  |
| 9/21/78 7:30 | 1,718.0 |  |  |  |  |  |  |
| 9/22/78 6:30 | 1,718.0 |  |  |  |  |  |  |
| 9/24/78 10:30 | 1,718.0 |  |  |  |  |  |  |
| 9/25/78 8:00 | 1,718.0 |  |  |  |  |  |  |
| 9/26/78 9:00 | 1,718.0 |  |  |  |  |  |  |
| 9/27/78 10:00 | 1,718.0 |  |  |  |  |  |  |
| 9/28/78 11:30 | 1,717.6 |  |  |  |  |  |  |
| 9/29/78 7:00 | 1,717.6 |  |  |  |  |  |  |
| 9/30/78 10:00 | 1,717.6 |  |  |  |  |  |  |
| 10/2/78 11:30 | 1,717.5 |  |  |  |  |  |  |
| 10/5/78 8:00 | 1,717.5 |  |  |  |  |  |  |
| 10/13/78 7:30 | 1,717.3 |  |  |  |  |  |  |
| 10/16/78 7:30 | 1,717.3 |  |  |  |  |  |  |
| 10/18/78 7:30 | 1,717.3 |  |  |  |  |  |  |
| 10/22/78 7:30 | 1,717.2 |  |  |  |  |  |  |
| 10/26/78 11:30 | 1,717.2 |  |  |  |  |  |  |
| 10/28/78 10:00 | 1,717.3 |  |  |  |  |  |  |
| 11/4/78 8:00 | 1,717.2 |  |  |  |  |  |  |
| 11/6/78 7:30 | 1,717.2 |  |  |  |  |  |  |
| 11/8/78 7:15 | 1,717.2 |  |  |  |  |  |  |
| 11/15/78 16:30 | 1,717.1 |  |  |  |  |  |  |
| 11/21/78 13:30 | 1,717.1 |  |  |  |  |  |  |
| 11/25/78 10:30 | 1,717.1 |  |  |  |  |  |  |
| 11/26/78 11:00 | 1,717.1 |  |  |  |  |  |  |
| 12/8/78 7:00 | 1,717.1 |  |  |  |  |  |  |
| 12/24/78 15:00 | 1,717.1 |  |  |  |  |  |  |
| 12/25/78 17:30 | 1,717.1 |  |  |  |  |  |  |
| 12/31/78 8:00 | 1,717.1 |  |  |  |  |  |  |
| 1/1/79 0:00 | 1,716.8 |  |  |  |  |  |  |
| 1/15/79 0:00 | 1,716.8 |  |  |  |  |  |  |
| 2/1/79 0:00 | 1,717.0 |  |  |  |  |  |  |
| 2/15/79 0:00 | 1,717.0 |  |  |  |  |  |  |
| 3/1/79 0:00 | 1,717.0 |  |  |  |  |  |  |
| 3/15/79 0:00 | 1,717.5 |  |  |  |  |  |  |
| 4/1/79 0:00 | 1,717.5 |  |  |  |  |  |  |
| 4/15/79 0:00 | 1,720.3 |  |  |  |  |  |  |
| 5/1/79 0:00 | 1,719.3 |  |  |  |  |  |  |
| 5/15/79 0:00 | 1,718.5 |  |  |  |  |  |  |
| 6/1/79 0:00 | 1,718.4 |  |  |  |  |  |  |
| 6/15/79 0:00 | 1,718.0 |  |  |  |  |  |  |
| 7/1/79 0:00 | 1,718.0 |  |  |  |  |  |  |
| 7/15/79 0:00 | 1,718.4 |  |  |  |  |  |  |
| 8/1/79 0:00 | 1,718.3 |  |  |  |  |  |  |
| 8/15/79 0:00 | 1,718.0 |  |  |  |  |  |  |
| 9/1/79 0:00 | 1,718.1 |  |  |  |  |  |  |
| 9/15/79 0:00 | 1,717.8 |  |  |  |  |  |  |
| 10/1/79 0:00 | 1,717.7 |  |  |  |  |  |  |
| 10/15/79 0:00 | 1,717.5 |  |  |  |  |  |  |
| 11/1/79 0:00 | 1,718.6 |  |  |  |  |  |  |
| 11/15/79 0:00 | 1,717.5 |  |  |  |  |  |  |
| 12/1/79 0:00 | 1,717.6 |  |  |  |  |  |  |
| 12/15/79 0:00 | 1,718.6 |  |  |  |  |  |  |
| 1/1/80 6:00 | 1,717.9 | 40 |  |  |  |  |  |
| 1/13/80 6:15 | 1,717.9 | 39 |  |  |  |  |  |
| 1/24/80 6:15 | 1,717.9 | 40 |  |  |  |  |  |
| 1/27/80 6:30 | 1,717.9 |  |  |  |  |  |  |
| 2/2/80 12:00 | 1,717.9 | 38 |  |  |  |  |  |
| 2/4/80 11:30 | 1,717.9 | 40 |  |  |  |  |  |
| 2/17/80 10:00 | 1,717.9 | 40 |  |  |  |  |  |
| 2/25/80 6:15 | 1,717.9 |  |  |  |  |  |  |
| 3/10/80 13:00 | 1,717.9 | 39 |  |  |  |  |  |
| 3/16/80 8:30 | 1,718.0 | 40 |  |  |  |  |  |
| 3/18/80 6:15 | 1,717.9 |  |  |  |  |  |  |
| 3/19/80 6:30 | 1,717.9 |  |  |  |  |  |  |
| 3/21/80 10:45 | 1,718.1 |  |  |  |  |  |  |
| 3/22/80 6:15 | 1,718.2 |  |  |  |  |  |  |
| 3/24/80 0:00 | 1,718.2 |  |  |  |  |  |  |
| 3/25/80 0:00 | 1,718.3 |  |  |  |  |  |  |
| 3/27/80 0:00 | 1,718.4 |  |  |  |  |  |  |
| 3/30/80 6:00 | 1,718.5 |  |  |  |  |  |  |
| 3/31/80 6:30 | 1,718.5 | 41 |  |  |  |  |  |
| 4/1/80 6:30 | 1,718.5 |  |  |  |  |  |  |
| 4/4/80 13:00 | 1,718.6 | 42 |  |  |  |  |  |
| 4/6/80 6:15 | 1,718.5 |  |  |  |  |  |  |
| 4/7/80 11:00 | 1,718.6 |  |  |  |  |  |  |
| 4/8/80 6:10 | 1,718.6 | 42 |  |  |  |  |  |
| 4/9/80 6:30 | 1,718.7 |  |  |  |  |  |  |
| 4/10/80 6:30 | 1,718.6 | 43 |  |  |  |  |  |
| 4/11/80 11:00 | 1,718.6 | 43 |  |  |  |  |  |
| 4/14/80 10:30 | 1,718.6 | 43 |  |  |  |  |  |
| 4/17/80 7:00 | 1,718.5 | 43 |  |  |  |  |  |
| 4/19/80 6:15 | 1,718.5 |  |  |  |  |  |  |
| 4/20/80 6:50 | 1,718.4 | 44 |  |  |  |  |  |
| 4/24/80 10:30 | 1,718.4 |  |  |  |  |  |  |
| 4/26/80 6:30 | 1,718.4 | 54 |  |  |  |  |  |
| 4/27/80 6:30 | 1,718.4 | 52 |  |  |  |  |  |
| 4/29/80 6:30 | 1,718.4 | 56 |  |  |  |  |  |
| 5/2/80 6:45 | 1,718.4 | 58 |  |  |  |  |  |
| 5/3/80 11:00 | 1,718.3 |  |  |  |  |  |  |
| 5/6/80 7:30 | 1,718.2 | 64 |  |  |  |  |  |
| 5/10/80 6:15 | 1,718.2 | 64 |  |  |  |  |  |
| 5/11/80 6:20 | 1,718.2 | 56 |  |  |  |  |  |
| 5/15/80 6:30 | 1,718.3 | 56 |  |  |  |  |  |
| 5/18/80 6:30 | 1,718.2 | 56 |  |  |  |  |  |
| 5/20/80 6:15 | 1,718.2 | 58 |  |  |  |  |  |
| 5/23/80 7:30 | 1,718.0 |  |  |  |  |  |  |
| 5/29/80 6:30 | 1,718.1 | 68 |  |  |  |  |  |
| 6/1/80 7:15 | 1,718.1 | 69 |  |  |  |  |  |
| 6/4/80 7:00 | 1,718.1 |  |  |  |  |  |  |
| 6/4/80 16:00 | 1,718.2 |  |  |  |  |  |  |
| 6/5/80 10:30 | 1,718.3 |  |  |  |  |  |  |
| 6/6/80 7:30 | 1,718.3 | 68 |  |  |  |  |  |
| 6/7/80 7:00 | 1,718.5 | 67 |  |  |  |  |  |
| 6/8/80 6:15 | 1,718.5 | 66 |  |  |  |  |  |
| 6/10/80 10:00 | 1,718.5 | 67 |  |  |  |  |  |
| 6/12/80 6:10 | 1,718.5 | 68 |  |  |  |  |  |
| 6/13/80 6:30 | 1,718.5 | 69 |  |  |  |  |  |
| 6/16/80 7:30 | 1,718.5 | 70 |  |  |  |  |  |
| 6/19/80 7:00 | 1,718.4 | 72 |  |  |  |  |  |
| 6/24/80 13:00 | 1,718.3 | 73 |  |  |  |  |  |
| 6/26/80 6:30 | 1,718.4 |  |  |  |  |  |  |
| 6/28/80 6:30 | 1,718.4 |  |  |  |  |  |  |
| 6/30/80 6:30 | 1,718.4 |  |  |  |  |  |  |
| 7/7/80 7:00 | 1,718.4 | 72 |  |  |  |  |  |
| 7/9/80 6:30 | 1,718.3 | 74 |  |  |  |  |  |
| 7/12/80 6:30 | 1,718.3 |  |  |  |  |  |  |
| 7/13/80 6:30 | 1,718.2 | 77 |  |  |  |  |  |
| 7/21/80 6:30 | 1,718.1 |  |  |  |  |  |  |
| 7/26/80 11:00 | 1,718.0 | 74 |  |  |  |  |  |
| 7/28/80 7:00 | 1,717.8 | 74 |  |  |  |  |  |
| 7/30/80 7:30 | 1,717.8 | 74 |  |  |  |  |  |
| 8/1/80 7:45 | 1,717.8 | 74 |  |  |  |  |  |
| 8/2/80 7:45 | 1,717.9 |  |  |  |  |  |  |
| 8/10/80 9:00 | 1,717.8 |  |  |  |  |  |  |
| 8/11/80 12:30 | 1,717.8 |  |  |  |  |  |  |
| 8/18/80 7:15 | 1,717.8 | 68 |  |  |  |  |  |
| 8/19/80 7:45 | 1,717.9 | 68 |  |  |  |  |  |
| 8/20/80 7:00 | 1,717.9 | 70 |  |  |  |  |  |
| 8/23/80 7:00 | 1,717.8 | 70 |  |  |  |  |  |
| 8/28/80 6:30 | 1,717.7 | 68 |  |  |  |  |  |
| 8/31/80 8:15 | 1,717.7 | 70 |  |  |  |  |  |
| 9/6/80 6:15 | 1,717.6 | 69 |  |  |  |  |  |
| 9/9/80 7:30 | 1,717.6 | 69 |  |  |  |  |  |
| 9/11/80 7:10 | 1,717.5 | 68 |  |  |  |  |  |
| 9/14/80 6:30 | 1,717.4 | 65 |  |  |  |  |  |
| 9/20/80 6:30 | 1,717.5 |  |  |  |  |  |  |
| 9/24/80 10:00 | 1,717.3 | 58 |  |  |  |  |  |
| 9/27/80 11:00 | 1,717.3 |  |  |  |  |  |  |
| 9/30/80 12:45 | 1,717.3 | 58 |  |  |  |  |  |
| 10/3/80 11:30 | 1,717.3 | 55 |  |  |  |  |  |
| 10/9/80 6:30 | 1,717.2 | 56 |  |  |  |  |  |
| 10/13/80 7:00 | 1,717.1 | 50 |  |  |  |  |  |
| 10/18/80 6:30 | 1,717.1 | 49 |  |  |  |  |  |
| 10/20/80 7:00 | 1,717.1 | 45 |  |  |  |  |  |
| 10/21/80 10:00 | 1,717.1 | 45 |  |  |  |  |  |
| 10/28/80 6:30 | 1,717.1 | 40 |  |  |  |  |  |
| 11/2/80 6:00 | 1,717.1 | 42 |  |  |  |  |  |
| 11/11/80 8:40 | 1,717.1 | 42 |  |  |  |  |  |
| 11/19/80 6:30 | 1,717.0 | 37 |  |  |  |  |  |
| 11/27/80 11:00 | 1,717.0 | 40 |  |  |  |  |  |
| 12/10/80 12:00 | 1,717.0 | 35 |  |  |  |  |  |
| 12/30/80 7:00 | 1,717.0 |  |  |  |  |  |  |
| 1/5/81 10:00 | 1,717.0 |  |  |  |  |  |  |
| 1/20/81 10:00 | 1,716.9 | 40 |  |  |  |  |  |
| 1/26/81 8:00 | 1,716.9 | 38 |  |  |  |  |  |
| 2/2/81 11:00 | 1,716.9 | 40 |  |  |  |  |  |
| 2/9/81 17:30 | 1,716.8 | 41 |  |  |  |  |  |
| 2/18/81 6:30 | 1,716.9 | 42 |  |  |  |  |  |
| 2/24/81 10:15 | 1,716.9 | 42 |  |  |  |  |  |
| 3/3/81 6:30 | 1,716.9 |  |  |  |  |  |  |
| 3/6/81 11:00 | 1,716.8 | 44 |  |  |  |  |  |
| 3/14/81 10:00 | 1,716.9 | 44 |  |  |  |  |  |
| 3/25/81 10:30 | 1,716.9 |  |  |  |  |  |  |
| 3/31/81 6:30 | 1,716.9 | 48 |  |  |  |  |  |
| 4/4/81 9:30 | 1,716.9 | 45 |  |  |  |  |  |
| 4/11/81 6:10 | 1,716.9 | 49 |  |  |  |  |  |
| 4/17/81 6:45 | 1,716.9 | 51 |  |  |  |  |  |
| 4/19/81 9:00 | 1,716.8 |  |  |  |  |  |  |
| 4/22/81 6:15 | 1,716.8 | 51 |  |  |  |  |  |
| 4/25/81 10:00 | 1,716.8 | 52 |  |  |  |  |  |
| 5/1/81 10:00 | 1,716.7 | 56 |  |  |  |  |  |
| 5/3/81 6:30 | 1,716.8 | 58 |  |  |  |  |  |
| 5/9/81 6:40 | 1,716.6 | 56 |  |  |  |  |  |
| 5/14/81 6:15 | 1,716.6 | 58 |  |  |  |  |  |
| 5/20/81 6:30 | 1,716.4 |  |  |  |  |  |  |
| 5/24/81 12:00 | 1,716.5 | 58 |  |  |  |  |  |
| 5/29/81 19:00 | 1,716.5 |  |  |  |  |  |  |
| 5/31/81 6:15 | 1,716.4 | 64 |  |  |  |  |  |
| 6/7/81 8:20 | 1,716.3 | 68 |  |  |  |  |  |
| 6/10/81 7:30 | 1,716.3 | 68 |  |  |  |  |  |
| 6/12/81 7:00 | 1,716.3 |  |  |  |  |  |  |
| 6/13/81 6:30 | 1,716.3 |  |  |  |  |  |  |
| 6/21/81 8:40 | 1,716.3 | 66 |  |  |  |  |  |
| 6/25/81 6:15 | 1,716.2 | 68 |  |  |  |  |  |
| 6/26/81 6:30 | 1,716.2 | 68 |  |  |  |  |  |
| 6/30/81 7:00 | 1,716.2 | 72 |  |  |  |  |  |
| 7/4/81 7:30 | 1,716.2 | 72 |  |  |  |  |  |
| 7/9/81 7:00 | 1,716.2 | 76 |  |  |  |  |  |
| 7/12/81 6:00 | 1,716.0 | 74 |  |  |  |  |  |
| 7/19/81 8:15 | 1,716.0 | 76 |  |  |  |  |  |
| 7/22/81 12:30 | 1,716.0 | 76 |  |  |  |  |  |
| 7/23/81 7:15 | 1,716.2 | 75 |  |  |  |  |  |
| 7/24/81 7:15 | 1,716.2 | 75 |  |  |  |  |  |
| 7/25/81 8:00 | 1,716.2 |  |  |  |  |  |  |
| 7/26/81 12:00 | 1,716.1 |  |  |  |  |  |  |
| 7/28/81 10:00 | 1,716.1 | 72 |  |  |  |  |  |
| 8/8/81 10:15 | 1,716.0 | 72 |  |  |  |  |  |
| 8/11/81 10:15 | 1,715.9 | 70 |  |  |  |  |  |
| 8/19/81 6:30 | 1,715.8 | 70 |  |  |  |  |  |
| 8/27/81 6:30 | 1,715.7 |  |  |  |  |  |  |
| 8/31/81 7:45 | 1,715.7 |  |  |  |  |  |  |
| 9/4/81 7:30 | 1,715.6 | 66 |  |  |  |  |  |
| 9/8/81 14:30 | 1,715.6 |  |  |  |  |  |  |
| 9/16/81 10:30 | 1,715.4 | 66 |  |  |  |  |  |
| 9/22/81 12:30 | 1,715.4 | 62 |  |  |  |  |  |
| 9/29/81 11:00 | 1,715.2 | 56 |  |  |  |  |  |
| 10/1/81 6:00 | 1,715.2 | 56 |  |  |  |  |  |
| 10/8/81 6:30 | 1,715.2 |  |  |  |  |  |  |
| 10/11/81 6:15 | 1,715.2 | 52 |  |  |  |  |  |
| 10/16/81 9:30 | 1,715.2 | 52 |  |  |  |  |  |
| 10/22/81 12:30 | 1,715.2 | 45 |  |  |  |  |  |
| 10/30/81 10:30 | 1,715.2 | 48 |  |  |  |  |  |
| 11/3/81 11:30 | 1,715.2 | 46 |  |  |  |  |  |
| 11/12/81 6:10 | 1,715.2 | 41 |  |  |  |  |  |
| 11/24/81 10:10 | 1,715.1 | 37 |  |  |  |  |  |
| 12/2/81 6:30 | 1,715.0 |  |  |  |  |  |  |
| 12/15/81 6:00 | 1,715.0 | 37 |  |  |  |  |  |
| 12/23/81 10:00 | 1,715.0 | 39 |  |  |  |  |  |
| 1/2/82 0:00 |  | 39 |  |  |  |  |  |
| 1/8/82 0:00 | 1,715.0 | 39 |  |  |  |  |  |
| 1/14/82 0:00 | 1,715.1 | 38 |  |  |  |  |  |
| 2/10/82 0:00 | 1,715.1 |  |  |  |  |  |  |
| 2/19/82 0:00 | 1,715.2 | 40 |  |  |  |  |  |
| 2/24/82 13:00 | 1,715.1 |  |  |  |  |  |  |
| 3/5/82 12:30 | 1,715.1 | 40 |  |  |  |  |  |
| 3/14/82 0:00 | 1,715.2 |  |  |  |  |  |  |
| 3/16/82 6:30 | 1,715.2 | 42 |  |  |  |  |  |
| 3/21/82 8:30 | 1,715.2 | 42 |  |  |  |  |  |
| 3/24/82 12:30 | 1,715.2 | 42 |  |  |  |  |  |
| 3/26/82 10:00 | 1,715.2 | 42 |  |  |  |  |  |
| 3/29/82 6:00 | 1,715.3 |  |  |  |  |  |  |
| 3/31/82 15:30 | 1,715.6 |  |  |  |  |  |  |
| 4/1/82 6:30 | 1,715.8 |  |  |  |  |  |  |
| 4/2/82 6:40 | 1,716.0 |  |  |  |  |  |  |
| 4/3/82 8:30 | 1,716.1 |  |  |  |  |  |  |
| 4/5/82 12:00 | 1,716.1 | 37 |  |  |  |  |  |
| 4/6/82 6:30 | 1,716.2 |  |  |  |  |  |  |
| 4/7/82 6:30 | 1,716.2 |  |  |  |  |  |  |
| 4/9/82 6:10 | 1,716.2 |  |  |  |  |  |  |
| 4/11/82 6:15 | 1,716.2 |  |  |  |  |  |  |
| 4/14/82 6:15 | 1,716.3 | 40 |  |  |  |  |  |
| 4/17/82 13:00 | 1,716.4 | 40 |  |  |  |  |  |
| 4/19/82 7:30 | 1,716.4 | 46 |  |  |  |  |  |
| 4/22/82 7:00 | 1,716.4 | 42 |  |  |  |  |  |
| 4/25/82 10:30 | 1,716.4 |  |  |  |  |  |  |
| 4/27/82 6:30 | 1,716.3 | 50 |  |  |  |  |  |
| 4/29/82 11:00 | 1,716.3 |  |  |  |  |  |  |
| 5/4/82 6:15 | 1,716.3 | 50 |  |  |  |  |  |
| 5/6/82 13:00 | 1,716.3 | 58 |  |  |  |  |  |
| 5/8/82 11:00 | 1,716.3 | 57 |  |  |  |  |  |
| 5/9/82 13:00 | 1,716.4 | 59 |  |  |  |  |  |
| 5/12/82 6:30 | 1,716.4 | 59 |  |  |  |  |  |
| 5/13/82 7:00 | 1,716.4 | 58 |  |  |  |  |  |
| 5/15/82 7:30 | 1,716.5 | 58 |  |  |  |  |  |
| 5/17/82 8:00 | 1,716.6 | 60 |  |  |  |  |  |
| 5/21/82 6:30 | 1,716.7 |  |  |  |  |  |  |
| 5/25/82 6:15 | 1,716.6 | 61 |  |  |  |  |  |
| 5/27/82 7:30 | 1,716.6 | 61 |  |  |  |  |  |
| 5/28/82 6:15 | 1,716.6 | 61 |  |  |  |  |  |
| 5/30/82 8:00 | 1,716.7 | 62 |  |  |  |  |  |
| 5/31/82 6:40 | 1,716.7 | 62 |  |  |  |  |  |
| 6/1/82 6:20 | 1,716.8 | 60 |  |  |  |  |  |
| 6/1/82 8:00 | 1,716.8 |  |  |  |  |  |  |
| 6/2/82 6:30 | 1,716.8 | 60 |  |  |  |  |  |
| 6/3/82 7:00 | 1,716.8 | 60 |  |  |  |  |  |
| 6/4/82 6:00 | 1,716.8 | 60 |  |  |  |  |  |
| 6/7/82 7:00 | 1,716.8 |  |  |  |  |  |  |
| 6/9/82 7:30 | 1,716.8 | 63 |  |  |  |  |  |
| 6/18/82 7:00 | 1,716.8 | 62 |  |  |  |  |  |
| 6/23/82 0:00 |  | 64 |  |  |  |  |  |
| 6/29/82 11:00 |  | 69 |  |  |  |  |  |
| 7/6/82 10:30 | 1,716.6 | 72 |  |  |  |  |  |
| 7/8/82 6:30 | 1,716.6 | 72 |  |  |  |  |  |
| 7/15/82 12:55 | 1,716.6 | 73 |  |  |  |  |  |
| 7/23/82 7:00 |  | 75 |  |  |  |  |  |
| 7/27/82 13:00 | 1,716.4 |  |  |  |  |  |  |
| 8/2/82 7:00 | 1,716.3 |  |  |  |  |  |  |
| 8/5/82 11:10 | 1,716.4 | 75 |  |  |  |  |  |
| 8/9/82 6:30 | 1,716.2 |  |  |  |  |  |  |
| 8/16/82 6:30 | 1,716.0 | 71 |  |  |  |  |  |
| 8/18/82 12:30 | 1,716.0 | 72 |  |  |  |  |  |
| 8/24/82 19:30 | 1,715.9 | 73 |  |  |  |  |  |
| 9/1/82 16:00 | 1,715.8 | 70 |  |  |  |  |  |
| 9/7/82 12:30 | 1,715.7 | 69 |  |  |  |  |  |
| 9/10/82 6:15 | 1,715.7 |  |  |  |  |  |  |
| 9/13/82 7:30 | 1,715.8 | 66 |  |  |  |  |  |
| 9/15/82 12:00 | 1,715.8 | 62 |  |  |  |  |  |
| 9/21/82 11:45 | 1,715.6 | 57 |  |  |  |  |  |
| 9/28/82 13:00 | 1,715.6 |  |  |  |  |  |  |
| 10/1/82 10:00 | 1,715.6 | 58 |  |  |  |  |  |
| 10/3/82 8:30 | 1,715.7 | 56 |  |  |  |  |  |
| 10/8/82 6:30 | 1,715.6 | 52 |  |  |  |  |  |
| 10/9/82 8:00 | 1,715.6 |  |  |  |  |  |  |
| 10/10/82 7:00 | 1,715.8 | 51 |  |  |  |  |  |
| 10/13/82 6:30 | 1,715.7 | 48 |  |  |  |  |  |
| 10/18/82 11:30 | 1,715.8 | 50 |  |  |  |  |  |
| 10/20/82 11:00 | 1,715.8 | 47 |  |  |  |  |  |
| 10/30/82 9:00 | 1,715.7 | 46 |  |  |  |  |  |
| 11/6/82 0:00 |  | 38 |  |  |  |  |  |
| 11/8/82 11:30 | 1,715.8 | 37 |  |  |  |  |  |
| 11/17/82 1:00 | 1,715.8 | 37 |  |  |  |  |  |
| 12/2/82 12:40 | 1,715.8 | 38 |  |  |  |  |  |
| 12/16/82 6:30 | 1,715.7 | 37 |  |  |  |  |  |
| 12/22/82 12:00 | 1,715.7 | 38 |  |  |  |  |  |
| 1/4/83 13:00 | 1,715.6 | 37 |  |  |  |  |  |
| 1/10/83 12:45 | 1,715.6 | 38 |  |  |  |  |  |
| 1/23/83 6:30 | 1,715.7 |  |  |  |  |  |  |
| 1/30/83 11:45 | 1,715.7 |  |  |  |  |  |  |
| 2/13/83 16:00 | 1,715.6 | 37 |  |  |  |  |  |
| 2/15/83 12:00 | 1,715.7 |  |  |  |  |  |  |
| 2/21/83 13:00 | 1,715.7 | 36 |  |  |  |  |  |
| 2/25/83 11:15 | 1,715.7 | 39 |  |  |  |  |  |
| 3/4/83 20:30 | 1,715.7 | 39 |  |  |  |  |  |
| 3/6/83 20:15 | 1,715.8 |  |  |  |  |  |  |
| 3/7/83 13:30 | 1,715.9 |  |  |  |  |  |  |
| 3/8/83 13:30 | 1,715.9 |  |  |  |  |  |  |
| 3/11/83 6:30 | 1,715.8 | 37 |  |  |  |  |  |
| 3/13/83 9:00 | 1,715.8 |  |  |  |  |  |  |
| 3/18/83 18:30 | 1,715.9 |  |  |  |  |  |  |
| 3/23/83 11:30 | 1,715.8 |  |  |  |  |  |  |
| 3/28/83 18:00 | 1,715.9 | 42 |  |  |  |  |  |
| 3/29/83 11:30 | 1,715.8 |  |  |  |  |  |  |
| 3/30/83 10:15 | 1,715.9 |  |  |  |  |  |  |
| 4/1/83 11:30 | 1,715.9 |  |  |  |  |  |  |
| 4/6/83 12:15 | 1,716.0 |  |  |  |  |  |  |
| 4/7/83 11:00 | 1,716.2 |  |  |  |  |  |  |
| 4/8/83 12:45 | 1,716.2 |  |  |  |  |  |  |
| 4/11/83 6:30 | 1,716.2 |  |  |  |  |  |  |
| 4/15/83 11:00 | 1,716.3 | 39 |  |  |  |  |  |
| 4/19/83 11:00 | 1,716.4 | 41 |  |  |  |  |  |
| 4/20/83 10:30 | 1,716.4 | 42 |  |  |  |  |  |
| 4/24/83 14:00 | 1,716.4 | 48 |  |  |  |  |  |
| 4/28/83 20:00 | 1,716.3 | 54 |  |  |  |  |  |
| 5/3/83 12:45 | 1,716.4 | 50 |  |  |  |  |  |
| 5/6/83 15:00 | 1,716.4 |  |  |  |  |  |  |
| 5/9/83 6:30 | 1,716.5 | 53 |  |  |  |  |  |
| 5/12/83 6:30 | 1,716.4 | 54 |  |  |  |  |  |
| 5/16/83 6:10 | 1,716.4 | 56 |  |  |  |  |  |
| 5/18/83 6:20 | 1,716.3 |  |  |  |  |  |  |
| 5/24/83 12:15 | 1,716.4 | 59 |  |  |  |  |  |
| 5/25/83 16:30 |  | 58 |  |  |  |  |  |
| 5/27/83 7:45 | 1,716.3 | 64 |  |  |  |  |  |
| 6/2/83 7:30 | 1,716.2 | 59 |  |  |  |  |  |
| 6/8/83 7:00 |  | 66 |  |  |  |  |  |
| 6/9/83 6:30 | 1,716.1 |  |  |  |  |  |  |
| 6/13/83 12:30 | 1,716.2 | 68 |  |  |  |  |  |
| 6/14/83 12:50 | 1,716.3 | 66 |  |  |  |  |  |
| 6/17/83 15:30 | 1,716.1 | 66 |  |  |  |  |  |
| 6/21/83 9:00 | 1,716.1 | 68 |  |  |  |  |  |
| 6/24/83 20:30 |  | 78 |  |  |  |  |  |
| 6/30/83 13:15 | 1,716.1 | 68 |  |  |  |  |  |
| 7/3/83 6:30 | 1,716.1 | 72 |  |  |  |  |  |
| 7/11/83 13:00 | 1,716.0 | 75 |  |  |  |  |  |
| 7/17/83 6:15 | 1,715.9 | 78 |  |  |  |  |  |
| 7/24/83 7:15 | 1,715.8 | 80 |  |  |  |  |  |
| 7/29/83 7:30 | 1,715.8 | 80 |  |  |  |  |  |
| 8/3/83 6:30 | 1,715.7 | 78 |  |  |  |  |  |
| 8/6/83 17:00 | 1,715.6 | 80 |  |  |  |  |  |
| 8/10/83 18:00 | 1,715.5 |  |  |  |  |  |  |
| 8/13/83 6:00 | 1,715.5 |  |  |  |  |  |  |
| 8/17/83 12:45 | 1,715.4 | 80 |  |  |  |  |  |
| 8/21/83 7:00 | 1,715.6 | 74 |  |  |  |  |  |
| 8/25/83 16:30 | 1,715.3 | 76 |  |  |  |  |  |
| 8/31/83 19:00 | 1,715.5 |  |  |  |  |  |  |
| 9/2/83 12:00 | 1,715.3 | 64 |  |  |  |  |  |
| 9/14/83 12:00 | 1,715.2 | 62 |  |  |  |  |  |
| 9/23/83 7:00 | 1,715.1 |  |  |  |  |  |  |
| 10/2/83 9:00 | 1,715.1 | 58 |  |  |  |  |  |
| 10/9/83 11:30 | 1,715.1 | 55 |  |  |  |  |  |
| 10/22/83 20:00 | 1,715.1 | 48 |  |  |  |  |  |
| 10/29/83 21:00 | 1,715.1 | 46 |  |  |  |  |  |
| 11/10/83 13:00 | 1,715.1 | 40 |  |  |  |  |  |
| 11/13/83 6:00 | 1,715.1 | 36 |  |  |  |  |  |
| 11/20/83 13:00 | 1,715.2 |  |  |  |  |  |  |
| 12/2/83 20:00 | 1,715.2 |  |  |  |  |  |  |
| 12/9/83 12:30 | 1,715.3 | 34 |  |  |  |  |  |
| 12/13/83 12:00 | 1,715.3 |  |  |  |  |  |  |
| 12/19/83 12:00 | 1,715.2 |  |  |  |  |  |  |
| 1/1/84 11:15 | 1,715.2 | 36 |  |  |  |  |  |
| 1/7/84 11:30 | 1,715.2 | 36 |  |  |  |  |  |
| 1/22/84 12:15 | 1,715.2 | 38 |  |  |  |  |  |
| 1/29/84 7:00 | 1,715.2 |  |  |  |  |  |  |
| 2/12/84 17:00 | 1,715.2 | 38 |  |  |  |  |  |
| 2/15/84 6:15 | 1,715.2 |  |  |  |  |  |  |
| 2/21/84 16:00 | 1,715.3 | 36 |  |  |  |  |  |
| 2/22/84 19:00 | 1,715.3 | 36 |  |  |  |  |  |
| 2/24/84 16:00 | 1,715.3 | 38 |  |  |  |  |  |
| 2/26/84 16:00 | 1,715.4 | 38 |  |  |  |  |  |
| 2/29/84 17:45 | 1,715.6 |  |  |  |  |  |  |
| 3/1/84 15:35 | 1,715.6 | 40 |  |  |  |  |  |
| 3/2/84 15:45 | 1,715.7 | 40 |  |  |  |  |  |
| 3/4/84 16:30 | 1,715.8 | 40 |  |  |  |  |  |
| 3/6/84 18:15 | 1,715.8 | 40 |  |  |  |  |  |
| 3/8/84 10:30 | 1,715.8 |  |  |  |  |  |  |
| 3/14/84 16:00 | 1,715.8 |  |  |  |  |  |  |
| 3/15/84 17:00 | 1,715.8 | 40 |  |  |  |  |  |
| 3/18/84 16:45 | 1,715.8 | 39 |  |  |  |  |  |
| 3/22/84 11:00 | 1,715.9 |  |  |  |  |  |  |
| 3/23/84 21:30 | 1,715.9 |  |  |  |  |  |  |
| 3/25/84 6:15 | 1,716.0 | 41 |  |  |  |  |  |
| 3/26/84 6:30 | 1,716.4 | 41 |  |  |  |  |  |
| 3/26/84 12:30 | 1,716.5 |  |  |  |  |  |  |
| 3/26/84 17:00 | 1,716.6 |  |  |  |  |  |  |
| 3/27/84 6:30 | 1,717.0 |  |  |  |  |  |  |
| 3/27/84 11:45 | 1,717.1 | 41 |  |  |  |  |  |
| 3/28/84 6:15 | 1,717.3 | 41 |  |  |  |  |  |
| 3/28/84 12:45 | 1,717.4 |  |  |  |  |  |  |
| 3/29/84 6:30 | 1,717.4 | 42 |  |  |  |  |  |
| 3/29/84 17:30 | 1,717.5 |  |  |  |  |  |  |
| 3/30/84 6:30 | 1,717.5 | 42 |  |  |  |  |  |
| 3/31/84 6:30 | 1,717.6 | 41 |  |  |  |  |  |
| 3/31/84 12:30 | 1,717.7 |  |  |  |  |  |  |
| 4/1/84 7:00 | 1,717.8 |  |  |  |  |  |  |
| 4/2/84 6:30 | 1,717.9 | 44 |  |  |  |  |  |
| 4/3/84 6:30 | 1,718.0 |  |  |  |  |  |  |
| 4/4/84 6:45 | 1,718.1 |  |  |  |  |  |  |
| 4/5/84 6:40 | 1,718.2 |  |  |  |  |  |  |
| 4/6/84 6:30 | 1,718.2 |  |  |  |  |  |  |
| 4/7/84 6:45 | 1,718.3 |  |  |  |  |  |  |
| 4/8/84 6:15 | 1,718.3 | 41 |  |  |  |  |  |
| 4/9/84 6:30 | 1,718.4 |  |  |  |  |  |  |
| 4/10/84 6:15 | 1,718.4 | 41 |  |  |  |  |  |
| 4/11/84 17:00 | 1,718.5 | 40 |  |  |  |  |  |
| 4/12/84 6:30 | 1,718.6 | 40 |  |  |  |  |  |
| 4/13/84 6:15 | 1,718.7 | 41 |  |  |  |  |  |
| 4/15/84 7:30 | 1,718.8 |  |  |  |  |  |  |
| 4/16/84 11:45 | 1,718.8 | 42 |  |  |  |  |  |
| 4/18/84 13:30 | 1,718.9 |  |  |  |  |  |  |
| 4/20/84 6:30 | 1,718.9 | 44 |  |  |  |  |  |
| 4/22/84 20:00 | 1,718.8 | 47 |  |  |  |  |  |
| 4/25/84 7:00 | 1,718.8 | 48 |  |  |  |  |  |
| 4/27/84 16:45 | 1,718.9 | 48 |  |  |  |  |  |
| 4/30/84 0:00 | 1,718.8 | 47 |  |  |  |  |  |
| 5/8/84 6:40 | 1,718.8 | 47 |  |  |  |  |  |
| 5/11/84 12:45 | 1,718.8 | 50 |  |  |  |  |  |
| 5/12/84 11:15 | 1,718.7 | 52 |  |  |  |  |  |
| 5/17/84 7:00 | 1,718.7 | 53 |  |  |  |  |  |
| 5/19/84 0:00 |  | 60 |  |  |  |  |  |
| 5/20/84 9:00 | 1,718.7 | 62 |  |  |  |  |  |
| 5/23/84 7:00 | 1,718.7 | 61 |  |  |  |  |  |
| 5/30/84 10:15 | 1,718.5 | 59 |  |  |  |  |  |
| 6/1/84 12:45 | 1,718.5 | 63 |  |  |  |  |  |
| 6/6/84 8:00 | 1,718.5 | 63 |  |  |  |  |  |
| 6/8/84 8:00 | 1,718.6 | 63 |  |  |  |  |  |
| 6/10/84 12:00 | 1,718.7 | 62 |  |  |  |  |  |
| 6/12/84 6:30 | 1,718.8 |  |  |  |  |  |  |
| 6/15/84 6:15 | 1,718.9 | 63 |  |  |  |  |  |
| 6/16/84 8:00 | 1,719.0 | 65 |  |  |  |  |  |
| 6/17/84 11:00 | 1,719.4 | 66 |  |  |  |  |  |
| 6/18/84 10:00 | 1,719.6 | 66 |  |  |  |  |  |
| 6/18/84 15:30 | 1,719.7 |  |  |  |  |  |  |
| 6/19/84 12:30 | 1,719.7 | 67 |  |  |  |  |  |
| 6/20/84 10:30 | 1,719.7 | 67 |  |  |  |  |  |
| 6/21/84 12:30 | 1,719.8 | 68 |  |  |  |  |  |
| 6/23/84 12:00 | 1,719.7 | 68 |  |  |  |  |  |
| 6/24/84 7:15 | 1,719.7 | 68 |  |  |  |  |  |
| 6/25/84 7:00 | 1,719.7 | 68 |  |  |  |  |  |
| 6/27/84 8:00 | 1,719.6 | 71 |  |  |  |  |  |
| 6/28/84 11:30 | 1,719.6 |  |  |  |  |  |  |
| 7/2/84 7:45 | 1,719.5 | 71 |  |  |  |  |  |
| 7/4/84 12:45 | 1,719.4 | 76 |  |  |  |  |  |
| 7/7/84 7:00 | 1,719.3 |  |  |  |  |  |  |
| 7/11/84 12:00 | 1,719.1 | 74 |  |  |  |  |  |
| 7/14/84 7:00 | 1,719.1 | 76 |  |  |  |  |  |
| 7/17/84 8:00 | 1,719.1 | 76 |  |  |  |  |  |
| 7/23/84 9:00 | 1,718.8 | 77 |  |  |  |  |  |
| 7/25/84 11:30 | 1,718.8 |  |  |  |  |  |  |
| 7/27/84 16:45 | 1,718.7 |  |  |  |  |  |  |
| 8/2/84 6:00 | 1,718.5 | 75 |  |  |  |  |  |
| 8/3/84 12:30 | 1,718.7 | 76 |  |  |  |  |  |
| 8/4/84 6:15 | 1,718.7 |  |  |  |  |  |  |
| 8/9/84 7:30 | 1,718.7 |  |  |  |  |  |  |
| 8/11/84 8:15 | 1,718.6 | 76 |  |  |  |  |  |
| 8/16/84 6:15 | 1,718.6 | 76 |  |  |  |  |  |
| 8/20/84 10:00 | 1,718.4 | 74 |  |  |  |  |  |
| 8/22/84 6:15 | 1,718.4 |  |  |  |  |  |  |
| 8/23/84 10:50 | 1,718.3 |  |  |  |  |  |  |
| 8/24/84 6:15 | 1,718.3 | 74 |  |  |  |  |  |
| 8/27/84 11:30 | 1,718.3 |  |  |  |  |  |  |
| 8/29/84 7:00 | 1,718.3 | 72 |  |  |  |  |  |
| 8/31/84 20:30 | 1,718.2 | 67 |  |  |  |  |  |
| 9/6/84 17:00 | 1,718.0 | 65 |  |  |  |  |  |
| 9/7/84 15:15 | 1,718.0 | 64 |  |  |  |  |  |
| 9/11/84 17:00 | 1,718.0 | 62 |  |  |  |  |  |
| 9/14/84 10:00 | 1,718.0 | 63 |  |  |  |  |  |
| 9/20/84 20:00 | 1,717.9 | 63 |  |  |  |  |  |
| 9/26/84 7:20 | 1,717.8 | 52 |  |  |  |  |  |
| 10/1/84 12:30 | 1,717.8 | 50 |  |  |  |  |  |
| 10/5/84 12:45 | 1,717.8 | 51 |  |  |  |  |  |
| 10/8/84 7:10 | 1,717.8 |  |  |  |  |  |  |
| 10/10/84 0:00 | 1,717.8 | 55 |  |  |  |  |  |
| 10/14/84 17:00 | 1,717.9 | 58 |  |  |  |  |  |
| 10/16/84 12:00 | 1,718.1 | 54 |  |  |  |  |  |
| 10/19/84 10:30 | 1,718.2 | 48 |  |  |  |  |  |
| 10/22/84 17:00 | 1,718.2 | 44 |  |  |  |  |  |
| 10/30/84 12:00 | 1,718.3 | 40 |  |  |  |  |  |
| 10/31/84 12:45 | 1,718.3 | 40 |  |  |  |  |  |
| 11/3/84 0:00 | 1,718.2 | 40 |  |  |  |  |  |
| 11/11/84 12:00 | 1,718.2 | 40 |  |  |  |  |  |
| 11/18/84 8:00 | 1,718.2 | 36 |  |  |  |  |  |
| 12/11/84 11:00 | 1,718.0 | 34 |  |  |  |  |  |
| 12/19/84 10:00 | 1,718.0 | 34 |  |  |  |  |  |
| 12/31/84 12:00 | 1,718.0 | 34 |  |  |  |  |  |
| 1/1/85 7:00 | 1,718.0 | 34 |  |  |  |  |  |
| 1/11/85 11:15 | 1,717.9 |  |  |  |  |  |  |
| 1/30/85 18:00 | 1,717.9 |  |  |  |  |  |  |
| 2/4/85 10:00 | 1,717.9 | 36 |  |  |  |  |  |
| 2/20/85 11:00 | 1,717.9 | 35 |  |  |  |  |  |
| 2/25/85 0:45 | 1,717.9 | 36 |  |  |  |  |  |
| 3/8/85 10:00 | 1,718.1 | 36 |  |  |  |  |  |
| 3/10/85 13:00 | 1,718.2 | 38 |  |  |  |  |  |
| 3/18/85 12:00 | 1,719.0 | 39 |  |  |  |  |  |
| 3/19/85 7:00 | 1,719.5 | 40 |  |  |  |  |  |
| 3/20/85 17:00 | 1,719.9 | 40 |  |  |  |  |  |
| 3/21/85 19:30 | 1,720.0 | 40 |  |  |  |  |  |
| 3/22/85 7:15 | 1,720.0 | 39 |  |  |  |  |  |
| 3/23/85 6:45 | 1,720.1 | 40 |  |  |  |  |  |
| 3/24/85 7:30 | 1,720.1 |  |  |  |  |  |  |
| 3/25/85 7:00 | 1,720.0 | 40 |  |  |  |  |  |
| 3/26/85 7:00 | 1,719.9 | 40 |  |  |  |  |  |
| 3/27/85 6:30 | 1,719.8 | 40 |  |  |  |  |  |
| 3/28/85 7:30 | 1,719.8 | 40 |  |  |  |  |  |
| 3/29/85 7:00 | 1,719.7 | 40 |  |  |  |  |  |
| 3/30/85 18:45 | 1,719.6 | 40 |  |  |  |  |  |
| 3/31/85 7:30 | 1,719.6 |  |  |  |  |  |  |
| 4/1/85 7:00 | 1,719.5 | 40 |  |  |  |  |  |
| 4/3/85 7:00 | 1,719.4 | 41 |  |  |  |  |  |
| 4/4/85 6:30 | 1,719.3 | 41 |  |  |  |  |  |
| 4/5/85 7:00 | 1,719.2 | 40 |  |  |  |  |  |
| 4/6/85 6:30 | 1,719.2 |  |  |  |  |  |  |
| 4/7/85 6:30 | 1,719.1 | 41 |  |  |  |  |  |
| 4/8/85 7:00 | 1,719.1 | 40 |  |  |  |  |  |
| 4/9/85 7:00 | 1,719.0 | 40 |  |  |  |  |  |
| 4/10/85 7:00 | 1,719.0 | 40 |  |  |  |  |  |
| 4/12/85 10:00 | 1,718.9 | 42 |  |  |  |  |  |
| 4/14/85 7:00 | 1,718.8 | 43 |  |  |  |  |  |
| 4/17/85 7:15 | 1,718.7 | 45 |  |  |  |  |  |
| 4/18/85 6:30 | 1,718.7 | 48 |  |  |  |  |  |
| 4/19/85 6:30 | 1,718.7 | 50 |  |  |  |  |  |
| 4/21/85 12:00 | 1,718.7 | 54 |  |  |  |  |  |
| 4/23/85 7:00 | 1,718.7 | 52 |  |  |  |  |  |
| 4/24/85 8:00 | 1,718.7 |  |  |  |  |  |  |
| 4/26/85 6:30 | 1,718.7 |  |  |  |  |  |  |
| 4/27/85 12:30 | 1,718.6 | 51 |  |  |  |  |  |
| 4/28/85 11:00 | 1,718.6 | 53 |  |  |  |  |  |
| 4/29/85 6:30 | 1,718.6 | 55 |  |  |  |  |  |
| 4/30/85 7:00 | 1,718.6 | 55 |  |  |  |  |  |
| 5/2/85 10:00 | 1,718.5 | 56 |  |  |  |  |  |
| 5/6/85 11:00 | 1,718.4 | 60 |  |  |  |  |  |
| 5/12/85 18:00 | 1,718.3 |  |  |  |  |  |  |
| 5/14/85 11:45 | 1,718.3 | 62 |  |  |  |  |  |
| 5/19/85 7:45 | 1,718.3 | 63 |  |  |  |  |  |
| 5/20/85 6:15 | 1,718.2 |  |  |  |  |  |  |
| 5/23/85 0:00 | 1,718.2 | 63 |  |  |  |  |  |
| 5/27/85 8:00 | 1,718.1 |  |  |  |  |  |  |
| 5/29/85 8:00 | 1,718.1 | 65 |  |  |  |  |  |
| 6/5/85 11:30 | 1,718.0 | 63 |  |  |  |  |  |
| 6/10/85 6:30 | 1,717.9 | 65 |  |  |  |  |  |
| 6/16/85 8:00 | 1,717.9 | 66 |  |  |  |  |  |
| 6/20/85 19:00 | 1,717.8 |  |  |  |  |  |  |
| 6/27/85 12:00 | 1,717.7 | 67 |  |  |  |  |  |
| 6/29/85 20:00 | 1,717.7 |  |  |  |  |  |  |
| 7/3/85 12:30 | 1,717.6 | 71 |  |  |  |  |  |
| 7/8/85 7:00 | 1,717.5 | 76 |  |  |  |  |  |
| 7/15/85 11:00 | 1,717.4 | 74 |  |  |  |  |  |
| 7/17/85 21:00 | 1,717.5 |  |  |  |  |  |  |
| 7/18/85 12:00 | 1,717.6 | 73 |  |  |  |  |  |
| 7/19/85 7:20 | 1,717.8 |  |  |  |  |  |  |
| 7/20/85 7:45 | 1,717.8 | 73 |  |  |  |  |  |
| 7/23/85 6:45 | 1,717.8 |  |  |  |  |  |  |
| 7/27/85 9:00 | 1,717.8 | 73 |  |  |  |  |  |
| 8/1/85 7:00 | 1,717.7 |  |  |  |  |  |  |
| 8/4/85 6:15 | 1,717.6 | 72 |  |  |  |  |  |
| 8/6/85 6:15 | 1,717.6 | 73 |  |  |  |  |  |
| 8/15/85 0:00 | 1,717.5 |  |  |  |  |  |  |
| 8/17/85 23:00 | 1,717.5 | 70 |  |  |  |  |  |
| 8/30/85 13:00 | 1,717.5 | 68 |  |  |  |  |  |
| 8/31/85 18:00 | 1,717.5 | 68 |  |  |  |  |  |
| 9/2/85 6:30 | 1,717.6 |  |  |  |  |  |  |
| 9/7/85 7:30 | 1,717.7 | 68 |  |  |  |  |  |
| 9/9/85 7:00 | 1,717.8 | 66 |  |  |  |  |  |
| 9/13/85 8:00 | 1,718.0 |  |  |  |  |  |  |
| 9/18/85 10:30 | 1,718.0 | 64 |  |  |  |  |  |
| 9/22/85 9:00 | 1,718.0 | 58 |  |  |  |  |  |
| 9/26/85 10:00 | 1,717.9 | 56 |  |  |  |  |  |
| 9/28/85 8:00 | 1,717.9 | 54 |  |  |  |  |  |
| 10/1/85 7:00 | 1,717.9 | 49 |  |  |  |  |  |
| 10/4/85 0:00 | 1,717.9 | 45 |  |  |  |  |  |
| 10/7/85 12:00 | 1,717.9 |  |  |  |  |  |  |
| 10/10/85 13:30 | 1,717.9 | 46 |  |  |  |  |  |
| 10/13/85 6:15 | 1,718.0 |  |  |  |  |  |  |
| 10/18/85 12:30 | 1,718.0 | 44 |  |  |  |  |  |
| 10/21/85 11:00 | 1,718.0 | 44 |  |  |  |  |  |
| 10/23/85 6:30 | 1,718.0 | 44 |  |  |  |  |  |
| 10/24/85 6:30 | 1,718.0 | 44 |  |  |  |  |  |
| 10/25/85 7:00 | 1,718.0 | 44 |  |  |  |  |  |
| 10/27/85 7:00 | 1,718.0 | 48 |  |  |  |  |  |
| 10/30/85 10:30 | 1,718.0 | 46 |  |  |  |  |  |
| 11/4/85 21:15 | 1,717.9 |  |  |  |  |  |  |
| 11/6/85 12:00 | 1,717.9 | 42 |  |  |  |  |  |
| 11/10/85 12:00 | 1,717.8 | 36 |  |  |  |  |  |
| 11/12/85 8:00 | 1,717.8 | 36 |  |  |  |  |  |
| 11/15/85 7:00 | 1,717.8 | 36 |  |  |  |  |  |
| 11/22/85 12:00 | 1,717.8 | 34 |  |  |  |  |  |
| 11/26/85 11:00 | 1,717.8 | 36 |  |  |  |  |  |
| 11/30/85 10:00 | 1,717.8 | 36 |  |  |  |  |  |
| 12/16/85 11:00 | 1,717.7 |  |  |  |  |  |  |
| 12/26/85 0:00 | 1,717.8 |  |  |  |  |  |  |
| 1/5/86 6:15 | 1,717.7 | 37 |  |  |  |  |  |
| 1/11/86 10:00 | 1,717.6 | 38 |  |  |  |  |  |
| 1/19/86 9:00 | 1,717.6 | 37 |  |  |  |  |  |
| 1/24/86 12:30 | 1,717.6 |  |  |  |  |  |  |
| 2/1/86 8:00 | 1,717.5 | 37 |  |  |  |  |  |
| 2/16/86 9:00 | 1,717.5 |  |  |  |  |  |  |
| 3/1/86 10:00 | 1,717.5 | 38 |  |  |  |  |  |
| 3/3/86 8:30 | 1,717.5 | 38 |  |  |  |  |  |
| 3/4/86 13:00 | 1,717.5 | 38 |  |  |  |  |  |
| 3/10/86 6:00 | 1,717.5 | 38 |  |  |  |  |  |
| 3/15/86 6:30 | 1,717.5 | 38 |  |  |  |  |  |
| 3/17/86 6:30 | 1,717.5 |  |  |  |  |  |  |
| 3/19/86 7:00 | 1,717.5 | 39 |  |  |  |  |  |
| 3/20/86 6:45 | 1,717.5 |  |  |  |  |  |  |
| 3/22/86 8:30 | 1,717.5 | 39 |  |  |  |  |  |
| 3/23/86 9:00 | 1,717.5 | 39 |  |  |  |  |  |
| 3/25/86 13:15 | 1,717.5 |  |  |  |  |  |  |
| 3/27/86 13:10 | 1,717.5 |  |  |  |  |  |  |
| 3/28/86 13:30 | 1,717.6 |  |  |  |  |  |  |
| 3/29/86 9:00 | 1,717.9 | 40 |  |  |  |  |  |
| 3/29/86 20:00 | 1,718.3 | 40 |  |  |  |  |  |
| 3/30/86 8:30 | 1,719.0 | 39 |  |  |  |  |  |
| 3/30/86 20:30 | 1,720.7 |  |  |  |  |  |  |
| 3/31/86 6:30 | 1,720.2 | 40 |  |  |  |  |  |
| 3/31/86 18:30 | 1,720.5 |  |  |  |  |  |  |
| 4/1/86 6:15 | 1,720.7 | 40 |  |  |  |  |  |
| 4/1/86 12:55 | 1,720.7 |  |  |  |  |  |  |
| 4/1/86 17:30 | 1,720.7 |  |  |  |  |  |  |
| 4/2/86 7:00 | 1,720.7 | 40 |  |  |  |  |  |
| 4/2/86 11:00 | 1,720.8 |  |  |  |  |  |  |
| 4/3/86 6:30 | 1,720.8 | 40 |  |  |  |  |  |
| 4/3/86 20:30 | 1,720.7 |  |  |  |  |  |  |
| 4/4/86 6:30 | 1,720.7 | 40 |  |  |  |  |  |
| 4/4/86 12:30 | 1,720.8 |  |  |  |  |  |  |
| 4/4/86 21:15 | 1,720.9 |  |  |  |  |  |  |
| 4/5/86 6:30 | 1,720.8 | 39 |  |  |  |  |  |
| 4/5/86 9:30 | 1,720.8 |  |  |  |  |  |  |
| 4/5/86 21:30 | 1,720.8 |  |  |  |  |  |  |
| 4/6/86 7:00 | 1,720.8 | 40 |  |  |  |  |  |
| 4/6/86 21:00 | 1,720.6 |  |  |  |  |  |  |
| 4/7/86 6:15 | 1,720.6 | 40 |  |  |  |  |  |
| 4/7/86 19:30 | 1,720.4 |  |  |  |  |  |  |
| 4/8/86 7:00 | 1,720.3 | 40 |  |  |  |  |  |
| 4/8/86 12:00 | 1,720.2 |  |  |  |  |  |  |
| 4/8/86 21:00 | 1,720.3 |  |  |  |  |  |  |
| 4/9/86 7:00 | 1,720.2 |  |  |  |  |  |  |
| 4/9/86 16:00 | 1,720.2 |  |  |  |  |  |  |
| 4/10/86 6:30 | 1,720.0 | 40 |  |  |  |  |  |
| 4/11/86 7:00 | 1,719.9 |  |  |  |  |  |  |
| 4/14/86 8:00 | 1,720.0 |  |  |  |  |  |  |
| 4/15/86 7:00 | 1,720.6 |  |  |  |  |  |  |
| 4/15/86 12:00 | 1,720.4 |  |  |  |  |  |  |
| 4/16/86 7:00 | 1,720.3 |  |  |  |  |  |  |
| 4/17/86 7:00 | 1,720.2 | 42 |  |  |  |  |  |
| 4/18/86 6:45 | 1,720.2 | 38 |  |  |  |  |  |
| 4/18/86 18:00 | 1,720.3 | 44 |  |  |  |  |  |
| 4/19/86 20:30 | 1,720.1 | 43 |  |  |  |  |  |
| 4/20/86 6:30 | 1,720.1 | 43 |  |  |  |  |  |
| 4/20/86 21:30 | 1,720.1 |  |  |  |  |  |  |
| 4/21/86 6:30 | 1,720.1 | 44 |  |  |  |  |  |
| 4/21/86 18:00 | 1,720.1 |  |  |  |  |  |  |
| 4/22/86 6:30 | 1,720.0 | 44 |  |  |  |  |  |
| 4/23/86 6:30 | 1,719.9 | 45 |  |  |  |  |  |
| 4/24/86 6:30 | 1,719.9 | 48 |  |  |  |  |  |
| 4/24/86 12:45 | 1,719.8 |  |  |  |  |  |  |
| 4/25/86 6:00 | 1,719.7 | 47 |  |  |  |  |  |
| 4/26/86 6:30 | 1,719.7 | 49 |  |  |  |  |  |
| 4/27/86 7:00 | 1,719.7 |  |  |  |  |  |  |
| 4/28/86 21:00 | 1,719.7 |  |  |  |  |  |  |
| 4/29/86 7:00 | 1,719.6 |  |  |  |  |  |  |
| 4/30/86 7:00 | 1,719.6 |  |  |  |  |  |  |
| 5/2/86 7:00 | 1,719.5 | 49 |  |  |  |  |  |
| 5/5/86 11:30 | 1,719.5 | 53 |  |  |  |  |  |
| 5/7/86 7:00 | 1,719.3 |  |  |  |  |  |  |
| 5/8/86 7:00 | 1,719.3 | 52 |  |  |  |  |  |
| 5/8/86 16:00 | 1,719.3 |  |  |  |  |  |  |
| 5/9/86 7:00 | 1,719.5 | 52 |  |  |  |  |  |
| 5/9/86 18:00 | 1,719.6 |  |  |  |  |  |  |
| 5/10/86 7:30 | 1,719.8 | 52 |  |  |  |  |  |
| 5/10/86 20:00 | 1,719.8 |  |  |  |  |  |  |
| 5/11/86 6:30 | 1,719.9 | 54 |  |  |  |  |  |
| 5/12/86 6:30 | 1,720.0 | 56 |  |  |  |  |  |
| 5/13/86 6:30 | 1,720.0 | 58 |  |  |  |  |  |
| 5/13/86 16:00 | 1,720.0 | 58 |  |  |  |  |  |
| 5/14/86 6:30 | 1,719.9 | 59 |  |  |  |  |  |
| 5/15/86 6:15 | 1,719.8 | 59 |  |  |  |  |  |
| 5/16/86 6:30 | 1,719.7 | 60 |  |  |  |  |  |
| 5/17/86 6:45 | 1,719.7 | 59 |  |  |  |  |  |
| 5/18/86 8:00 | 1,719.6 |  |  |  |  |  |  |
| 5/19/86 6:45 | 1,719.5 |  |  |  |  |  |  |
| 5/20/86 6:30 | 1,719.4 | 59 |  |  |  |  |  |
| 5/21/86 7:00 | 1,719.4 |  |  |  |  |  |  |
| 5/22/86 7:00 | 1,719.3 | 59 |  |  |  |  |  |
| 5/23/86 6:45 | 1,719.2 | 59 |  |  |  |  |  |
| 5/24/86 7:30 | 1,719.3 |  |  |  |  |  |  |
| 5/24/86 19:30 | 1,719.3 |  |  |  |  |  |  |
| 5/25/86 21:30 | 1,719.3 |  |  |  |  |  |  |
| 5/26/86 18:30 | 1,719.2 | 60 |  |  |  |  |  |
| 5/28/86 18:00 | 1,719.2 | 61 |  |  |  |  |  |
| 5/29/86 7:30 | 1,719.2 |  |  |  |  |  |  |
| 5/30/86 10:00 | 1,719.2 | 62 |  |  |  |  |  |
| 5/31/86 6:30 | 1,719.2 | 64 |  |  |  |  |  |
| 6/1/86 6:15 | 1,719.1 | 64 |  |  |  |  |  |
| 6/3/86 13:30 | 1,719.0 | 67 |  |  |  |  |  |
| 6/5/86 6:30 | 1,719.1 | 68 |  |  |  |  |  |
| 6/6/86 6:30 | 1,719.0 | 67 |  |  |  |  |  |
| 6/7/86 11:00 | 1,719.0 | 68 |  |  |  |  |  |
| 6/9/86 8:00 | 1,719.0 | 68 |  |  |  |  |  |
| 6/10/86 7:30 | 1,719.0 | 68 |  |  |  |  |  |
| 6/11/86 7:00 | 1,719.1 | 68 |  |  |  |  |  |
| 6/12/86 7:30 | 1,719.2 | 68 |  |  |  |  |  |
| 6/13/86 7:40 | 1,719.4 | 68 |  |  |  |  |  |
| 6/14/86 6:30 | 1,719.5 |  |  |  |  |  |  |
| 6/15/86 6:30 | 1,719.5 | 66 |  |  |  |  |  |
| 6/17/86 7:00 | 1,719.5 |  |  |  |  |  |  |
| 6/20/86 13:00 | 1,719.4 | 69 |  |  |  |  |  |
| 6/21/86 12:00 | 1,719.3 |  |  |  |  |  |  |
| 6/22/86 6:30 | 1,719.3 | 70 |  |  |  |  |  |
| 6/23/86 6:15 | 1,719.3 | 70 |  |  |  |  |  |
| 6/25/86 18:00 | 1,719.2 | 70 |  |  |  |  |  |
| 6/27/86 6:15 | 1,719.2 | 72 |  |  |  |  |  |
| 6/29/86 6:30 | 1,719.2 | 72 |  |  |  |  |  |
| 6/30/86 21:00 | 1,719.2 | 71 |  |  |  |  |  |
| 7/1/86 6:45 | 1,719.2 |  |  |  |  |  |  |
| 7/5/86 8:30 | 1,719.0 | 72 |  |  |  |  |  |
| 7/7/86 6:30 | 1,718.9 | 74 |  |  |  |  |  |
| 7/9/86 5:00 | 1,718.9 |  |  |  |  |  |  |
| 7/11/86 17:00 | 1,718.9 |  |  |  |  |  |  |
| 7/13/86 8:00 | 1,718.9 | 76 |  |  |  |  |  |
| 7/14/86 7:00 | 1,718.9 |  |  |  |  |  |  |
| 7/15/86 10:00 | 1,718.9 |  |  |  |  |  |  |
| 7/17/86 16:00 | 1,718.8 | 79 |  |  |  |  |  |
| 7/18/86 12:00 | 1,718.8 | 78 |  |  |  |  |  |
| 7/21/86 7:00 | 1,718.8 | 79 |  |  |  |  |  |
| 7/23/86 8:00 | 1,718.7 |  |  |  |  |  |  |
| 7/24/86 7:45 | 1,718.7 |  |  |  |  |  |  |
| 7/25/86 7:00 | 1,718.7 |  |  |  |  |  |  |
| 7/27/86 11:15 | 1,718.7 | 78 |  |  |  |  |  |
| 7/28/86 7:00 | 1,718.7 |  |  |  |  |  |  |
| 7/31/86 6:30 | 1,718.7 |  |  |  |  |  |  |
| 8/4/86 17:30 | 1,718.6 | 74 |  |  |  |  |  |
| 8/6/86 12:00 | 1,718.6 | 76 |  |  |  |  |  |
| 8/8/86 12:15 | 1,718.6 | 75 |  |  |  |  |  |
| 8/11/86 8:00 | 1,718.6 |  |  |  |  |  |  |
| 8/13/86 7:45 | 1,718.5 | 72 |  |  |  |  |  |
| 8/15/86 8:00 | 1,718.5 | 75 |  |  |  |  |  |
| 8/17/86 8:15 | 1,718.5 | 74 |  |  |  |  |  |
| 8/20/86 10:30 | 1,718.5 | 74 |  |  |  |  |  |
| 8/23/86 13:30 | 1,718.4 |  |  |  |  |  |  |
| 8/26/86 20:00 | 1,718.4 | 72 |  |  |  |  |  |
| 8/28/86 8:30 | 1,718.4 | 69 |  |  |  |  |  |
| 8/30/86 12:30 | 1,718.4 | 68 |  |  |  |  |  |
| 8/31/86 9:30 | 1,718.3 | 68 |  |  |  |  |  |
| 9/2/86 15:45 | 1,718.4 | 68 |  |  |  |  |  |
| 9/5/86 8:30 | 1,718.3 | 66 |  |  |  |  |  |
| 9/8/86 20:00 | 1,718.3 | 63 |  |  |  |  |  |
| 9/9/86 19:00 | 1,718.3 | 62 |  |  |  |  |  |
| 9/11/86 12:15 | 1,718.3 |  |  |  |  |  |  |
| 9/14/86 10:00 | 1,718.3 | 59 |  |  |  |  |  |
| 9/17/86 6:45 | 1,718.4 | 59 |  |  |  |  |  |
| 9/18/86 6:15 | 1,718.4 | 59 |  |  |  |  |  |
| 9/19/86 6:30 | 1,718.4 |  |  |  |  |  |  |
| 9/20/86 6:30 | 1,718.4 | 59 |  |  |  |  |  |
| 9/22/86 0:00 | 1,718.4 | 59 |  |  |  |  |  |
| 9/24/86 0:00 | 1,718.5 |  |  |  |  |  |  |
| 9/29/86 18:30 | 1,718.5 | 58 |  |  |  |  |  |
| 10/2/86 17:30 | 1,718.5 | 58 |  |  |  |  |  |
| 10/7/86 16:00 | 1,718.4 | 58 |  |  |  |  |  |
| 10/12/86 8:30 | 1,718.3 | 49 |  |  |  |  |  |
| 10/13/86 7:00 | 1,718.3 |  |  |  |  |  |  |
| 10/13/86 19:00 | 1,718.3 |  |  |  |  |  |  |
| 10/19/86 17:30 | 1,718.1 | 50 |  |  |  |  |  |
| 10/24/86 6:30 | 1,718.1 | 50 |  |  |  |  |  |
| 10/26/86 6:15 | 1,718.1 |  |  |  |  |  |  |
| 10/29/86 19:00 | 1,718.0 | 49 |  |  |  |  |  |
| 11/2/86 9:30 | 1,718.0 | 44 |  |  |  |  |  |
| 11/6/86 10:00 | 1,717.9 | 44 |  |  |  |  |  |
| 11/11/86 20:00 | 1,717.8 | 36 |  |  |  |  |  |
| 11/14/86 12:00 | 1,717.8 | 37 |  |  |  |  |  |
| 11/19/86 6:15 | 1,717.8 | 36 |  |  |  |  |  |
| 11/24/86 8:00 | 1,717.8 | 36 |  |  |  |  |  |
| 12/1/86 6:30 | 1,717.8 | 38 |  |  |  |  |  |
| 12/6/86 9:00 | 1,717.8 |  |  |  |  |  |  |
| 12/10/86 6:30 | 1,717.8 | 37 |  |  |  |  |  |
| 12/13/86 9:30 | 1,717.7 |  |  |  |  |  |  |
| 12/18/86 0:00 | 1,717.6 | 37 |  |  |  |  |  |
| 12/30/86 0:00 | 1,717.6 |  |  |  |  |  |  |
| 1/6/87 7:00 | 1,717.6 |  |  |  |  |  |  |
| 1/16/87 13:00 | 1,717.6 | 38 |  |  |  |  |  |
| 1/25/87 10:00 | 1,717.5 | 36 |  |  |  |  |  |
| 2/1/87 7:00 | 1,717.4 | 42 |  |  |  |  |  |
| 2/8/87 9:00 | 1,717.4 | 42 |  |  |  |  |  |
| 2/17/87 12:30 | 1,717.4 | 42 |  |  |  |  |  |
| 2/21/87 18:30 | 1,717.4 | 44 |  |  |  |  |  |
| 3/1/87 10:00 | 1,717.4 | 44 |  |  |  |  |  |
| 3/5/87 6:30 | 1,717.4 | 44 |  |  |  |  |  |
| 3/7/87 9:30 | 1,717.4 | 44 |  |  |  |  |  |
| 3/15/87 13:30 | 1,717.5 | 40 |  |  |  |  |  |
| 3/20/87 13:00 | 1,717.6 | 40 |  |  |  |  |  |
| 3/22/87 6:30 | 1,717.7 | 40 |  |  |  |  |  |
| 3/22/87 18:00 | 1,717.9 | 40 |  |  |  |  |  |
| 3/23/87 10:30 | 1,717.9 | 40 |  |  |  |  |  |
| 3/24/87 10:00 | 1,718.0 | 40 |  |  |  |  |  |
| 3/25/87 12:00 | 1,718.1 | 40 |  |  |  |  |  |
| 3/26/87 6:30 | 1,718.2 | 40 |  |  |  |  |  |
| 3/29/87 7:00 | 1,718.5 |  |  |  |  |  |  |
| 3/30/87 6:30 | 1,718.5 | 37 |  |  |  |  |  |
| 4/2/87 6:30 | 1,718.6 | 37 |  |  |  |  |  |
| 4/3/87 13:15 | 1,718.5 | 38 |  |  |  |  |  |
| 4/6/87 10:15 | 1,718.5 | 42 |  |  |  |  |  |
| 4/8/87 13:10 | 1,718.5 | 43 |  |  |  |  |  |
| 4/10/87 6:15 | 1,718.4 | 46 |  |  |  |  |  |
| 4/12/87 9:00 | 1,718.4 | 47 |  |  |  |  |  |
| 4/15/87 12:00 | 1,718.4 | 50 |  |  |  |  |  |
| 4/17/87 7:00 | 1,718.4 | 53 |  |  |  |  |  |
| 4/19/87 0:00 | 1,718.4 | 58 |  |  |  |  |  |
| 4/24/87 19:00 | 1,718.5 | 57 |  |  |  |  |  |
| 4/28/87 17:30 | 1,718.2 | 60 |  |  |  |  |  |
| 5/1/87 11:30 | 1,718.1 | 60 |  |  |  |  |  |
| 5/4/87 21:00 | 1,718.0 | 60 |  |  |  |  |  |
| 5/6/87 13:00 | 1,718.0 | 62 |  |  |  |  |  |
| 5/12/87 0:00 | 1,717.9 | 66 |  |  |  |  |  |
| 5/18/87 16:00 | 1,717.7 | 65 |  |  |  |  |  |
| 5/19/87 6:10 | 1,717.6 |  |  |  |  |  |  |
| 5/24/87 6:15 | 1,717.6 | 62 |  |  |  |  |  |
| 5/27/87 16:00 | 1,717.6 |  |  |  |  |  |  |
| 5/31/87 8:00 | 1,717.6 | 70 |  |  |  |  |  |
| 6/5/87 6:30 | 1,717.4 | 72 |  |  |  |  |  |
| 6/11/87 10:00 | 1,717.4 | 73 |  |  |  |  |  |
| 6/15/87 17:00 | 1,717.3 | 78 |  |  |  |  |  |
| 6/18/87 13:00 | 1,717.2 | 78 |  |  |  |  |  |
| 6/24/87 7:00 | 1,717.0 |  |  |  |  |  |  |
| 6/29/87 10:30 | 1,716.9 | 74 |  |  |  |  |  |
| 7/4/87 7:00 | 1,717.0 | 74 |  |  |  |  |  |
| 7/5/87 6:15 | 1,717.0 | 73 |  |  |  |  |  |
| 7/8/87 10:00 | 1,717.2 | 74 |  |  |  |  |  |
| 7/11/87 8:30 | 1,717.0 | 76 |  |  |  |  |  |
| 7/13/87 12:00 | 1,717.0 | 72 |  |  |  |  |  |
| 7/21/87 13:00 | 1,717.0 | 76 |  |  |  |  |  |
| 7/27/87 17:00 | 1,716.8 | 80 |  |  |  |  |  |
| 7/31/87 11:00 | 1,716.8 | 80 |  |  |  |  |  |
| 8/6/87 14:00 | 1,716.7 | 80 |  |  |  |  |  |
| 8/9/87 11:00 | 1,716.9 | 80 |  |  |  |  |  |
| 8/11/87 10:30 | 1,717.0 | 75 |  |  |  |  |  |
| 8/14/87 12:00 | 1,716.9 | 76 |  |  |  |  |  |
| 8/26/87 12:00 | 1,716.8 | 70 |  |  |  |  |  |
| 8/28/87 7:00 | 1,716.8 | 68 |  |  |  |  |  |
| 9/5/87 6:15 | 1,716.7 | 70 |  |  |  |  |  |
| 9/12/87 11:00 | 1,716.7 | 66 |  |  |  |  |  |
| 9/18/87 7:00 | 1,716.6 | 65 |  |  |  |  |  |
| 9/21/87 15:30 | 1,716.6 | 62 |  |  |  |  |  |
| 9/29/87 12:00 | 1,716.6 |  |  |  |  |  |  |
| 10/7/87 16:00 | 1,716.5 | 59 |  |  |  |  |  |
| 10/16/87 22:30 | 1,716.4 | 58 |  |  |  |  |  |
| 11/1/87 11:30 | 1,716.5 | 50 |  |  |  |  |  |
| 11/10/87 16:00 | 1,716.3 | 41 |  |  |  |  |  |
| 11/24/87 16:00 | 1,716.3 | 46 |  |  |  |  |  |
| 11/29/87 19:00 | 1,716.3 | 42 |  |  |  |  |  |
| 12/1/87 15:00 | 1,716.4 | 38 |  |  |  |  |  |
| 12/11/87 18:10 | 1,716.4 | 35 |  |  |  |  |  |
| 12/21/87 11:00 | 1,716.4 | 36 |  |  |  |  |  |
| 12/26/87 8:30 | 1,716.4 | 36 |  |  |  |  |  |
| 12/31/87 0:00 | 1,716.4 | 36 |  |  |  |  |  |
| 1/1/88 6:00 | 1,716.4 | 36 |  |  |  |  |  |
| 1/10/88 11:00 | 1,716.4 | 36 |  |  |  |  |  |
| 1/15/88 15:00 | 1,716.4 |  |  |  |  |  |  |
| 1/28/88 11:30 | 1,716.4 | 36 |  |  |  |  |  |
| 2/10/88 7:00 | 1,716.5 | 36 |  |  |  |  |  |
| 2/17/88 7:00 | 1,716.5 | 36 |  |  |  |  |  |
| 2/24/88 10:30 | 1,716.6 | 37 |  |  |  |  |  |
| 2/29/88 16:00 | 1,716.3 | 38 |  |  |  |  |  |
| 3/9/88 13:30 | 1,716.5 |  |  |  |  |  |  |
| 3/13/88 11:30 |  | 40 |  |  |  |  |  |
| 3/17/88 11:15 | 1,716.5 | 42 |  |  |  |  |  |
| 3/21/88 8:00 | 1,716.5 | 43 |  |  |  |  |  |
| 3/25/88 7:10 | 1,716.7 | 43 |  |  |  |  |  |
| 3/28/88 13:00 | 1,716.7 | 36 |  |  |  |  |  |
| 4/1/88 11:00 | 1,716.7 | 36 |  |  |  |  |  |
| 4/3/88 10:30 | 1,716.7 | 40 |  |  |  |  |  |
| 4/5/88 16:45 | 1,716.7 | 41 |  |  |  |  |  |
| 4/10/88 10:00 | 1,716.8 | 47 |  |  |  |  |  |
| 4/15/88 17:30 | 1,716.6 | 47 |  |  |  |  |  |
| 4/20/88 12:30 | 1,716.6 | 47 |  |  |  |  |  |
| 4/24/88 11:30 | 1,716.6 | 48 |  |  |  |  |  |
| 4/27/88 12:15 | 1,716.6 | 45 |  |  |  |  |  |
| 5/6/88 13:15 | 1,716.5 | 56 |  |  |  |  |  |
| 5/13/88 13:00 | 1,716.5 | 59 |  |  |  |  |  |
| 5/20/88 17:00 | 1,716.5 |  |  |  |  |  |  |
| 5/29/88 7:00 | 1,716.5 | 67 |  |  |  |  |  |
| 6/3/88 16:00 | 1,716.6 | 74 |  |  |  |  |  |
| 6/6/88 13:30 | 1,716.5 | 74 |  |  |  |  |  |
| 6/19/88 19:30 | 1,716.3 | 80 |  |  |  |  |  |
| 6/21/88 0:45 | 1,716.1 | 80 |  |  |  |  |  |
| 6/27/88 12:45 | 1,716.0 | 81 |  |  |  |  |  |
| 6/29/88 10:45 | 1,715.9 | 79 |  |  |  |  |  |
| 7/5/88 15:30 | 1,715.8 | 74 |  |  |  |  |  |
| 7/11/88 19:00 | 1,715.7 | 74 |  |  |  |  |  |
| 7/14/88 16:00 | 1,715.6 | 76 |  |  |  |  |  |
| 7/17/88 12:30 | 1,715.5 | 75 |  |  |  |  |  |
| 7/20/88 13:00 | 1,715.5 | 74 |  |  |  |  |  |
| 7/24/88 19:45 | 1,715.4 | 78 |  |  |  |  |  |
| 7/31/88 12:00 | 1,715.3 | 78 |  |  |  |  |  |
| 8/2/88 12:45 | 1,715.4 | 77 |  |  |  |  |  |
| 8/10/88 19:45 | 1,715.2 | 74 |  |  |  |  |  |
| 8/22/88 12:00 | 1,715.2 | 74 |  |  |  |  |  |
| 8/23/88 13:30 | 1,715.2 | 73 |  |  |  |  |  |
| 9/4/88 16:00 | 1,715.0 | 67 |  |  |  |  |  |
| 9/11/88 6:30 | 1,714.9 | 66 |  |  |  |  |  |
| 9/19/88 11:30 | 1,714.8 | 64 |  |  |  |  |  |
| 9/28/88 10:30 | 1,714.8 | 58 |  |  |  |  |  |
| 10/1/88 12:40 | 1,714.8 | 58 |  |  |  |  |  |
| 10/6/88 13:30 | 1,714.6 | 53 |  |  |  |  |  |
| 10/11/88 20:45 | 1,714.6 | 54 |  |  |  |  |  |
| 10/20/88 12:30 | 1,714.5 | 47 |  |  |  |  |  |
| 10/30/88 11:30 | 1,714.4 | 35 |  |  |  |  |  |
| 11/2/88 19:00 | 1,714.5 | 42 |  |  |  |  |  |
| 11/7/88 20:15 | 1,714.5 | 42 |  |  |  |  |  |
| 11/10/88 13:00 | 1,714.5 | 36 |  |  |  |  |  |
| 11/12/88 13:00 | 1,714.5 | 38 |  |  |  |  |  |
| 11/18/88 13:30 | 1,714.5 | 34 |  |  |  |  |  |
| 11/22/88 20:15 | 1,714.4 | 38 |  |  |  |  |  |
| 12/5/88 13:00 | 1,714.4 | 42 |  |  |  |  |  |
| 12/18/88 19:15 | 1,714.4 | 42 |  |  |  |  |  |
| 12/27/88 12:00 | 1,714.5 | 40 |  |  |  |  |  |
| 1/6/89 22:15 | 1,714.4 | 40 |  |  |  |  |  |
| 2/28/89 13:00 | 1,714.6 | 40 |  |  |  |  |  |
| 3/13/89 8:30 | 1,715.1 | 38 |  |  |  |  |  |
| 3/14/89 8:30 | 1,715.2 | 38 |  |  |  |  |  |
| 3/15/89 18:00 | 1,715.4 | 38 |  |  |  |  |  |
| 3/16/89 7:45 | 1,715.5 | 38 |  |  |  |  |  |
| 3/16/89 17:15 | 1,715.5 | 38 |  |  |  |  |  |
| 3/17/89 8:30 | 1,715.3 | 42 |  |  |  |  |  |
| 3/17/89 17:45 | 1,715.4 | 38 |  |  |  |  |  |
| 3/18/89 17:15 | 1,715.4 | 38 |  |  |  |  |  |
| 3/19/89 17:00 | 1,715.4 | 39 |  |  |  |  |  |
| 3/22/89 8:00 | 1,715.5 | 40 |  |  |  |  |  |
| 3/23/89 13:10 | 1,715.5 | 40 |  |  |  |  |  |
| 3/24/89 8:00 | 1,715.5 | 40 |  |  |  |  |  |
| 3/25/89 8:00 | 1,715.7 | 39 |  |  |  |  |  |
| 3/25/89 19:00 | 1,715.9 | 39 |  |  |  |  |  |
| 3/26/89 7:15 | 1,716.1 | 39 |  |  |  |  |  |
| 3/26/89 20:00 | 1,716.3 | 39 |  |  |  |  |  |
| 3/27/89 8:00 | 1,716.5 | 39 |  |  |  |  |  |
| 3/27/89 18:00 | 1,716.7 | 39 |  |  |  |  |  |
| 3/28/89 7:30 | 1,716.9 | 39 |  |  |  |  |  |
| 3/28/89 20:00 | 1,717.1 | 39 |  |  |  |  |  |
| 3/29/89 7:00 | 1,717.1 | 39 |  |  |  |  |  |
| 3/30/89 7:15 | 1,717.3 | 39 |  |  |  |  |  |
| 3/31/89 8:15 | 1,717.4 | 39 |  |  |  |  |  |
| 3/31/89 18:00 | 1,717.4 | 39 |  |  |  |  |  |
| 4/1/89 8:15 | 1,717.4 | 39 |  |  |  |  |  |
| 4/2/89 20:00 | 1,717.4 | 39 |  |  |  |  |  |
| 4/3/89 9:30 | 1,717.4 | 39 |  |  |  |  |  |
| 4/5/89 8:15 | 1,717.4 | 39 |  |  |  |  |  |
| 4/6/89 9:30 | 1,717.5 | 39 |  |  |  |  |  |
| 4/8/89 13:00 | 1,717.6 | 40 |  |  |  |  |  |
| 4/9/89 11:24 | 1,717.6 | 40 |  |  |  |  |  |
| 4/10/89 13:15 | 1,717.6 | 40 |  |  |  |  |  |
| 4/11/89 10:00 | 1,717.6 | 40 |  |  |  |  |  |
| 4/13/89 12:00 | 1,717.6 | 40 |  |  |  |  |  |
| 4/14/89 7:00 | 1,717.5 | 40 |  |  |  |  |  |
| 4/16/89 7:00 | 1,717.5 | 40 |  |  |  |  |  |
| 4/18/89 11:30 | 1,717.5 | 38 |  |  |  |  |  |
| 4/20/89 17:07 | 1,717.6 | 52 |  |  |  |  |  |
| 4/26/89 9:00 | 1,717.6 | 53 |  |  |  |  |  |
| 4/28/89 7:30 | 1,717.6 |  |  |  |  |  |  |
| 4/29/89 6:30 | 1,717.7 | 54 |  |  |  |  |  |
| 5/5/89 6:45 | 1,717.7 | 52 |  |  |  |  |  |
| 5/7/89 15:15 | 1,717.7 | 50 |  |  |  |  |  |
| 5/9/89 7:35 | 1,717.7 | 53 |  |  |  |  |  |
| 5/14/89 11:30 | 1,717.7 | 56 |  |  |  |  |  |
| 5/19/89 13:00 | 1,717.7 | 64 |  |  |  |  |  |
| 5/25/89 6:30 | 1,717.7 | 64 |  |  |  |  |  |
| 6/4/89 8:00 | 1,717.6 | 66 |  |  |  |  |  |
| 6/13/89 18:00 | 1,717.5 | 62 |  |  |  |  |  |
| 6/18/89 21:00 | 1,717.4 | 66 |  |  |  |  |  |
| 6/29/89 13:00 | 1,717.4 | 73 |  |  |  |  |  |
| 7/11/89 10:30 | 1,717.1 | 76 |  |  |  |  |  |
| 7/12/89 12:15 | 1,717.1 | 78 |  |  |  |  |  |
| 7/26/89 9:40 | 1,716.9 | 78 |  |  |  |  |  |
| 7/30/89 8:00 | 1,716.9 | 78 |  |  |  |  |  |
| 8/1/89 12:15 | 1,716.8 | 78 |  |  |  |  |  |
| 8/14/89 20:00 | 1,716.7 | 75 |  |  |  |  |  |
| 8/28/89 20:45 | 1,716.6 | 75 |  |  |  |  |  |
| 9/6/89 11:15 | 1,716.5 | 72 |  |  |  |  |  |
| 9/10/89 15:45 | 1,716.5 | 64 |  |  |  |  |  |
| 9/29/89 8:30 | 1,716.3 | 58 |  |  |  |  |  |
| 10/7/89 10:30 | 1,716.2 | 52 |  |  |  |  |  |
| 10/11/89 12:30 | 1,716.2 | 49 |  |  |  |  |  |
| 10/23/89 10:30 | 1,716.1 | 45 |  |  |  |  |  |
| 10/27/89 11:30 | 1,716.0 | 52 |  |  |  |  |  |
| 11/1/89 10:45 | 1,716.0 | 50 |  |  |  |  |  |
| 11/13/89 10:15 | 1,715.9 | 41 |  |  |  |  |  |
| 11/24/89 11:30 | 1,715.9 | 39 |  |  |  |  |  |
| 12/9/89 21:00 | 1,715.9 |  |  |  |  |  |  |
| 1/9/90 16:00 | 1,715.8 | 39 |  |  |  |  |  |
| 1/22/90 19:30 | 1,715.8 | 39 |  |  |  |  |  |
| 2/2/90 12:00 | 1,715.8 | 40 |  |  |  |  |  |
| 2/10/90 10:00 | 1,715.9 | 40 |  |  |  |  |  |
| 2/18/90 16:00 | 1,715.8 | 40 |  |  |  |  |  |
| 2/27/90 12:30 | 1,715.8 | 40 |  |  |  |  |  |
| 3/6/90 0:00 | 1,715.7 | 45 |  |  |  |  |  |
| 3/13/90 7:30 | 1,715.9 | 40 |  |  |  |  |  |
| 3/16/90 10:45 | 1,715.8 | 40 |  |  |  |  |  |
| 3/18/90 11:00 | 1,715.8 | 40 |  |  |  |  |  |
| 3/20/90 14:00 | 1,715.8 | 38 |  |  |  |  |  |
| 3/25/90 9:30 | 1,715.9 | 37 |  |  |  |  |  |
| 3/28/90 17:00 | 1,715.9 | 36 |  |  |  |  |  |
| 3/29/90 16:30 | 1,715.9 | 38 |  |  |  |  |  |
| 4/4/90 11:00 | 1,715.9 | 41 |  |  |  |  |  |
| 4/7/90 12:30 | 1,715.8 | 40 |  |  |  |  |  |
| 4/9/90 10:30 | 1,715.8 | 42 |  |  |  |  |  |
| 4/10/90 16:20 | 1,715.8 | 42 |  |  |  |  |  |
| 4/17/90 20:30 | 1,715.7 | 43 |  |  |  |  |  |
| 4/20/90 11:00 | 1,715.7 | 46 |  |  |  |  |  |
| 4/21/90 11:30 | 1,715.6 | 48 |  |  |  |  |  |
| 4/26/90 13:00 | 1,715.6 | 58 |  |  |  |  |  |
| 4/27/90 16:30 | 1,715.7 | 56 |  |  |  |  |  |
| 4/30/90 13:00 | 1,715.7 | 48 |  |  |  |  |  |
| 5/2/90 12:00 | 1,715.7 | 47 |  |  |  |  |  |
| 5/6/90 10:30 | 1,715.6 | 52 |  |  |  |  |  |
| 5/18/90 12:00 | 1,715.6 | 53 |  |  |  |  |  |
| 5/23/90 10:00 | 1,715.7 | 55 |  |  |  |  |  |
| 5/25/90 10:30 | 1,715.6 | 58 |  |  |  |  |  |
| 5/30/90 12:15 | 1,715.7 | 61 |  |  |  |  |  |
| 6/2/90 12:15 | 1,715.7 | 64 |  |  |  |  |  |
| 6/12/90 11:30 | 1,715.6 | 68 |  |  |  |  |  |
| 6/17/90 19:00 | 1,715.9 | 67 |  |  |  |  |  |
| 6/19/90 12:30 | 1,716.0 | 67 |  |  |  |  |  |
| 6/20/90 12:30 | 1,716.0 | 68 |  |  |  |  |  |
| 6/23/90 11:05 | 1,715.9 | 68 |  |  |  |  |  |
| 6/26/90 12:15 | 1,715.8 | 72 |  |  |  |  |  |
| 6/28/90 11:30 | 1,715.9 | 72 |  |  |  |  |  |
| 7/1/90 10:15 | 1,715.9 | 77 |  |  |  |  |  |
| 7/12/90 12:00 | 1,715.7 | 72 |  |  |  |  |  |
| 7/17/90 9:30 | 1,715.6 | 72 |  |  |  |  |  |
| 7/21/90 12:30 | 1,715.6 | 72 |  |  |  |  |  |
| 7/23/90 16:15 | 1,715.7 | 70 |  |  |  |  |  |
| 8/2/90 10:45 | 1,715.6 | 72 |  |  |  |  |  |
| 8/7/90 10:45 | 1,715.4 | 71 |  |  |  |  |  |
| 8/12/90 21:30 | 1,715.4 | 70 |  |  |  |  |  |
| 8/17/90 11:00 | 1,715.3 | 72 |  |  |  |  |  |
| 8/23/90 10:45 | 1,715.3 | 68 |  |  |  |  |  |
| 8/25/90 12:00 | 1,715.4 | 69 |  |  |  |  |  |
| 8/25/90 11:30 | 1,715.3 | 69 |  |  |  |  |  |
| 9/4/90 0:30 | 1,715.3 | 69 |  |  |  |  |  |
| 9/8/90 11:30 | 1,715.3 | 72 |  |  |  |  |  |
| 9/15/90 12:30 | 1,715.0 | 67 |  |  |  |  |  |
| 9/16/90 12:00 | 1,715.0 | 65 |  |  |  |  |  |
| 9/17/90 16:00 | 1,715.0 | 64 |  |  |  |  |  |
| 9/23/90 12:00 | 1,715.0 | 54 |  |  |  |  |  |
| 9/29/90 21:00 | 1,714.8 | 57 |  |  |  |  |  |
| 10/5/90 10:30 | 1,715.0 | 56 |  |  |  |  |  |
| 10/7/90 13:00 | 1,715.1 | 51 |  |  |  |  |  |
| 10/8/90 13:00 | 1,715.1 | 52 |  |  |  |  |  |
| 10/9/90 12:45 | 1,715.0 | 52 |  |  |  |  |  |
| 10/12/90 15:00 | 1,714.9 | 48 |  |  |  |  |  |
| 10/19/90 11:45 | 1,714.9 | 42 |  |  |  |  |  |
| 10/22/90 13:00 | 1,714.9 | 40 |  |  |  |  |  |
| 10/28/90 12:00 | 1,714.9 | 42 |  |  |  |  |  |
| 10/31/90 13:15 | 1,714.8 | 43 |  |  |  |  |  |
| 11/4/90 17:45 | 1,714.7 | 42 |  |  |  |  |  |
| 11/7/90 12:30 | 1,714.7 | 36 |  |  |  |  |  |
| 11/10/90 10:30 | 1,714.8 | 34 |  |  |  |  |  |
| 11/18/90 10:45 | 1,714.8 | 36 |  |  |  |  |  |
| 11/22/90 12:00 | 1,714.8 | 37 |  |  |  |  |  |
| 12/2/90 11:30 | 1,714.7 | 34 |  |  |  |  |  |
| 12/7/90 12:30 | 1,714.7 | 34 |  |  |  |  |  |
| 12/16/90 8:00 | 1,714.7 | 36 |  |  |  |  |  |
| 12/18/90 11:00 | 1,714.7 | 36 |  |  |  |  |  |
| 12/31/90 12:30 | 1,714.7 | 35 |  |  |  |  |  |
| 1/2/91 18:00 | 1,714.7 | 36 |  |  |  |  |  |
| 1/8/91 12:00 | 1,714.7 | 37 |  |  |  |  |  |
| 1/15/91 12:00 | 1,714.6 | 36 |  |  |  |  |  |
| 1/21/91 11:30 | 1,714.7 | 35 |  |  |  |  |  |
| 1/28/91 12:15 | 1,714.7 | 35 |  |  |  |  |  |
| 2/4/91 11:45 | 1,714.7 | 36 |  |  |  |  |  |
| 2/8/91 11:30 | 1,714.6 | 38 |  |  |  |  |  |
| 2/20/91 18:00 | 1,714.7 | 39 |  |  |  |  |  |
| 3/2/91 9:45 | 1,714.7 | 39 |  |  |  |  |  |
| 3/6/91 0:00 | 1,714.7 | 40 |  |  |  |  |  |
| 3/13/91 16:00 | 1,714.7 | 42 |  |  |  |  |  |
| 3/22/91 12:30 | 1,714.8 | 43 |  |  |  |  |  |
| 3/25/91 16:10 | 1,714.8 |  |  |  |  |  |  |
| 3/26/91 15:45 | 1,714.8 |  |  |  |  |  |  |
| 4/1/91 17:00 | 1,714.8 | 38 |  |  |  |  |  |
| 4/9/91 11:45 | 1,714.6 | 44 |  |  |  |  |  |
| 4/13/91 15:30 | 1,714.7 | 40 |  |  |  |  |  |
| 4/14/91 10:45 | 1,714.8 | 38 |  |  |  |  |  |
| 4/15/91 13:45 | 1,714.8 | 40 |  |  |  |  |  |
| 4/18/91 12:30 | 1,714.8 | 43 |  |  |  |  |  |
| 4/22/91 11:00 | 1,714.8 | 44 |  |  |  |  |  |
| 5/1/91 15:30 | 1,715.0 | 46 |  |  |  |  |  |
| 5/2/91 7:00 | 1,715.1 | 46 |  |  |  |  |  |
| 5/3/91 11:00 | 1,715.1 |  |  |  |  |  |  |
| 5/4/91 11:30 | 1,715.1 | 43 |  |  |  |  |  |
| 5/5/91 10:30 | 1,715.1 | 42 |  |  |  |  |  |
| 5/5/91 16:00 | 1,715.1 | 43 |  |  |  |  |  |
| 5/6/91 13:00 | 1,715.1 | 42 |  |  |  |  |  |
| 5/7/91 13:15 | 1,715.2 | 42 |  |  |  |  |  |
| 5/9/91 13:45 | 1,715.2 | 43 |  |  |  |  |  |
| 5/11/91 10:15 | 1,715.3 | 54 |  |  |  |  |  |
| 5/14/91 12:30 | 1,715.4 | 60 |  |  |  |  |  |
| 5/16/91 11:15 | 1,715.4 | 64 |  |  |  |  |  |
| 5/17/91 13:00 | 1,715.3 | 64 |  |  |  |  |  |
| 5/20/91 18:15 | 1,715.3 |  |  |  |  |  |  |
| 5/17/91 0:30 | 1,715.3 | 62 |  |  |  |  |  |
| 5/24/91 12:30 | 1,715.5 | 64 |  |  |  |  |  |
| 5/28/91 11:15 | 1,715.5 | 66 |  |  |  |  |  |
| 5/29/91 11:00 | 1,715.7 | 66 |  |  |  |  |  |
| 5/29/91 16:30 | 1,715.6 | 68 |  |  |  |  |  |
| 5/31/91 16:00 | 1,715.8 | 70 |  |  |  |  |  |
| 6/1/91 6:45 | 1,715.7 | 70 |  |  |  |  |  |
| 6/3/91 11:45 | 1,715.8 | 71 |  |  |  |  |  |
| 6/4/91 13:15 | 1,716.0 | 71 |  |  |  |  |  |
| 6/5/91 11:00 | 1,716.0 | 70 |  |  |  |  |  |
| 6/6/91 16:00 | 1,716.0 |  |  |  |  |  |  |
| 6/9/91 16:00 | 1,716.1 | 70 |  |  |  |  |  |
| 6/10/91 20:40 | 1,716.1 | 72 |  |  |  |  |  |
| 6/12/91 15:45 | 1,716.0 | 72 |  |  |  |  |  |
| 6/15/91 6:50 | 1,716.0 | 72 |  |  |  |  |  |
| 6/17/91 1:46 | 1,716.0 |  |  |  |  |  |  |
| 6/18/91 13:30 | 1,716.0 | 72 |  |  |  |  |  |
| 6/20/91 12:00 | 1,716.3 | 70 |  |  |  |  |  |
| 6/21/91 10:30 | 1,716.5 | 72 |  |  |  |  |  |
| 6/21/91 11:15 | 1,716.9 | 72 |  |  |  |  |  |
| 6/21/91 21:00 | 1,717.5 | 72 |  |  |  |  |  |
| 6/22/91 6:45 | 1,717.8 | 69 |  |  |  |  |  |
| 6/22/91 15:00 | 1,717.9 | 69 |  |  |  |  |  |
| 6/22/91 19:00 | 1,718.0 | 69 |  |  |  |  |  |
| 6/23/91 8:00 | 1,718.0 | 69 |  |  |  |  |  |
| 6/23/91 16:30 | 1,718.1 | 69 |  |  |  |  |  |
| 6/24/91 7:00 | 1,718.2 | 69 |  |  |  |  |  |
| 6/24/91 16:30 | 1,718.2 | 69 |  |  |  |  |  |
| 6/24/91 7:00 | 1,718.2 | 67 |  |  |  |  |  |
| 6/25/91 18:15 | 1,718.1 | 69 |  |  |  |  |  |
| 6/26/91 19:00 | 1,718.2 | 69 |  |  |  |  |  |
| 6/26/91 16:00 | 1,718.2 | 70 |  |  |  |  |  |
| 6/27/91 20:00 | 1,718.4 | 70 |  |  |  |  |  |
| 6/29/91 11:00 | 1,718.4 | 74 |  |  |  |  |  |
| 7/1/91 10:15 | 1,718.8 | 74 |  |  |  |  |  |
| 7/1/91 20:45 | 1,718.9 | 74 |  |  |  |  |  |
| 7/2/91 12:00 | 1,719.1 | 72 |  |  |  |  |  |
| 7/2/91 21:00 | 1,719.1 | 72 |  |  |  |  |  |
| 7/3/91 11:30 | 1,719.2 | 71 |  |  |  |  |  |
| 7/5/91 10:45 | 1,719.3 | 71 |  |  |  |  |  |
| 7/8/91 13:00 | 1,719.3 | 71 |  |  |  |  |  |
| 7/9/91 11:00 | 1,719.2 | 71 |  |  |  |  |  |
| 7/12/91 11:15 | 1,719.2 | 71 |  |  |  |  |  |
| 7/15/91 11:30 | 1,719.1 | 72 |  |  |  |  |  |
| 7/20/91 21:30 | 1,718.9 | 77 |  |  |  |  |  |
| 7/22/91 11:45 | 1,718.9 | 76 |  |  |  |  |  |
| 7/26/91 16:00 | 1,718.7 | 73 |  |  |  |  |  |
| 8/3/91 12:45 | 1,718.9 | 71 |  |  |  |  |  |
| 8/6/91 12:45 | 1,719.3 | 70 |  |  |  |  |  |
| 8/7/91 13:00 | 1,719.4 | 70 |  |  |  |  |  |
| 8/7/91 20:30 | 1,719.4 | 68 |  |  |  |  |  |
| 8/9/91 19:30 | 1,719.4 | 68 |  |  |  |  |  |
| 8/11/91 17:00 | 1,719.5 | 68 |  |  |  |  |  |
| 8/11/91 19:45 | 1,719.5 | 68 |  |  |  |  |  |
| 8/16/91 13:00 | 1,719.3 | 71 |  |  |  |  |  |
| 8/19/91 11:00 | 1,719.2 | 71 |  |  |  |  |  |
| 8/20/91 13:00 | 1,719.3 | 74 |  |  |  |  |  |
| 8/26/91 11:00 | 1,719.1 | 74 |  |  |  |  |  |
| 8/29/91 14:30 | 1,718.9 | 74 |  |  |  |  |  |
| 9/1/91 11:30 | 1,718.9 | 72 |  |  |  |  |  |
| 9/5/91 0:00 | 1,718.6 | 70 |  |  |  |  |  |
| 9/6/91 10:30 | 1,718.6 |  |  |  |  |  |  |
| 9/9/91 12:00 | 1,718.7 | 70 |  |  |  |  |  |
| 9/14/91 11:30 | 1,718.6 | 69 |  |  |  |  |  |
| 9/19/91 7:15 | 1,718.4 | 57 |  |  |  |  |  |
| 9/22/91 11:45 | 1,718.4 | 56 |  |  |  |  |  |
| 9/23/91 11:45 | 1,718.3 | 56 |  |  |  |  |  |
| 9/27/91 7:00 | 1,718.3 |  |  |  |  |  |  |
| 9/28/91 6:45 | 1,718.2 | 53 |  |  |  |  |  |
| 9/30/91 6:40 | 1,718.2 | 53 |  |  |  |  |  |
| 10/2/91 6:50 | 1,718.2 |  |  |  |  |  |  |
| 10/3/91 6:45 | 1,718.2 | 54 |  |  |  |  |  |
| 10/10/91 12:45 | 1,718.0 | 51 |  |  |  |  |  |
| 10/12/91 11:15 | 1,718.1 | 52 |  |  |  |  |  |
| 10/13/91 7:00 | 1,718.0 | 52 |  |  |  |  |  |
| 10/17/91 7:00 | 1,718.0 | 52 |  |  |  |  |  |
| 10/20/91 6:30 | 1,718.0 | 47 |  |  |  |  |  |
| 10/21/91 6:40 | 1,717.9 | 47 |  |  |  |  |  |
| 10/24/91 6:50 | 1,717.9 |  |  |  |  |  |  |
| 10/25/91 7:30 | 1,717.8 | 45 |  |  |  |  |  |
| 10/29/91 7:00 | 1,717.9 | 46 |  |  |  |  |  |
| 10/31/91 7:05 | 1,717.9 | 40 |  |  |  |  |  |
| 11/1/91 7:05 | 1,717.9 | 38 |  |  |  |  |  |
| 11/3/91 9:00 | 1,717.8 | 36 |  |  |  |  |  |
| 11/11/91 7:00 | 1,717.8 | 35 |  |  |  |  |  |
| 11/13/91 6:50 | 1,717.8 | 34 |  |  |  |  |  |
| 11/20/91 7:15 | 1,717.8 | 33 |  |  |  |  |  |
| 11/28/91 9:00 | 1,717.9 | 37 |  |  |  |  |  |
| 12/5/91 6:50 | 1,717.9 | 33 |  |  |  |  |  |
| 12/9/91 7:00 | 1,717.8 | 36 |  |  |  |  |  |
| 12/15/91 7:00 | 1,717.8 | 36 |  |  |  |  |  |
| 12/20/91 7:00 | 1,717.8 | 38 |  |  |  |  |  |
| 12/25/91 6:43 | 1,717.8 | 38 |  |  |  |  |  |
| 12/26/91 6:41 | 1,717.8 | 38 |  |  |  |  |  |
| 1/2/92 7:00 | 1,717.8 | 37 |  |  |  |  |  |
| 1/6/92 7:00 | 1,717.8 | 37 |  |  |  |  |  |
| 1/12/92 9:00 | 1,717.7 | 34 |  |  |  |  |  |
| 1/30/92 8:30 | 1,717.6 | 34 |  |  |  |  |  |
| 2/10/92 7:45 | 1,717.6 | 35 |  |  |  |  |  |
| 2/16/92 9:00 | 1,717.6 | 36 |  |  |  |  |  |
| 2/26/92 8:15 | 1,717.8 | 38 |  |  |  |  |  |
| 2/28/92 7:20 | 1,717.8 |  |  |  |  |  |  |
| 3/2/92 7:25 | 1,717.8 |  |  |  |  |  |  |
| 3/3/92 7:25 | 1,717.8 |  |  |  |  |  |  |
| 3/4/92 7:20 | 1,717.8 |  |  |  |  |  |  |
| 3/5/92 7:35 | 1,717.9 |  |  |  |  |  |  |
| 3/10/92 7:30 | 1,718.0 | 38 |  |  |  |  |  |
| 3/16/92 8:15 | 1,718.0 | 36 |  |  |  |  |  |
| 3/22/92 8:45 | 1,718.1 | 34 |  |  |  |  |  |
| 3/24/92 8:15 | 1,718.1 | 36 |  |  |  |  |  |
| 3/26/92 18:35 | 1,718.0 | 36 |  |  |  |  |  |
| 3/29/92 8:30 | 1,718.0 | 38 |  |  |  |  |  |
| 4/7/92 12:00 | 1,718.0 | 44 |  |  |  |  |  |
| 4/12/92 8:05 | 1,718.0 | 42 |  |  |  |  |  |
| 4/17/92 10:30 | 1,718.0 | 42 |  |  |  |  |  |
| 4/22/92 7:45 | 1,718.1 | 42 |  |  |  |  |  |
| 4/26/92 11:00 | 1,718.1 | 39 |  |  |  |  |  |
| 5/5/92 8:15 | 1,718.0 | 52 |  |  |  |  |  |
| 5/10/92 10:30 | 1,718.0 | 59 |  |  |  |  |  |
| 5/14/92 6:00 | 1,717.9 | 55 |  |  |  |  |  |
| 5/20/92 6:45 | 1,717.8 | 63 |  |  |  |  |  |
| 6/2/92 8:15 | 1,717.8 | 62 |  |  |  |  |  |
| 6/7/92 12:40 | 1,717.8 | 62 |  |  |  |  |  |
| 6/10/92 12:45 | 1,717.8 | 61 |  |  |  |  |  |
| 6/17/92 10:45 | 1,717.8 | 64 |  |  |  |  |  |
| 6/19/92 9:30 | 1,717.8 | 64 |  |  |  |  |  |
| 6/20/92 7:00 | 1,717.8 | 66 |  |  |  |  |  |
| 6/21/92 6:30 | 1,717.9 | 66 |  |  |  |  |  |
| 6/24/92 10:30 | 1,718.0 |  |  |  |  |  |  |
| 6/27/92 8:30 | 1,718.0 | 67 |  |  |  |  |  |
| 6/30/92 11:30 | 1,717.9 | 66 |  |  |  |  |  |
| 7/2/92 8:45 | 1,718.2 | 64 |  |  |  |  |  |
| 7/4/92 15:30 | 1,718.4 | 64 |  |  |  |  |  |
| 7/7/92 9:30 | 1,718.4 | 64 |  |  |  |  |  |
| 7/9/92 21:00 | 1,718.4 | 66 |  |  |  |  |  |
| 7/12/92 12:30 | 1,718.3 | 66 |  |  |  |  |  |
| 7/17/92 11:45 | 1,718.3 | 66 |  |  |  |  |  |
| 7/20/92 11:50 | 1,718.3 | 66 |  |  |  |  |  |
| 7/23/92 8:30 | 1,718.3 |  |  |  |  |  |  |
| 7/29/92 16:40 | 1,718.2 | 66 |  |  |  |  |  |
| 8/9/92 6:35 | 1,718.0 | 70 |  |  |  |  |  |
| 8/13/92 11:40 | 1,718.0 | 69 |  |  |  |  |  |
| 8/19/92 16:15 | 1,717.8 |  |  |  |  |  |  |
| 8/21/92 9:30 | 1,717.8 | 70 |  |  |  |  |  |
| 8/27/92 7:00 | 1,717.9 | 70 |  |  |  |  |  |
| 8/31/92 11:00 | 1,718.0 | 62 |  |  |  |  |  |
| 9/3/92 7:15 | 1,717.7 | 62 |  |  |  |  |  |
| 9/7/92 7:38 | 1,717.8 | 64 |  |  |  |  |  |
| 9/9/92 11:30 | 1,717.9 | 64 |  |  |  |  |  |
| 9/11/92 17:30 | 1,717.8 | 58 |  |  |  |  |  |
| 9/17/92 7:45 | 1,717.7 | 60 |  |  |  |  |  |
| 9/19/92 19:15 | 1,717.7 | 58 |  |  |  |  |  |
| 9/21/92 7:15 | 1,717.7 | 58 |  |  |  |  |  |
| 9/27/92 7:30 | 1,717.6 | 55 |  |  |  |  |  |
| 10/2/92 18:50 | 1,717.5 | 56 |  |  |  |  |  |
| 10/5/92 7:00 | 1,717.5 | 55 |  |  |  |  |  |
| 10/13/92 7:00 | 1,717.3 | 44 |  |  |  |  |  |
| 10/22/92 14:25 | 1,717.3 | 42 |  |  |  |  |  |
| 10/26/92 20:05 | 1,717.3 | 46 |  |  |  |  |  |
| 11/2/92 8:45 | 1,717.4 | 39 |  |  |  |  |  |
| 11/11/92 8:15 | 1,717.3 | 33 |  |  |  |  |  |
| 11/14/92 15:40 | 1,717.3 | 33 |  |  |  |  |  |
| 11/19/92 7:00 | 1,717.3 | 34 |  |  |  |  |  |
| 11/23/92 12:50 | 1,717.4 | 34 |  |  |  |  |  |
| 12/4/92 7:15 | 1,717.4 | 33 |  |  |  |  |  |
| 12/16/92 12:50 | 1,717.4 | 34 |  |  |  |  |  |
| 12/17/92 11:35 | 1,717.5 | 34 |  |  |  |  |  |
| 12/22/92 7:50 | 1,717.5 | 34 |  |  |  |  |  |
| 12/28/92 18:00 | 1,717.4 | 34 |  |  |  |  |  |
| 1/1/93 11:30 | 1,717.4 | 34 |  |  |  |  |  |
| 1/16/93 10:30 | 1,717.4 | 34 |  |  |  |  |  |
| 1/22/93 12:30 | 1,717.4 | 34 |  |  |  |  |  |
| 2/1/93 12:20 | 1,717.5 | 34 |  |  |  |  |  |
| 2/6/93 10:30 | 1,717.4 | 34 |  |  |  |  |  |
| 2/12/93 9:00 | 1,717.5 | 34 |  |  |  |  |  |
| 2/19/93 18:30 | 1,717.4 | 34 |  |  |  |  |  |
| 2/24/93 20:30 | 1,717.5 | 34 |  |  |  |  |  |
| 3/4/93 8:00 | 1,717.5 | 34 |  |  |  |  |  |
| 3/9/93 12:30 | 1,717.5 | 34 |  |  |  |  |  |
| 3/14/93 15:30 | 1,717.4 | 34 |  |  |  |  |  |
| 3/24/93 11:00 | 1,717.4 | 34 |  |  |  |  |  |
| 3/28/93 6:45 | 1,717.5 | 36 |  |  |  |  |  |
| 3/28/93 12:45 | 1,717.7 |  |  |  |  |  |  |
| 3/28/93 15:30 | 1,717.8 |  |  |  |  |  |  |
| 3/28/93 20:40 | 1,718.0 |  |  |  |  |  |  |
| 3/29/93 6:45 | 1,718.4 |  |  |  |  |  |  |
| 3/29/93 15:30 | 1,718.8 |  |  |  |  |  |  |
| 3/29/93 20:00 | 1,718.8 | 31 |  |  |  |  |  |
| 3/30/93 20:00 | 1,719.1 |  |  |  |  |  |  |
| 3/31/93 10:00 | 1,719.3 |  |  |  |  |  |  |
| 4/1/93 8:20 | 1,719.4 | 36 |  |  |  |  |  |
| 4/1/93 20:00 | 1,719.4 | 36 |  |  |  |  |  |
| 4/2/93 7:15 | 1,719.5 | 37 |  |  |  |  |  |
| 4/3/93 8:30 | 1,719.6 | 36 |  |  |  |  |  |
| 4/4/93 7:20 | 1,719.5 | 37 |  |  |  |  |  |
| 4/5/93 6:55 | 1,719.5 | 37 |  |  |  |  |  |
| 4/5/93 8:15 | 1,719.4 | 37 |  |  |  |  |  |
| 4/6/93 8:00 | 1,719.4 | 38 |  |  |  |  |  |
| 4/7/93 7:00 | 1,719.4 | 38 |  |  |  |  |  |
| 4/8/93 7:45 | 1,719.4 | 38 |  |  |  |  |  |
| 4/9/93 8:30 | 1,719.4 | 39 |  |  |  |  |  |
| 4/11/93 7:00 | 1,719.4 | 40 |  |  |  |  |  |
| 4/14/93 13:45 | 1,719.4 | 40 |  |  |  |  |  |
| 4/17/93 11:30 | 1,719.5 | 41 |  |  |  |  |  |
| 4/20/93 7:00 | 1,719.4 | 38 |  |  |  |  |  |
| 4/22/93 8:45 | 1,719.3 | 44 |  |  |  |  |  |
| 4/25/93 16:30 | 1,719.2 | 44 |  |  |  |  |  |
| 4/26/93 17:50 | 1,719.2 | 44 |  |  |  |  |  |
| 4/28/93 19:25 | 1,718.9 | 48 |  |  |  |  |  |
| 4/30/93 12:30 | 1,718.9 | 40 |  |  |  |  |  |
| 5/2/93 11:15 | 1,718.9 | 48 |  |  |  |  |  |
| 5/6/93 9:00 | 1,718.9 | 55 |  |  |  |  |  |
| 5/9/93 12:00 | 1,718.7 | 56 |  |  |  |  |  |
| 5/11/93 19:00 | 1,718.7 | 62 |  |  |  |  |  |
| 5/20/93 7:00 | 1,718.4 | 56 |  |  |  |  |  |
| 5/26/93 11:30 | 1,718.3 | 56 |  |  |  |  |  |
| 5/30/93 9:30 | 1,718.3 | 57 |  |  |  |  |  |
| 6/4/93 0:45 | 1,718.1 | 57 |  |  |  |  |  |
| 6/7/93 5:25 | 1,718.0 | 59 |  |  |  |  |  |
| 6/11/93 5:00 | 1,718.1 |  |  |  |  |  |  |
| 6/15/93 8:45 | 1,718.1 | 62 |  |  |  |  |  |
| 6/18/93 13:15 | 1,718.1 | 62 |  |  |  |  |  |
| 6/21/93 5:00 | 1,718.3 | 61 |  |  |  |  |  |
| 6/22/93 20:30 | 1,718.4 | 64 |  |  |  |  |  |
| 6/24/93 1:04 | 1,718.6 | 66 |  |  |  |  |  |
| 6/25/93 13:20 | 1,718.6 | 65 |  |  |  |  |  |
| 6/27/93 16:20 | 1,718.5 |  |  |  |  |  |  |
| 6/29/93 10:20 | 1,718.5 | 64 |  |  |  |  |  |
| 6/30/93 13:00 | 1,718.7 | 64 |  |  |  |  |  |
| 6/30/93 18:45 | 1,718.6 | 64 |  |  |  |  |  |
| 7/2/93 12:00 | 1,718.9 | 65 |  |  |  |  |  |
| 7/4/93 0:00 | 1,718.7 | 66 |  |  |  |  |  |
| 7/5/93 2:30 | 1,719.1 | 66 |  |  |  |  |  |
| 7/6/93 1:00 | 1,719.2 | 66 |  |  |  |  |  |
| 7/7/93 2:00 | 1,719.3 | 66 |  |  |  |  |  |
| 7/7/93 16:00 | 1,719.3 | 66 |  |  |  |  |  |
| 7/8/93 5:35 | 1,719.3 | 66 |  |  |  |  |  |
| 7/8/93 10:00 | 1,719.3 |  |  |  |  |  |  |
| 7/10/93 1:05 | 1,719.3 | 67 |  |  |  |  |  |
| 7/11/93 4:35 | 1,719.3 | 67 |  |  |  |  |  |
| 7/12/93 4:30 | 1,719.3 | 68 |  |  |  |  |  |
| 7/14/93 17:20 | 1,719.2 | 66 |  |  |  |  |  |
| 7/15/93 17:45 | 1,719.2 |  |  |  |  |  |  |
| 7/17/93 1:00 | 1,718.9 | 68 |  |  |  |  |  |
| 7/18/93 2:00 | 1,719.3 | 70 |  |  |  |  |  |
| 7/18/93 20:50 | 1,719.6 |  |  |  |  |  |  |
| 7/19/93 1:30 | 1,719.6 | 70 |  |  |  |  |  |
| 7/19/93 12:00 | 1,719.8 | 68 |  |  |  |  |  |
| 7/20/93 5:00 | 1,719.9 |  |  |  |  |  |  |
| 7/20/93 19:05 | 1,720.0 | 72 |  |  |  |  |  |
| 7/21/93 1:15 | 1,720.0 |  |  |  |  |  |  |
| 7/22/93 12:44 | 1,720.1 | 70 |  |  |  |  |  |
| 7/23/93 8:15 | 1,720.1 | 70 |  |  |  |  |  |
| 7/24/93 9:10 | 1,720.0 | 70 |  |  |  |  |  |
| 7/25/93 12:30 | 1,720.1 | 69 |  |  |  |  |  |
| 7/26/93 5:30 | 1,720.3 | 69 |  |  |  |  |  |
| 7/26/93 9:15 | 1,720.4 | 68 |  |  |  |  |  |
| 7/26/93 16:40 | 1,720.6 | 69 |  |  |  |  |  |
| 7/27/93 0:30 | 1,720.6 | 68 |  |  |  |  |  |
| 7/27/93 5:00 | 1,720.7 | 68 |  |  |  |  |  |
| 7/27/93 12:30 | 1,720.8 | 68 |  |  |  |  |  |
| 7/27/93 16:20 | 1,720.9 | 68 |  |  |  |  |  |
| 7/28/93 4:37 | 1,720.9 | 68 |  |  |  |  |  |
| 7/29/93 1:00 | 1,720.9 | 68 |  |  |  |  |  |
| 7/29/93 11:20 | 1,720.9 | 68 |  |  |  |  |  |
| 7/30/93 0:35 | 1,720.8 | 68 |  |  |  |  |  |
| 7/31/93 5:10 | 1,720.7 |  |  |  |  |  |  |
| 8/1/93 0:55 | 1,720.6 | 71 |  |  |  |  |  |
| 8/2/93 0:50 | 1,720.5 | 71 |  |  |  |  |  |
| 8/3/93 0:55 | 1,720.5 |  |  |  |  |  |  |
| 8/4/93 1:50 | 1,720.3 |  |  |  |  |  |  |
| 8/5/93 2:55 | 1,720.2 |  |  |  |  |  |  |
| 8/6/93 4:00 | 1,720.1 | 70 |  |  |  |  |  |
| 8/7/93 1:00 | 1,720.0 | 69 |  |  |  |  |  |
| 8/8/93 0:30 | 1,720.0 | 69 |  |  |  |  |  |
| 8/9/93 0:30 | 1,719.9 | 70 |  |  |  |  |  |
| 8/10/93 0:30 | 1,719.8 | 70 |  |  |  |  |  |
| 8/11/93 4:20 | 1,719.7 | 70 |  |  |  |  |  |
| 8/13/93 1:30 | 1,719.6 | 71 |  |  |  |  |  |
| 8/14/93 1:45 | 1,719.5 | 71 |  |  |  |  |  |
| 8/15/93 0:30 | 1,719.7 | 71 |  |  |  |  |  |
| 8/16/93 1:45 | 1,719.6 | 71 |  |  |  |  |  |
| 8/17/93 1:00 | 1,719.6 | 71 |  |  |  |  |  |
| 8/18/93 1:30 | 1,719.6 |  |  |  |  |  |  |
| 8/19/93 1:30 | 1,719.5 |  |  |  |  |  |  |
| 8/22/93 0:30 | 1,719.3 | 72 |  |  |  |  |  |
| 8/24/93 4:30 | 1,719.2 | 72 |  |  |  |  |  |
| 8/27/93 0:45 | 1,719.1 |  |  |  |  |  |  |
| 8/27/93 1:45 | 1,719.1 |  |  |  |  |  |  |
| 8/30/93 1:00 | 1,719.0 | 74 |  |  |  |  |  |
| 8/31/93 0:05 | 1,718.9 |  |  |  |  |  |  |
| 9/2/93 1:10 | 1,718.9 | 73 |  |  |  |  |  |
| 9/3/93 1:30 | 1,718.8 | 72 |  |  |  |  |  |
| 9/4/93 0:25 | 1,718.8 | 73 |  |  |  |  |  |
| 9/5/93 0:15 | 1,718.7 | 69 |  |  |  |  |  |
| 9/7/93 23:55 | 1,718.7 | 69 |  |  |  |  |  |
| 9/8/93 4:10 | 1,718.7 | 68 |  |  |  |  |  |
| 9/10/93 1:15 | 1,718.5 |  |  |  |  |  |  |
| 9/12/93 12:00 | 1,718.4 | 65 |  |  |  |  |  |
| 9/15/93 2:22 | 1,718.4 | 60 |  |  |  |  |  |
| 9/16/93 12:00 | 1,718.4 | 59 |  |  |  |  |  |
| 9/18/93 3:40 | 1,718.4 | 60 |  |  |  |  |  |
| 9/20/93 0:30 | 1,718.4 | 58 |  |  |  |  |  |
| 9/22/93 4:00 | 1,718.5 | 58 |  |  |  |  |  |
| 9/24/93 0:00 | 1,718.5 | 59 |  |  |  |  |  |
| 9/29/93 4:40 | 1,718.4 | 54 |  |  |  |  |  |
| 10/2/93 4:15 | 1,718.3 | 57 |  |  |  |  |  |
| 10/3/93 0:15 | 1,718.3 | 57 |  |  |  |  |  |
| 10/4/93 0:10 | 1,718.4 | 57 |  |  |  |  |  |
| 10/5/93 0:15 | 1,718.4 | 55 |  |  |  |  |  |
| 10/6/93 0:10 | 1,718.4 | 55 |  |  |  |  |  |
| 10/8/93 2:00 | 1,718.2 | 54 |  |  |  |  |  |
| 10/11/93 3:30 | 1,718.1 | 50 |  |  |  |  |  |
| 10/14/93 12:30 | 1,718.1 | 50 |  |  |  |  |  |
| 10/15/93 4:20 | 1,718.2 | 50 |  |  |  |  |  |
| 10/15/93 13:15 | 1,718.1 | 50 |  |  |  |  |  |
| 10/18/93 4:00 | 1,718.1 | 50 |  |  |  |  |  |
| 10/22/93 1:15 | 1,718.0 | 48 |  |  |  |  |  |
| 10/27/93 0:55 | 1,717.9 | 46 |  |  |  |  |  |
| 11/1/93 0:00 | 1,717.8 | 38 |  |  |  |  |  |
| 11/2/93 12:39 | 1,717.8 |  |  |  |  |  |  |
| 11/5/93 1:05 | 1,717.8 | 36 |  |  |  |  |  |
| 11/6/93 16:30 | 1,717.8 | 35 |  |  |  |  |  |
| 11/13/93 1:00 | 1,717.8 | 36 |  |  |  |  |  |
| 11/16/93 8:50 | 1,717.8 | 34 |  |  |  |  |  |
| 11/18/93 0:30 | 1,717.8 | 34 |  |  |  |  |  |
| 11/22/93 0:40 | 1,717.8 |  |  |  |  |  |  |
| 11/24/93 1:00 | 1,717.9 | 34 |  |  |  |  |  |
| 11/25/93 16:30 | 1,717.9 | 34 |  |  |  |  |  |
| 12/3/93 4:00 | 1,717.9 | 34 |  |  |  |  |  |
| 12/7/93 12:00 | 1,717.8 |  |  |  |  |  |  |
| 12/11/93 12:30 | 1,717.8 |  |  |  |  |  |  |
| 12/18/93 0:30 | 1,717.8 |  |  |  |  |  |  |
| 12/23/93 13:10 | 1,717.9 |  |  |  |  |  |  |
| 12/30/93 17:40 | 1,717.8 | 36 |  |  |  |  |  |
| 1/3/94 12:20 | 1,717.8 |  |  |  |  |  |  |
| 1/5/94 1:00 | 1,717.8 |  |  |  |  |  |  |
| 1/16/94 1:10 | 1,717.8 | 36 |  |  |  |  |  |
| 1/29/94 17:15 | 1,717.7 |  |  |  |  |  |  |
| 2/1/94 1:43 | 1,717.8 | 36 |  |  |  |  |  |
| 2/8/94 12:45 | 1,717.7 | 36 |  |  |  |  |  |
| 2/15/94 1:10 | 1,717.7 |  |  |  |  |  |  |
| 2/21/94 1:00 | 1,717.5 | 38 |  |  |  |  |  |
| 2/25/94 14:20 | 1,717.7 | 38 |  |  |  |  |  |
| 2/28/94 1:15 | 1,717.6 | 38 |  |  |  |  |  |
| 2/28/94 12:05 | 1,717.7 | 38 |  |  |  |  |  |
| 3/3/94 9:00 | 1,717.6 | 38 |  |  |  |  |  |
| 3/5/94 9:30 | 1,717.7 | 38 |  |  |  |  |  |
| 3/12/94 8:15 | 1,717.8 | 39 |  |  |  |  |  |
| 3/13/94 8:45 | 1,717.9 | 39 |  |  |  |  |  |
| 3/14/94 1:30 | 1,717.8 | 39 |  |  |  |  |  |
| 3/15/94 8:30 | 1,717.9 | 39 |  |  |  |  |  |
| 3/16/94 0:45 | 1,718.0 | 39 |  |  |  |  |  |
| 3/17/94 1:00 | 1,718.3 | 39 |  |  |  |  |  |
| 3/18/94 1:15 | 1,718.6 | 40 |  |  |  |  |  |
| 3/19/94 5:00 | 1,718.9 | 40 |  |  |  |  |  |
| 3/19/94 19:00 | 1,719.0 | 40 |  |  |  |  |  |
| 3/20/94 0:50 | 1,719.1 | 41 |  |  |  |  |  |
| 3/20/94 12:00 | 1,719.3 | 40 |  |  |  |  |  |
| 3/21/94 7:20 | 1,719.8 | 40 |  |  |  |  |  |
| 3/21/94 12:00 | 1,719.9 | 40 |  |  |  |  |  |
| 3/21/94 21:00 | 1,720.0 | 40 |  |  |  |  |  |
| 3/22/94 1:15 | 1,720.0 | 40 |  |  |  |  |  |
| 3/22/94 4:10 | 1,720.1 | 40 |  |  |  |  |  |
| 3/22/94 7:00 | 1,720.1 | 40 |  |  |  |  |  |
| 3/22/94 17:45 | 1,720.1 | 40 |  |  |  |  |  |
| 3/23/94 0:20 | 1,720.1 | 40 |  |  |  |  |  |
| 3/23/94 5:45 | 1,720.1 |  |  |  |  |  |  |
| 3/24/94 17:25 | 1,720.1 | 39 |  |  |  |  |  |
| 3/25/94 5:15 | 1,720.0 | 40 |  |  |  |  |  |
| 3/25/94 16:55 | 1,719.9 |  |  |  |  |  |  |
| 3/26/94 0:32 | 1,719.9 | 40 |  |  |  |  |  |
| 3/26/94 16:50 | 1,719.9 | 40 |  |  |  |  |  |
| 3/27/94 0:10 | 1,719.9 | 40 |  |  |  |  |  |
| 3/27/94 16:50 | 1,719.8 | 40 |  |  |  |  |  |
| 3/28/94 1:23 | 1,719.8 | 40 |  |  |  |  |  |
| 3/29/94 1:00 | 1,719.7 | 40 |  |  |  |  |  |
| 3/30/94 0:05 | 1,719.6 | 40 |  |  |  |  |  |
| 3/31/94 2:00 | 1,719.5 | 40 |  |  |  |  |  |
| 3/31/94 13:45 | 1,719.4 | 40 |  |  |  |  |  |
| 4/1/94 5:00 | 1,719.4 | 40 |  |  |  |  |  |
| 4/1/94 13:30 | 1,719.4 | 40 |  |  |  |  |  |
| 4/2/94 4:20 | 1,719.3 | 40 |  |  |  |  |  |
| 4/2/94 17:10 | 1,719.3 | 40 |  |  |  |  |  |
| 4/4/94 12:00 | 1,719.2 | 40 |  |  |  |  |  |
| 4/5/94 12:30 | 1,719.1 | 40 |  |  |  |  |  |
| 4/6/94 0:30 | 1,719.1 | 42 |  |  |  |  |  |
| 4/8/94 1:30 | 1,719.0 | 40 |  |  |  |  |  |
| 4/9/94 1:10 | 1,718.9 | 42 |  |  |  |  |  |
| 4/11/94 0:00 | 1,718.9 | 42 |  |  |  |  |  |
| 4/13/94 5:00 | 1,718.8 | 41 |  |  |  |  |  |
| 4/15/94 1:00 | 1,718.8 | 42 |  |  |  |  |  |
| 4/16/94 10:16 | 1,718.7 |  |  |  |  |  |  |
| 4/17/94 1:00 | 1,718.7 | 43 |  |  |  |  |  |
| 4/18/94 0:50 | 1,718.7 |  |  |  |  |  |  |
| 4/19/94 0:35 | 1,718.7 | 48 |  |  |  |  |  |
| 4/20/94 4:45 | 1,718.7 | 50 |  |  |  |  |  |
| 4/21/94 0:40 | 1,718.7 | 50 |  |  |  |  |  |
| 4/22/94 1:10 | 1,718.6 | 50 |  |  |  |  |  |
| 4/23/94 0:30 | 1,718.6 | 51 |  |  |  |  |  |
| 4/24/94 2:20 | 1,718.6 | 53 |  |  |  |  |  |
| 4/25/94 5:15 | 1,718.5 | 54 |  |  |  |  |  |
| 4/26/94 1:00 | 1,718.5 | 54 |  |  |  |  |  |
| 4/27/94 0:30 | 1,718.6 |  |  |  |  |  |  |
| 4/28/94 7:28 | 1,718.6 |  |  |  |  |  |  |
| 4/29/94 8:00 | 1,718.8 |  |  |  |  |  |  |
| 4/30/94 12:30 | 1,718.8 |  |  |  |  |  |  |
| 5/1/94 10:10 | 1,718.8 |  |  |  |  |  |  |
| 5/2/94 6:45 | 1,718.8 | 48 |  |  |  |  |  |
| 5/3/94 10:05 | 1,718.9 | 49 |  |  |  |  |  |
| 5/6/94 1:30 | 1,718.9 | 50 |  |  |  |  |  |
| 5/7/94 0:30 | 1,718.8 | 51 |  |  |  |  |  |
| 5/8/94 12:00 | 1,718.7 | 52 |  |  |  |  |  |
| 5/9/94 0:50 | 1,718.8 | 54 |  |  |  |  |  |
| 5/10/94 0:45 | 1,718.7 | 54 |  |  |  |  |  |
| 5/11/94 4:30 | 1,718.7 | 56 |  |  |  |  |  |
| 5/12/94 0:50 | 1,718.7 | 56 |  |  |  |  |  |
| 5/13/94 5:00 | 1,718.7 | 58 |  |  |  |  |  |
| 5/15/94 4:15 | 1,718.6 | 61 |  |  |  |  |  |
| 5/16/94 1:20 | 1,718.6 | 61 |  |  |  |  |  |
| 5/17/94 0:50 | 1,718.6 | 62 |  |  |  |  |  |
| 5/18/94 0:45 | 1,718.5 | 62 |  |  |  |  |  |
| 5/19/94 2:00 | 1,718.5 | 63 |  |  |  |  |  |
| 5/20/94 1:00 | 1,718.5 | 66 |  |  |  |  |  |
| 5/21/94 2:30 | 1,718.5 | 66 |  |  |  |  |  |
| 5/23/94 1:18 | 1,718.4 | 66 |  |  |  |  |  |
| 5/25/94 1:27 | 1,718.3 | 68 |  |  |  |  |  |
| 5/28/94 4:45 | 1,718.2 | 67 |  |  |  |  |  |
| 5/31/94 9:05 | 1,718.2 | 68 |  |  |  |  |  |
| 6/6/94 0:00 | 1,718.1 | 70 |  |  |  |  |  |
| 6/8/94 16:00 | 1,718.0 | 69 |  |  |  |  |  |
| 6/10/94 4:50 | 1,717.9 | 68 |  |  |  |  |  |
| 6/11/94 5:10 | 1,717.9 | 68 |  |  |  |  |  |
| 6/13/94 0:45 | 1,717.9 | 67 |  |  |  |  |  |
| 6/16/94 17:00 | 1,718.0 |  |  |  |  |  |  |
| 6/17/94 14:00 | 1,718.0 | 67 |  |  |  |  |  |
| 6/17/94 12:00 | 1,718.1 | 66 |  |  |  |  |  |
| 6/15/94 16:10 | 1,718.1 | 66 |  |  |  |  |  |
| 6/18/94 12:30 | 1,718.3 |  |  |  |  |  |  |
| 6/18/94 17:00 | 1,718.4 | 66 |  |  |  |  |  |
| 6/19/94 1:13 | 1,718.5 | 66 |  |  |  |  |  |
| 6/20/94 1:00 | 1,718.6 | 66 |  |  |  |  |  |
| 6/21/94 5:20 | 1,718.6 | 70 |  |  |  |  |  |
| 6/23/94 1:05 | 1,718.8 | 70 |  |  |  |  |  |
| 6/24/94 9:30 | 1,719.0 | 71 |  |  |  |  |  |
| 6/28/94 10:00 | 1,718.9 | 70 |  |  |  |  |  |
| 6/30/94 16:45 | 1,718.7 | 70 |  |  |  |  |  |
| 7/1/94 2:15 | 1,718.7 | 70 |  |  |  |  |  |
| 7/4/94 5:10 | 1,718.5 | 71 |  |  |  |  |  |
| 7/5/94 1:10 | 1,718.5 | 71 |  |  |  |  |  |
| 7/6/94 4:30 | 1,718.4 | 71 |  |  |  |  |  |
| 7/7/94 9:49 | 1,718.6 | 72 |  |  |  |  |  |
| 7/8/94 11:12 | 1,718.6 | 71 |  |  |  |  |  |
| 7/9/94 5:25 | 1,718.7 | 69 |  |  |  |  |  |
| 7/11/94 0:00 | 1,718.9 | 69 |  |  |  |  |  |
| 7/13/94 12:10 | 1,719.0 | 71 |  |  |  |  |  |
| 7/14/94 17:05 | 1,719.0 | 72 |  |  |  |  |  |
| 7/15/94 1:15 | 1,719.0 | 73 |  |  |  |  |  |
| 7/16/94 12:04 | 1,719.0 | 72 |  |  |  |  |  |
| 7/17/94 4:00 | 1,718.9 | 73 |  |  |  |  |  |
| 7/18/94 1:30 | 1,718.9 | 73 |  |  |  |  |  |
| 7/19/94 12:50 | 1,718.8 | 73 |  |  |  |  |  |
| 7/20/94 1:30 | 1,718.8 | 73 |  |  |  |  |  |
| 7/21/94 1:00 | 1,718.7 | 73 |  |  |  |  |  |
| 7/22/94 4:15 | 1,718.7 | 73 |  |  |  |  |  |
| 7/24/94 13:00 | 1,718.7 |  |  |  |  |  |  |
| 7/25/94 12:45 | 1,718.7 | 74 |  |  |  |  |  |
| 7/28/94 12:45 | 1,718.5 | 73 |  |  |  |  |  |
| 7/29/94 12:50 | 1,718.4 | 72 |  |  |  |  |  |
| 7/31/94 12:30 | 1,718.4 |  |  |  |  |  |  |
| 8/1/94 12:30 | 1,718.3 | 74 |  |  |  |  |  |
| 8/4/94 1:00 | 1,718.2 | 76 |  |  |  |  |  |
| 8/9/94 1:20 | 1,718.1 | 72 |  |  |  |  |  |
| 8/10/94 10:50 | 1,718.3 | 70 |  |  |  |  |  |
| 8/10/94 16:16 | 1,718.3 | 68 |  |  |  |  |  |
| 8/11/94 1:45 | 1,718.4 | 68 |  |  |  |  |  |
| 8/12/94 1:00 | 1,718.3 | 68 |  |  |  |  |  |
| 8/13/94 5:30 | 1,718.3 | 69 |  |  |  |  |  |
| 8/14/94 0:00 | 1,718.3 | 69 |  |  |  |  |  |
| 8/15/94 1:00 | 1,718.3 | 69 |  |  |  |  |  |
| 8/16/94 0:00 | 1,718.3 | 69 |  |  |  |  |  |
| 8/17/94 0:00 | 1,718.2 | 69 |  |  |  |  |  |
| 8/18/94 0:00 | 1,718.2 | 68 |  |  |  |  |  |
| 8/18/94 12:10 | 1,718.3 | 68 |  |  |  |  |  |
| 8/19/94 2:10 | 1,718.3 | 69 |  |  |  |  |  |
| 8/20/94 0:05 | 1,718.3 | 69 |  |  |  |  |  |
| 8/21/94 0:15 | 1,718.2 | 70 |  |  |  |  |  |
| 8/22/94 0:15 | 1,718.2 | 69 |  |  |  |  |  |
| 8/25/94 0:05 | 1,718.0 | 70 |  |  |  |  |  |
| 8/26/94 1:10 | 1,718.0 | 71 |  |  |  |  |  |
| 8/29/94 1:00 | 1,717.9 | 73 |  |  |  |  |  |
| 8/31/94 4:35 | 1,717.9 | 68 |  |  |  |  |  |
| 9/1/94 1:00 | 1,717.9 | 67 |  |  |  |  |  |
| 9/2/94 23:50 | 1,718.0 | 66 |  |  |  |  |  |
| 9/4/94 0:00 | 1,717.9 | 64 |  |  |  |  |  |
| 9/5/94 0:05 | 1,717.9 | 64 |  |  |  |  |  |
| 9/6/94 0:40 | 1,717.9 | 64 |  |  |  |  |  |
| 9/7/94 0:45 | 1,717.8 | 63 |  |  |  |  |  |
| 9/9/94 16:00 | 1,717.9 | 64 |  |  |  |  |  |
| 9/11/94 4:17 | 1,717.8 | 68 |  |  |  |  |  |
| 9/12/94 4:17 | 1,717.8 | 69 |  |  |  |  |  |
| 9/13/94 4:17 | 1,717.8 | 69 |  |  |  |  |  |
| 9/14/94 1:20 | 1,717.8 | 69 |  |  |  |  |  |
| 9/15/94 0:40 | 1,717.8 | 70 |  |  |  |  |  |
| 9/17/94 9:15 | 1,717.8 | 66 |  |  |  |  |  |
| 9/18/94 0:05 | 1,717.8 | 68 |  |  |  |  |  |
| 9/19/94 23:55 | 1,717.8 | 68 |  |  |  |  |  |
| 9/20/94 0:30 | 1,717.8 | 68 |  |  |  |  |  |
| 9/21/94 0:40 | 1,717.8 | 68 |  |  |  |  |  |
| 9/22/94 0:10 | 1,717.7 | 64 |  |  |  |  |  |
| 9/24/94 0:40 | 1,717.6 | 61 |  |  |  |  |  |
| 9/26/94 1:00 | 1,717.5 | 61 |  |  |  |  |  |
| 9/27/94 0:30 | 1,717.5 | 60 |  |  |  |  |  |
| 9/29/94 0:45 | 1,717.5 | 60 |  |  |  |  |  |
| 9/30/94 0:10 | 1,717.5 | 58 |  |  |  |  |  |
| 10/1/94 0:10 | 1,717.4 | 58 |  |  |  |  |  |
| 10/2/94 5:05 | 1,717.4 | 58 |  |  |  |  |  |
| 10/5/94 0:40 | 1,717.5 | 55 |  |  |  |  |  |
| 10/6/94 0:35 | 1,717.5 | 54 |  |  |  |  |  |
| 10/7/94 1:00 | 1,717.5 | 56 |  |  |  |  |  |
| 10/8/94 0:20 | 1,717.6 | 55 |  |  |  |  |  |
| 10/9/94 1:00 | 1,717.6 | 53 |  |  |  |  |  |
| 10/10/94 2:10 | 1,717.6 | 53 |  |  |  |  |  |
| 10/12/94 1:00 | 1,717.5 | 53 |  |  |  |  |  |
| 10/14/94 0:30 | 1,717.5 | 52 |  |  |  |  |  |
| 10/17/94 1:00 | 1,717.6 | 55 |  |  |  |  |  |
| 10/21/94 7:35 | 1,717.6 | 53 |  |  |  |  |  |
| 10/22/94 5:00 | 1,717.6 | 53 |  |  |  |  |  |
| 10/23/94 23:15 | 1,717.6 | 50 |  |  |  |  |  |
| 10/26/94 4:45 | 1,717.6 | 46 |  |  |  |  |  |
| 10/30/94 7:28 | 1,717.4 | 45 |  |  |  |  |  |
| 10/31/94 13:00 | 1,717.5 | 45 |  |  |  |  |  |
| 11/1/94 19:00 | 1,717.5 | 44 |  |  |  |  |  |
| 11/5/94 19:35 | 1,717.5 | 41 |  |  |  |  |  |
| 11/9/94 18:00 | 1,717.5 | 41 |  |  |  |  |  |
| 11/12/94 4:15 | 1,717.5 | 40 |  |  |  |  |  |
| 11/13/94 9:00 | 1,717.5 | 40 |  |  |  |  |  |
| 11/16/94 21:00 | 1,717.5 | 39 |  |  |  |  |  |
| 11/18/94 19:55 | 1,717.5 | 38 |  |  |  |  |  |
| 11/24/94 4:45 | 1,717.5 | 33 |  |  |  |  |  |
| 11/26/94 8:00 | 1,717.5 | 34 |  |  |  |  |  |
| 11/30/94 18:00 | 1,717.5 | 33 |  |  |  |  |  |
| 12/7/94 5:00 | 1,717.6 | 33 |  |  |  |  |  |
| 12/9/94 8:30 | 1,717.5 | 34 |  |  |  |  |  |
| 12/11/94 0:00 | 1,717.5 | 34 |  |  |  |  |  |
| 12/16/94 16:30 | 1,717.5 | 33 |  |  |  |  |  |
| 12/21/94 1:30 | 1,717.5 | 35 |  |  |  |  |  |
| 12/23/94 4:45 | 1,717.5 | 35 |  |  |  |  |  |
| 12/24/94 4:50 | 1,717.4 | 35 |  |  |  |  |  |
| 12/28/94 0:20 | 1,717.5 | 36 |  |  |  |  |  |
| 1/1/95 0:30 | 1,717.5 | 35 |  |  |  |  |  |
| 1/4/95 1:12 | 1,717.5 | 35 |  |  |  |  |  |
| 1/6/95 12:55 | 1,717.4 | 36 |  |  |  |  |  |
| 1/13/95 0:00 | 1,717.5 | 36 |  |  |  |  |  |
| 1/16/95 1:00 | 1,717.4 | 36 |  |  |  |  |  |
| 1/20/95 0:40 | 1,717.4 | 38 |  |  |  |  |  |
| 1/26/95 0:15 | 1,717.5 | 36 |  |  |  |  |  |
| 1/30/95 1:45 | 1,717.5 | 36 |  |  |  |  |  |
| 2/2/95 1:00 | 1,717.4 | 36 |  |  |  |  |  |
| 2/11/95 1:00 | 1,717.4 | 37 |  |  |  |  |  |
| 2/14/95 5:00 | 1,717.4 | 37 |  |  |  |  |  |
| 2/19/95 17:30 | 1,717.4 | 37 |  |  |  |  |  |
| 2/25/95 3:45 | 1,717.4 | 37 |  |  |  |  |  |
| 3/3/95 13:30 | 1,717.4 | 38 |  |  |  |  |  |
| 3/5/95 12:50 | 1,717.4 | 38 |  |  |  |  |  |
| 3/11/95 23:50 | 1,717.4 | 38 |  |  |  |  |  |
| 3/12/95 13:10 | 1,717.4 | 38 |  |  |  |  |  |
| 3/12/95 23:45 | 1,717.7 | 38 |  |  |  |  |  |
| 3/13/95 4:40 | 1,717.7 | 38 |  |  |  |  |  |
| 3/13/95 12:30 | 1,718.1 | 38 |  |  |  |  |  |
| 3/13/95 18:02 | 1,718.1 | 38 |  |  |  |  |  |
| 3/14/95 4:45 | 1,718.4 | 38 |  |  |  |  |  |
| 3/14/95 12:45 | 1,718.8 | 38 |  |  |  |  |  |
| 3/15/95 2:15 | 1,718.7 | 38 |  |  |  |  |  |
| 3/15/95 22:55 | 1,718.7 | 38 |  |  |  |  |  |
| 3/16/95 12:30 | 1,718.7 | 38 |  |  |  |  |  |
| 3/17/95 4:35 | 1,718.8 | 38 |  |  |  |  |  |
| 3/18/95 2:10 | 1,718.8 | 39 |  |  |  |  |  |
| 3/18/95 11:00 | 1,718.7 | 39 |  |  |  |  |  |
| 3/19/95 2:35 | 1,718.7 | 39 |  |  |  |  |  |
| 3/20/95 4:15 | 1,718.7 | 40 |  |  |  |  |  |
| 3/21/95 4:15 | 1,718.7 | 40 |  |  |  |  |  |
| 3/21/95 3:25 | 1,718.6 | 40 |  |  |  |  |  |
| 3/22/95 1:25 | 1,718.6 | 41 |  |  |  |  |  |
| 3/23/95 0:50 | 1,718.5 | 40 |  |  |  |  |  |
| 3/24/95 1:05 | 1,718.5 | 40 |  |  |  |  |  |
| 3/25/95 1:07 | 1,718.5 | 40 |  |  |  |  |  |
| 3/26/95 0:10 | 1,718.5 | 41 |  |  |  |  |  |
| 3/27/95 0:00 | 1,718.6 | 39 |  |  |  |  |  |
| 3/28/95 1:00 | 1,718.8 | 36 |  |  |  |  |  |
| 3/29/95 0:30 | 1,719.1 | 37 |  |  |  |  |  |
| 3/30/95 0:05 | 1,719.2 | 36 |  |  |  |  |  |
| 3/30/95 7:30 | 1,719.2 | 35 |  |  |  |  |  |
| 3/30/95 19:40 | 1,719.2 | 36 |  |  |  |  |  |
| 4/1/95 0:45 | 1,719.1 | 35 |  |  |  |  |  |
| 4/1/95 14:45 | 1,719.1 | 36 |  |  |  |  |  |
| 4/2/95 0:45 | 1,719.0 | 37 |  |  |  |  |  |
| 4/2/95 18:45 | 1,719.0 | 39 |  |  |  |  |  |
| 4/4/95 0:45 | 1,718.9 | 39 |  |  |  |  |  |
| 4/4/95 8:00 | 1,718.9 | 36 |  |  |  |  |  |
| 4/6/95 0:20 | 1,718.7 | 38 |  |  |  |  |  |
| 4/7/95 1:45 | 1,718.7 | 38 |  |  |  |  |  |
| 4/8/95 17:25 | 1,718.7 | 38 |  |  |  |  |  |
| 4/9/95 1:00 | 1,718.6 | 37 |  |  |  |  |  |
| 4/9/95 0:55 | 1,718.6 | 37 |  |  |  |  |  |
| 4/12/95 0:55 | 1,718.6 | 34 |  |  |  |  |  |
| 4/13/95 1:45 | 1,718.6 | 34 |  |  |  |  |  |
| 4/14/95 0:00 | 1,718.5 | 36 |  |  |  |  |  |
| 4/15/95 4:30 | 1,718.5 | 38 |  |  |  |  |  |
| 4/17/95 0:30 | 1,719.2 | 38 |  |  |  |  |  |
| 4/17/95 11:00 | 1,719.2 | 38 |  |  |  |  |  |
| 4/18/95 1:20 | 1,719.3 | 38 |  |  |  |  |  |
| 4/18/95 12:30 | 1,719.4 | 38 |  |  |  |  |  |
| 4/19/95 0:10 | 1,719.5 | 39 |  |  |  |  |  |
| 4/20/95 0:25 | 1,719.8 | 39 |  |  |  |  |  |
| 4/20/95 18:20 | 1,719.9 | 39 |  |  |  |  |  |
| 4/21/95 5:25 | 1,719.9 | 40 |  |  |  |  |  |
| 4/21/95 17:30 | 1,719.9 | 41 |  |  |  |  |  |
| 4/22/95 0:30 | 1,719.9 | 40 |  |  |  |  |  |
| 4/22/95 16:50 | 1,719.8 | 42 |  |  |  |  |  |
| 4/24/95 1:35 | 1,719.8 | 43 |  |  |  |  |  |
| 4/24/95 17:55 | 1,719.6 | 44 |  |  |  |  |  |
| 4/25/95 2:05 | 1,719.6 | 44 |  |  |  |  |  |
| 4/25/95 12:15 | 1,719.6 | 43 |  |  |  |  |  |
| 4/26/95 4:10 | 1,719.6 | 43 |  |  |  |  |  |
| 4/27/95 3:40 | 1,719.5 | 43 |  |  |  |  |  |
| 4/28/95 7:25 | 1,719.4 | 44 |  |  |  |  |  |
| 4/28/95 13:05 | 1,719.4 | 44 |  |  |  |  |  |
| 4/29/95 18:30 | 1,719.4 | 44 |  |  |  |  |  |
| 4/29/95 13:15 | 1,719.4 | 45 |  |  |  |  |  |
| 4/30/95 12:55 | 1,719.3 | 45 |  |  |  |  |  |
| 5/1/95 13:20 | 1,719.2 | 45 |  |  |  |  |  |
| 5/2/95 13:30 | 1,719.1 | 46 |  |  |  |  |  |
| 5/3/95 0:00 | 1,719.1 | 48 |  |  |  |  |  |
| 5/3/95 13:00 | 1,719.1 | 47 |  |  |  |  |  |
| 5/3/95 17:00 | 1,719.1 | 47 |  |  |  |  |  |
| 5/5/95 4:20 | 1,719.0 | 49 |  |  |  |  |  |
| 5/6/95 0:45 | 1,719.0 | 47 |  |  |  |  |  |
| 5/6/95 10:30 | 1,718.9 | 47 |  |  |  |  |  |
| 5/7/95 0:10 | 1,718.9 | 47 |  |  |  |  |  |
| 5/7/95 16:30 | 1,718.9 | 50 |  |  |  |  |  |
| 5/8/95 1:10 | 1,718.9 | 50 |  |  |  |  |  |
| 5/8/95 23:50 | 1,718.9 | 52 |  |  |  |  |  |
| 5/9/95 3:50 | 1,718.9 | 52 |  |  |  |  |  |
| 5/10/95 0:15 | 1,718.9 | 51 |  |  |  |  |  |
| 5/10/95 6:50 | 1,719.0 | 51 |  |  |  |  |  |
| 5/11/95 5:15 | 1,719.1 | 50 |  |  |  |  |  |
| 5/12/95 6:45 | 1,719.2 | 50 |  |  |  |  |  |
| 5/13/95 2:00 | 1,719.2 | 53 |  |  |  |  |  |
| 5/13/95 11:00 | 1,719.3 | 53 |  |  |  |  |  |
| 5/13/95 20:15 | 1,719.3 | 52 |  |  |  |  |  |
| 5/14/95 0:10 | 1,719.3 | 52 |  |  |  |  |  |
| 5/14/95 7:00 | 1,719.3 | 51 |  |  |  |  |  |
| 5/15/95 0:55 | 1,719.3 | 54 |  |  |  |  |  |
| 5/16/95 1:15 | 1,719.5 | 54 |  |  |  |  |  |
| 5/17/95 0:50 | 1,719.3 | 54 |  |  |  |  |  |
| 5/20/95 2:30 | 1,719.1 | 58 |  |  |  |  |  |
| 5/21/95 1:45 | 1,718.9 | 56 |  |  |  |  |  |
| 5/23/95 3:30 | 1,718.8 | 56 |  |  |  |  |  |
| 5/24/95 1:00 | 1,718.8 | 56 |  |  |  |  |  |
| 5/24/95 13:30 | 1,718.9 | 56 |  |  |  |  |  |
| 5/25/95 0:05 | 1,718.9 | 58 |  |  |  |  |  |
| 5/26/95 1:00 | 1,718.8 | 57 |  |  |  |  |  |
| 5/28/95 17:30 | 1,719.0 | 55 |  |  |  |  |  |
| 5/31/95 2:00 | 1,718.9 | 61 |  |  |  |  |  |
| 6/1/95 1:00 | 1,718.9 | 62 |  |  |  |  |  |
| 6/2/95 1:25 | 1,718.9 | 58 |  |  |  |  |  |
| 6/3/95 5:10 | 1,718.8 | 58 |  |  |  |  |  |
| 6/5/95 0:55 | 1,718.8 | 60 |  |  |  |  |  |
| 6/6/95 5:20 | 1,718.7 | 63 |  |  |  |  |  |
| 6/6/95 23:50 | 1,718.7 | 65 |  |  |  |  |  |
| 6/8/95 1:00 | 1,718.7 | 64 |  |  |  |  |  |
| 6/8/95 12:10 | 1,718.7 | 62 |  |  |  |  |  |
| 6/9/95 0:50 | 1,718.7 | 61 |  |  |  |  |  |
| 6/10/95 0:10 | 1,718.7 | 61 |  |  |  |  |  |
| 6/14/95 0:35 | 1,718.6 | 63 |  |  |  |  |  |
| 6/15/95 0:20 | 1,718.6 | 65 |  |  |  |  |  |
| 6/16/95 12:00 | 1,718.6 | 68 |  |  |  |  |  |
| 6/18/95 3:45 | 1,718.6 | 72 |  |  |  |  |  |
| 6/19/95 1:45 | 1,718.6 | 73 |  |  |  |  |  |
| 6/20/95 0:35 | 1,718.6 | 74 |  |  |  |  |  |
| 6/21/95 0:01 | 1,718.5 | 74 |  |  |  |  |  |
| 6/22/95 1:20 | 1,718.5 | 75 |  |  |  |  |  |
| 6/22/95 12:45 | 1,718.4 | 74 |  |  |  |  |  |
| 6/23/95 0:45 | 1,718.4 | 75 |  |  |  |  |  |
| 6/26/95 0:20 | 1,718.3 | 73 |  |  |  |  |  |
| 6/26/95 16:10 | 1,718.3 | 73 |  |  |  |  |  |
| 6/27/95 12:00 | 1,718.3 | 73 |  |  |  |  |  |
| 6/28/95 0:30 | 1,718.3 | 73 |  |  |  |  |  |
| 7/2/95 11:05 | 1,718.0 | 68 |  |  |  |  |  |
| 7/3/95 23:35 | 1,718.0 | 70 |  |  |  |  |  |
| 7/4/95 5:15 | 1,718.0 | 70 |  |  |  |  |  |
| 7/5/95 1:30 | 1,718.0 | 69 |  |  |  |  |  |
| 7/5/95 5:10 | 1,718.1 | 69 |  |  |  |  |  |
| 7/6/95 2:00 | 1,718.1 | 66 |  |  |  |  |  |
| 7/6/95 5:10 | 1,718.2 | 66 |  |  |  |  |  |
| 7/7/95 0:30 | 1,718.4 | 67 |  |  |  |  |  |
| 7/8/95 0:00 | 1,718.7 | 66 |  |  |  |  |  |
| 7/9/95 5:25 | 1,719.0 | 70 |  |  |  |  |  |
| 7/10/95 4:50 | 1,718.9 | 70 |  |  |  |  |  |
| 7/11/95 0:05 | 1,718.9 | 70 |  |  |  |  |  |
| 7/12/95 0:00 | 1,718.9 | 72 |  |  |  |  |  |
| 7/13/95 16:55 | 1,718.8 | 74 |  |  |  |  |  |
| 7/14/95 4:30 | 1,718.9 | 76 |  |  |  |  |  |
| 7/15/95 1:00 | 1,718.9 | 78 |  |  |  |  |  |
| 7/17/95 1:00 | 1,718.6 | 76 |  |  |  |  |  |
| 7/18/95 4:00 | 1,718.5 | 74 |  |  |  |  |  |
| 7/19/95 0:40 | 1,718.4 | 75 |  |  |  |  |  |
| 7/20/95 4:25 | 1,718.4 | 75 |  |  |  |  |  |
| 7/21/95 4:10 | 1,718.4 | 75 |  |  |  |  |  |
| 7/22/95 4:30 | 1,718.4 | 75 |  |  |  |  |  |
| 7/22/95 16:30 | 1,718.4 | 75 |  |  |  |  |  |
| 7/24/95 0:05 | 1,718.3 | 76 |  |  |  |  |  |
| 7/27/95 0:40 | 1,718.2 | 74 |  |  |  |  |  |
| 7/27/95 8:35 | 1,718.2 | 73 |  |  |  |  |  |
| 7/28/95 0:20 | 1,718.2 | 74 |  |  |  |  |  |
| 7/29/95 12:30 | 1,717.9 | 75 |  |  |  |  |  |
| 7/31/95 6:40 | 1,718.2 | 74 |  |  |  |  |  |
| 8/1/95 1:20 | 1,718.0 | 72 |  |  |  |  |  |
| 8/1/95 12:40 | 1,717.9 | 70 |  |  |  |  |  |
| 8/2/95 4:50 | 1,718.0 | 72 |  |  |  |  |  |
| 8/5/95 4:50 | 1,717.8 | 74 |  |  |  |  |  |
| 8/6/95 4:50 | 1,718.2 | 74 |  |  |  |  |  |
| 8/7/95 4:55 | 1,718.2 | 74 |  |  |  |  |  |
| 8/8/95 2:45 | 1,718.1 | 76 |  |  |  |  |  |
| 8/10/95 3:00 | 1,718.0 | 77 |  |  |  |  |  |
| 8/11/95 1:30 | 1,718.0 | 75 |  |  |  |  |  |
| 8/13/95 15:10 | 1,718.0 | 75 |  |  |  |  |  |
| 8/15/95 0:01 | 1,718.1 | 73 |  |  |  |  |  |
| 8/17/95 4:03 | 1,718.0 | 74 |  |  |  |  |  |
| 8/18/95 1:05 | 1,718.1 | 75 |  |  |  |  |  |
| 8/21/95 1:00 | 1,718.1 | 75 |  |  |  |  |  |
| 8/23/95 1:00 | 1,718.0 | 75 |  |  |  |  |  |
| 8/25/95 0:35 | 1,717.9 | 76 |  |  |  |  |  |
| 8/26/95 4:03 | 1,717.9 | 75 |  |  |  |  |  |
| 8/26/95 7:00 | 1,717.9 | 75 |  |  |  |  |  |
| 8/27/95 15:03 | 1,718.0 | 74 |  |  |  |  |  |
| 8/28/95 1:03 | 1,717.9 | 75 |  |  |  |  |  |
| 8/30/95 4:45 | 1,717.9 | 73 |  |  |  |  |  |
| 8/31/95 0:25 | 1,718.1 | 73 |  |  |  |  |  |
| 9/2/95 16:00 | 1,718.3 | 71 |  |  |  |  |  |
| 9/4/95 12:30 | 1,718.3 | 72 |  |  |  |  |  |
| 9/5/95 2:55 | 1,718.3 | 72 |  |  |  |  |  |
| 9/5/95 16:55 | 1,718.2 | 73 |  |  |  |  |  |
| 9/5/95 17:01 | 1,718.3 | 73 |  |  |  |  |  |
| 9/7/95 16:01 | 1,718.3 | 68 |  |  |  |  |  |
| 9/8/95 16:03 | 1,718.3 | 66 |  |  |  |  |  |
| 9/11/95 18:00 | 1,718.2 | 65 |  |  |  |  |  |
| 9/13/95 20:50 | 1,718.1 | 67 |  |  |  |  |  |
| 9/14/95 13:20 | 1,718.1 | 65 |  |  |  |  |  |
| 9/15/95 12:45 | 1,718.1 | 64 |  |  |  |  |  |
| 9/16/95 0:45 | 1,718.1 | 64 |  |  |  |  |  |
| 9/16/95 18:50 | 1,718.0 | 64 |  |  |  |  |  |
| 9/17/95 18:15 | 1,717.9 | 63 |  |  |  |  |  |
| 9/18/95 13:30 | 1,717.9 | 62 |  |  |  |  |  |
| 9/20/95 0:45 | 1,717.9 | 60 |  |  |  |  |  |
| 9/21/95 4:04 | 1,717.9 | 58 |  |  |  |  |  |
| 9/22/95 0:15 | 1,717.8 | 54 |  |  |  |  |  |
| 9/23/95 0:05 | 1,717.7 | 54 |  |  |  |  |  |
| 9/23/95 11:40 | 1,717.7 | 52 |  |  |  |  |  |
| 9/24/95 0:55 | 1,717.8 | 53 |  |  |  |  |  |
| 9/24/95 9:15 | 1,717.9 | 53 |  |  |  |  |  |
| 9/25/95 0:05 | 1,717.8 | 53 |  |  |  |  |  |
| 9/26/95 0:10 | 1,717.7 | 56 |  |  |  |  |  |
| 9/27/95 0:02 | 1,717.6 |  |  |  |  |  |  |
| 9/30/95 4:10 | 1,717.9 | 58 |  |  |  |  |  |
| 10/1/95 0:15 | 1,717.9 | 56 |  |  |  |  |  |
| 10/2/95 0:30 | 1,718.0 | 57 |  |  |  |  |  |
| 10/2/95 8:03 | 1,718.1 | 56 |  |  |  |  |  |
| 10/3/95 4:02 | 1,718.0 | 56 |  |  |  |  |  |
| 10/3/95 17:02 | 1,717.9 | 56 |  |  |  |  |  |
| 10/4/95 0:03 | 1,717.7 | 56 |  |  |  |  |  |
| 10/4/95 12:45 | 1,717.8 | 55 |  |  |  |  |  |
| 10/5/95 8:45 | 1,718.1 | 55 |  |  |  |  |  |
| 10/6/95 0:03 | 1,718.3 | 56 |  |  |  |  |  |
| 10/6/95 18:35 | 1,718.3 | 55 |  |  |  |  |  |
| 10/6/95 23:55 | 1,718.3 | 56 |  |  |  |  |  |
| 10/9/95 12:05 | 1,718.4 | 57 |  |  |  |  |  |
| 10/10/95 0:05 | 1,718.4 | 58 |  |  |  |  |  |
| 10/11/95 0:15 | 1,718.4 | 57 |  |  |  |  |  |
| 10/12/95 0:05 | 1,718.3 | 58 |  |  |  |  |  |
| 10/13/95 0:10 | 1,718.3 | 58 |  |  |  |  |  |
| 10/15/95 0:45 | 1,718.5 | 58 |  |  |  |  |  |
| 10/16/95 9:45 | 1,718.4 | 49 |  |  |  |  |  |
| 10/21/95 0:05 | 1,718.8 |  |  |  |  |  |  |
| 10/21/95 4:00 | 1,718.7 |  |  |  |  |  |  |
| 10/21/95 6:45 | 1,718.7 |  |  |  |  |  |  |
| 10/21/95 18:20 | 1,718.6 |  |  |  |  |  |  |
| 10/22/95 0:15 | 1,718.6 |  |  |  |  |  |  |
| 10/22/95 7:00 | 1,718.5 | 44 |  |  |  |  |  |
| 10/23/95 0:05 | 1,718.5 |  |  |  |  |  |  |
| 10/24/95 3:00 | 1,719.3 |  |  |  |  |  |  |
| 10/25/95 0:15 | 1,718.9 |  |  |  |  |  |  |
| 10/26/95 0:10 | 1,718.6 |  |  |  |  |  |  |
| 10/27/95 0:00 | 1,718.5 |  |  |  |  |  |  |
| 10/27/95 23:56 | 1,718.4 |  |  |  |  |  |  |
| 10/29/95 0:10 | 1,718.5 |  |  |  |  |  |  |
| 10/30/95 0:25 | 1,718.5 | 40 |  |  |  |  |  |
| 10/30/95 23:20 | 1,718.6 | 40 |  |  |  |  |  |
| 11/1/95 0:05 | 1,718.6 | 40 |  |  |  |  |  |
| 11/1/95 16:15 | 1,718.6 | 40 |  |  |  |  |  |
| 11/1/95 23:35 | 1,718.6 |  |  |  |  |  |  |
| 11/4/95 14:50 | 1,719.1 |  |  |  |  |  |  |
| 11/5/95 0:00 | 1,719.0 |  |  |  |  |  |  |
| 11/5/95 14:04 | 1,718.9 | 34 |  |  |  |  |  |
| 11/6/95 0:45 | 1,718.9 | 34 |  |  |  |  |  |
| 11/6/95 20:04 | 1,718.8 |  |  |  |  |  |  |
| 11/7/95 0:15 | 1,718.8 |  |  |  |  |  |  |
| 11/8/95 0:30 | 1,718.7 |  |  |  |  |  |  |
| 11/9/95 0:30 | 1,718.7 |  |  |  |  |  |  |
| 11/11/95 12:55 | 1,718.6 |  |  |  |  |  |  |
| 11/12/95 1:00 | 1,718.6 | 34 |  |  |  |  |  |
| 11/12/95 11:04 | 1,718.5 |  |  |  |  |  |  |
| 11/15/95 0:15 | 1,718.4 | 34 |  |  |  |  |  |
| 11/17/95 0:05 | 1,718.4 |  |  |  |  |  |  |
| 11/18/95 0:05 | 1,718.4 |  |  |  |  |  |  |
| 11/20/95 0:01 | 1,718.3 |  |  |  |  |  |  |
| 11/21/95 0:05 | 1,718.3 |  |  |  |  |  |  |
| 11/22/95 0:10 | 1,718.3 |  |  |  |  |  |  |
| 11/23/95 0:05 | 1,718.3 |  |  |  |  |  |  |
| 11/25/95 0:02 | 1,718.2 |  |  |  |  |  |  |
| 11/27/95 12:03 | 1,718.1 |  |  |  |  |  |  |
| 11/29/95 18:45 | 1,718.1 |  |  |  |  |  |  |
| 11/30/95 17:35 | 1,717.8 |  |  |  |  |  |  |
| 12/2/95 0:02 | 1,717.8 | 36 |  |  |  |  |  |
| 12/3/95 0:15 | 1,717.8 | 36 |  |  |  |  |  |
| 12/6/95 0:02 | 1,717.7 | 34 |  |  |  |  |  |
| 12/6/95 18:01 | 1,717.7 | 33 |  |  |  |  |  |
| 12/10/95 12:25 | 1,717.6 | 33 |  |  |  |  |  |
| 12/16/95 11:15 | 1,717.4 | 34 |  |  |  |  |  |
| 12/23/95 11:03 | 1,717.4 | 34 |  |  |  |  |  |
| 12/26/95 4:00 | 1,717.2 | 34 |  |  |  |  |  |
| 12/30/95 2:45 | 1,717.1 | 36 |  |  |  |  |  |
| 1/1/96 16:01 | 1,717.2 | 36 |  |  |  |  |  |
| 1/2/96 18:03 | 1,717.2 | 36 |  |  |  |  |  |
| 1/8/96 11:45 | 1,717.2 | 36 |  |  |  |  |  |
| 1/12/96 1:00 | 1,717.0 | 37 |  |  |  |  |  |
| 1/13/96 0:45 | 1,717.1 | 37 |  |  |  |  |  |
| 1/14/96 0:45 | 1,717.1 | 37 |  |  |  |  |  |
| 1/15/96 4:04 | 1,717.0 | 37 |  |  |  |  |  |
| 1/16/96 1:00 | 1,717.1 | 37 |  |  |  |  |  |
| 1/17/96 0:45 | 1,716.9 | 37 |  |  |  |  |  |
| 1/17/96 4:45 | 1,717.0 | 37 |  |  |  |  |  |
| 1/22/96 1:04 | 1,716.8 | 37 |  |  |  |  |  |
| 1/24/96 16:02 | 1,717.0 | 37 |  |  |  |  |  |
| 1/30/96 1:00 | 1,717.0 | 37 |  |  |  |  |  |
| 1/31/96 17:00 | 1,717.1 | 36 |  |  |  |  |  |
| 2/1/96 11:00 | 1,716.9 | 36 |  |  |  |  |  |
| 2/4/96 0:03 | 1,717.0 | 37 |  |  |  |  |  |
| 2/5/96 11:45 | 1,717.0 | 37 |  |  |  |  |  |
| 2/8/96 6:45 | 1,717.2 | 36 |  |  |  |  |  |
| 2/9/96 0:45 | 1,717.0 | 37 |  |  |  |  |  |
| 2/11/96 1:00 | 1,716.9 | 37 |  |  |  |  |  |
| 2/12/96 1:45 | 1,717.0 | 37 |  |  |  |  |  |
| 2/12/96 11:03 | 1,717.0 | 37 |  |  |  |  |  |
| 2/13/96 5:00 | 1,716.9 | 38 |  |  |  |  |  |
| 2/14/96 3:03 | 1,716.8 | 38 |  |  |  |  |  |
| 2/15/96 0:45 | 1,717.0 | 38 |  |  |  |  |  |
| 2/17/96 0:01 | 1,717.1 | 37 |  |  |  |  |  |
| 2/20/96 20:45 | 1,716.8 | 38 |  |  |  |  |  |
| 2/23/96 3:04 | 1,717.0 | 37 |  |  |  |  |  |
| 2/25/96 16:45 | 1,716.8 | 39 |  |  |  |  |  |
| 2/29/96 17:02 | 1,716.9 | 39 |  |  |  |  |  |
| 3/3/96 13:25 | 1,717.0 | 39 |  |  |  |  |  |
| 3/5/96 10:02 | 1,717.0 | 39 |  |  |  |  |  |
| 3/8/96 1:45 | 1,717.0 | 38 |  |  |  |  |  |
| 3/9/96 0:50 | 1,717.0 | 38 |  |  |  |  |  |
| 3/9/96 6:30 | 1,717.0 | 38 |  |  |  |  |  |
| 3/10/96 0:45 | 1,717.1 | 38 |  |  |  |  |  |
| 3/10/96 6:30 | 1,717.2 | 37 |  |  |  |  |  |
| 3/11/96 1:20 | 1,717.0 | 38 |  |  |  |  |  |
| 3/12/96 1:15 | 1,717.0 | 38 |  |  |  |  |  |
| 3/13/96 1:05 | 1,717.0 | 38 |  |  |  |  |  |
| 3/13/96 13:35 | 1,717.0 | 38 |  |  |  |  |  |
| 3/14/96 0:45 | 1,717.5 | 39 |  |  |  |  |  |
| 3/14/96 3:04 | 1,717.5 | 39 |  |  |  |  |  |
| 3/15/96 0:02 | 1,718.2 | 38 |  |  |  |  |  |
| 3/16/96 12:00 | 1,718.7 | 38 |  |  |  |  |  |
| 3/17/96 4:01 | 1,719.0 | 38 |  |  |  |  |  |
| 3/17/96 11:15 | 1,719.0 | 38 |  |  |  |  |  |
| 3/18/96 1:00 | 1,719.1 | 38 |  |  |  |  |  |
| 3/19/96 0:01 | 1,719.1 | 38 |  |  |  |  |  |
| 3/20/96 0:20 | 1,719.1 | 38 |  |  |  |  |  |
| 3/20/96 17:01 | 1,719.1 | 38 |  |  |  |  |  |
| 3/21/96 20:55 | 1,719.0 | 38 |  |  |  |  |  |
| 3/23/96 0:45 | 1,718.9 | 38 |  |  |  |  |  |
| 3/24/96 15:45 | 1,719.0 | 38 |  |  |  |  |  |
| 3/24/96 0:01 | 1,718.9 | 38 |  |  |  |  |  |
| 3/24/96 17:05 | 1,718.9 | 38 |  |  |  |  |  |
| 3/25/96 12:00 | 1,718.9 | 38 |  |  |  |  |  |
| 3/26/96 4:30 | 1,718.9 | 38 |  |  |  |  |  |
| 3/29/96 13:00 | 1,718.7 | 38 |  |  |  |  |  |
| 3/30/96 4:03 | 1,718.7 | 38 |  |  |  |  |  |
| 3/30/96 12:45 | 1,718.6 | 38 |  |  |  |  |  |
| 3/31/96 0:01 | 1,718.6 | 39 |  |  |  |  |  |
| 3/31/96 16:03 | 1,718.8 | 38 |  |  |  |  |  |
| 4/1/96 13:00 | 1,718.5 | 39 |  |  |  |  |  |
| 4/1/96 17:02 | 1,718.6 | 39 |  |  |  |  |  |
| 4/2/96 0:35 | 1,718.6 | 39 |  |  |  |  |  |
| 4/2/96 13:00 | 1,718.6 | 39 |  |  |  |  |  |
| 4/3/96 0:15 | 1,718.6 | 39 |  |  |  |  |  |
| 4/5/96 1:35 | 1,718.6 | 39 |  |  |  |  |  |
| 4/5/96 19:00 | 1,718.6 | 40 |  |  |  |  |  |
| 4/6/96 0:01 | 1,718.6 | 40 |  |  |  |  |  |
| 4/6/96 23:45 | 1,718.6 | 40 |  |  |  |  |  |
| 4/8/96 0:05 | 1,718.6 | 40 |  |  |  |  |  |
| 4/9/96 2:00 | 1,718.5 | 40 |  |  |  |  |  |
| 4/9/96 4:55 | 1,718.5 | 40 |  |  |  |  |  |
| 4/10/96 4:55 | 1,718.5 | 40 |  |  |  |  |  |
| 4/11/96 0:30 | 1,718.5 | 40 |  |  |  |  |  |
| 4/12/96 4:45 | 1,718.6 | 40 |  |  |  |  |  |
| 4/12/96 12:45 | 1,718.5 | 40 |  |  |  |  |  |
| 4/13/96 0:00 | 1,718.5 | 39 |  |  |  |  |  |
| 4/14/96 0:35 | 1,718.6 | 40 |  |  |  |  |  |
| 4/15/96 0:05 | 1,718.5 | 39 |  |  |  |  |  |
| 4/16/96 3:15 | 1,718.6 | 39 |  |  |  |  |  |
| 4/16/96 12:15 | 1,718.5 | 39 |  |  |  |  |  |
| 4/17/96 0:25 | 1,718.5 | 37 |  |  |  |  |  |
| 4/18/96 4:00 | 1,718.5 | 39 |  |  |  |  |  |
| 4/18/96 18:00 | 1,718.3 | 39 |  |  |  |  |  |
| 4/19/96 4:00 | 1,718.3 | 39 |  |  |  |  |  |
| 4/20/96 0:02 | 1,718.2 | 38 |  |  |  |  |  |
| 4/21/96 15:30 | 1,718.2 | 40 |  |  |  |  |  |
| 4/23/96 3:03 | 1,718.2 | 41 |  |  |  |  |  |
| 4/25/96 0:45 | 1,718.2 | 44 |  |  |  |  |  |
| 4/25/96 17:15 | 1,718.4 | 45 |  |  |  |  |  |
| 4/27/96 5:00 | 1,718.1 | 44 |  |  |  |  |  |
| 4/28/96 2:00 | 1,718.0 | 44 |  |  |  |  |  |
| 4/28/96 22:00 | 1,718.1 | 44 |  |  |  |  |  |
| 4/28/96 16:00 | 1,718.3 | 44 |  |  |  |  |  |
| 4/29/96 1:30 | 1,718.1 | 44 |  |  |  |  |  |
| 5/2/96 0:30 | 1,718.0 | 48 |  |  |  |  |  |
| 5/3/96 0:15 | 1,718.1 | 47 |  |  |  |  |  |
| 5/4/96 23:45 | 1,718.1 | 47 |  |  |  |  |  |
| 5/4/96 11:45 | 1,717.9 | 46 |  |  |  |  |  |
| 5/5/96 4:00 | 1,718.1 | 47 |  |  |  |  |  |
| 5/6/96 1:45 | 1,717.9 | 47 |  |  |  |  |  |
| 5/6/96 4:45 | 1,717.9 | 47 |  |  |  |  |  |
| 5/7/96 1:05 | 1,717.9 | 46 |  |  |  |  |  |
| 5/7/96 12:02 | 1,718.1 | 46 |  |  |  |  |  |
| 5/8/96 18:00 | 1,718.1 | 49 |  |  |  |  |  |
| 5/9/96 2:00 | 1,718.1 | 48 |  |  |  |  |  |
| 5/10/96 4:03 | 1,718.2 | 46 |  |  |  |  |  |
| 5/15/96 4:00 | 1,718.4 | 50 |  |  |  |  |  |
| 5/17/96 4:03 | 1,718.2 | 56 |  |  |  |  |  |
| 5/17/96 17:02 | 1,718.4 | 56 |  |  |  |  |  |
| 5/18/96 17:02 | 1,718.4 | 58 |  |  |  |  |  |
| 5/20/96 3:15 | 1,718.6 | 60 |  |  |  |  |  |
| 5/21/96 0:15 | 1,718.6 | 62 |  |  |  |  |  |
| 5/23/96 2:45 | 1,718.7 | 61 |  |  |  |  |  |
| 5/23/96 13:05 | 1,718.7 | 60 |  |  |  |  |  |
| 5/24/96 0:25 | 1,718.7 | 60 |  |  |  |  |  |
| 5/25/96 5:15 | 1,718.7 | 58 |  |  |  |  |  |
| 5/25/96 18:04 | 1,718.6 | 59 |  |  |  |  |  |
| 5/26/96 5:45 | 1,718.6 | 56 |  |  |  |  |  |
| 5/26/96 6:35 | 1,718.6 | 56 |  |  |  |  |  |
| 5/31/96 1:25 | 1,718.4 | 58 |  |  |  |  |  |
| 6/1/96 1:00 | 1,718.4 | 59 |  |  |  |  |  |
| 6/1/96 10:20 | 1,718.5 | 59 |  |  |  |  |  |
| 6/2/96 1:40 | 1,718.5 | 62 |  |  |  |  |  |
| 6/2/96 12:01 | 1,718.5 | 61 |  |  |  |  |  |
| 6/3/96 0:45 | 1,718.5 | 60 |  |  |  |  |  |
| 6/3/96 12:15 | 1,718.6 | 58 |  |  |  |  |  |
| 6/4/96 5:15 | 1,718.5 | 58 |  |  |  |  |  |
| 6/5/96 1:00 | 1,718.5 | 57 |  |  |  |  |  |
| 6/6/96 1:05 | 1,718.4 | 61 |  |  |  |  |  |
| 6/6/96 4:04 | 1,718.5 | 61 |  |  |  |  |  |
| 6/6/96 23:54 | 1,718.5 | 61 |  |  |  |  |  |
| 6/8/96 3:35 | 1,718.4 | 62 |  |  |  |  |  |
| 6/10/96 0:30 | 1,718.4 | 63 |  |  |  |  |  |
| 6/11/96 3:40 | 1,718.3 | 62 |  |  |  |  |  |
| 6/12/96 3:04 | 1,718.3 | 64 |  |  |  |  |  |
| 6/13/96 17:02 | 1,718.3 | 69 |  |  |  |  |  |
| 6/14/96 17:03 | 1,718.1 | 62 |  |  |  |  |  |
| 6/15/96 0:03 | 1,718.2 | 63 |  |  |  |  |  |
| 6/15/96 17:00 | 1,718.0 | 69 |  |  |  |  |  |
| 6/17/96 16:05 | 1,717.8 | 69 |  |  |  |  |  |
| 6/19/96 3:45 | 1,717.7 | 69 |  |  |  |  |  |
| 6/20/96 1:00 | 1,717.8 | 70 |  |  |  |  |  |
| 6/20/96 13:05 | 1,717.8 | 70 |  |  |  |  |  |
| 6/21/96 13:00 | 1,717.7 | 70 |  |  |  |  |  |
| 6/24/96 1:00 | 1,717.7 | 68 |  |  |  |  |  |
| 6/25/96 12:00 | 1,717.5 | 68 |  |  |  |  |  |
| 7/1/96 22:30 | 1,717.3 | 77 |  |  |  |  |  |
| 7/4/96 1:05 | 1,717.1 | 74 |  |  |  |  |  |
| 7/8/96 0:05 | 1,716.8 | 75 |  |  |  |  |  |
| 7/12/96 0:05 | 1,716.9 | 71 |  |  |  |  |  |
| 7/14/96 0:15 | 1,716.8 | 69 |  |  |  |  |  |
| 7/16/96 0:03 | 1,716.6 | 73 |  |  |  |  |  |
| 7/19/96 12:45 | 1,716.7 | 73 |  |  |  |  |  |
| 7/20/96 0:03 | 1,716.7 | 72 |  |  |  |  |  |
| 7/20/96 18:15 | 1,716.7 | 72 |  |  |  |  |  |
| 7/25/96 0:03 | 1,716.5 | 66 |  |  |  |  |  |
| 7/27/96 0:03 | 1,716.5 | 66 |  |  |  |  |  |
| 7/28/96 7:08 | 1,716.5 | 66 |  |  |  |  |  |
| 8/1/96 0:15 | 1,716.4 | 56 |  |  |  |  |  |
| 8/5/96 0:15 | 1,716.4 | 57 |  |  |  |  |  |
| 8/9/96 0:15 | 1,716.4 | 72 |  |  |  |  |  |
| 8/12/96 0:03 | 1,716.4 | 72 |  |  |  |  |  |
| 8/15/96 0:05 | 1,716.5 | 72 |  |  |  |  |  |
| 8/19/96 0:55 | 1,716.2 | 72 |  |  |  |  |  |
| 8/23/96 0:02 | 1,716.2 | 72 |  |  |  |  |  |
| 8/24/96 16:03 | 1,716.2 | 72 |  |  |  |  |  |
| 8/26/96 0:15 | 1,716.2 | 72 |  |  |  |  |  |
| 8/28/96 0:25 | 1,716.1 | 70 |  |  |  |  |  |
| 8/29/96 0:55 | 1,716.1 | 70 |  |  |  |  |  |
| 8/30/96 0:15 | 1,716.1 | 70 |  |  |  |  |  |
| 9/1/96 0:45 | 1,716.1 | 62 |  |  |  |  |  |
| 9/3/96 0:05 | 1,716.1 | 64 |  |  |  |  |  |
| 9/4/96 0:45 | 1,716.1 | 64 |  |  |  |  |  |
| 9/5/96 10:45 | 1,715.9 | 72 |  |  |  |  |  |
| 9/9/96 0:05 | 1,715.9 | 70 |  |  |  |  |  |
| 9/17/96 1:00 | 1,715.8 | 60 |  |  |  |  |  |
| 9/20/96 12:04 | 1,715.8 | 61 |  |  |  |  |  |
| 9/23/96 0:35 | 1,715.8 | 60 |  |  |  |  |  |
| 9/25/96 0:03 | 1,715.8 | 56 |  |  |  |  |  |
| 9/26/96 12:30 | 1,715.8 | 56 |  |  |  |  |  |
| 9/30/96 1:05 | 1,715.8 | 58 |  |  |  |  |  |
| 10/6/96 0:03 | 1,715.8 | 58 |  |  |  |  |  |
| 10/12/96 10:03 | 1,715.7 | 54 |  |  |  |  |  |
| 10/18/96 7:15 | 1,715.9 | 59 |  |  |  |  |  |
| 10/21/96 0:04 | 1,715.9 | 54 |  |  |  |  |  |
| 10/23/96 0:25 | 1,715.9 | 46 |  |  |  |  |  |
| 10/25/96 10:00 | 1,715.8 | 43 |  |  |  |  |  |
| 10/27/96 0:03 | 1,715.9 | 56 |  |  |  |  |  |
| 10/31/96 0:05 | 1,715.9 | 43 |  |  |  |  |  |
| 10/31/96 10:00 | 1,716.0 | 38 |  |  |  |  |  |
| 11/3/96 16:00 | 1,716.0 | 38 |  |  |  |  |  |
| 11/6/96 23:45 | 1,716.0 |  |  |  |  |  |  |
| 11/6/96 19:28 | 1,716.0 | 46 |  |  |  |  |  |
| 11/8/96 13:00 | 1,716.1 | 35 |  |  |  |  |  |
| 11/10/96 8:55 | 1,716.1 | 32 |  |  |  |  |  |
| 11/11/96 1:01 | 1,716.0 | 32 |  |  |  |  |  |
| 11/11/96 11:00 | 1,716.0 | 33 |  |  |  |  |  |
| 11/12/96 7:02 | 1,716.0 | 33 |  |  |  |  |  |
| 11/15/96 0:05 | 1,716.0 | 33 |  |  |  |  |  |
| 11/18/96 0:15 | 1,716.1 | 33 |  |  |  |  |  |
| 11/19/96 0:05 | 1,716.1 | 33 |  |  |  |  |  |
| 11/21/96 1:02 | 1,716.1 | 34 |  |  |  |  |  |
| 11/21/96 23:45 | 1,716.1 | 34 |  |  |  |  |  |
| 11/25/96 0:15 | 1,716.1 | 34 |  |  |  |  |  |
| 11/26/96 0:15 | 1,716.3 | 34 |  |  |  |  |  |
| 11/27/96 0:03 | 1,716.1 | 34 |  |  |  |  |  |
| 11/27/96 17:00 | 1,716.2 | 34 |  |  |  |  |  |
| 11/28/96 18:35 | 1,716.2 | 34 |  |  |  |  |  |
| 11/30/96 16:45 | 1,716.2 | 34 |  |  |  |  |  |
| 12/1/96 15:45 | 1,716.2 | 34 |  |  |  |  |  |
| 12/3/96 16:00 | 1,716.3 | 34 |  |  |  |  |  |
| 12/4/96 12:05 | 1,716.2 | 34 |  |  |  |  |  |
| 12/7/96 14:55 | 1,716.5 | 35 |  |  |  |  |  |
| 12/8/96 0:01 | 1,716.3 | 35 |  |  |  |  |  |
| 12/12/96 23:55 | 1,716.1 | 35 |  |  |  |  |  |
| 12/15/96 4:05 | 1,716.2 | 35 |  |  |  |  |  |
| 12/16/96 0:35 | 1,716.1 | 35 |  |  |  |  |  |
| 12/19/96 4:01 | 1,716.1 | 35 |  |  |  |  |  |
| 12/21/96 0:03 | 1,716.2 | 35 |  |  |  |  |  |
| 12/25/96 16:04 | 1,716.1 | 35 |  |  |  |  |  |
| 12/28/96 16:04 | 1,716.2 | 35 |  |  |  |  |  |
| 12/29/96 16:04 | 1,716.2 | 35 |  |  |  |  |  |
| 12/30/96 21:02 | 1,716.2 | 36 |  |  |  |  |  |
| 1/3/97 12:55 | 1,716.2 | 36 |  |  |  |  |  |
| 1/6/97 9:30 | 1,716.4 | 36 |  |  |  |  |  |
| 1/11/97 0:25 | 1,716.3 | 36 |  |  |  |  |  |
| 1/12/97 1:10 | 1,716.4 | 36 |  |  |  |  |  |
| 1/13/97 2:20 | 1,716.3 | 36 |  |  |  |  |  |
| 1/14/97 1:50 | 1,716.4 | 36 |  |  |  |  |  |
| 1/15/97 5:00 | 1,716.4 | 36 |  |  |  |  |  |
| 1/17/97 3:50 | 1,716.5 | 36 |  |  |  |  |  |
| 1/18/97 4:15 | 1,716.4 | 36 |  |  |  |  |  |
| 1/19/97 4:45 | 1,716.4 | 36 |  |  |  |  |  |
| 1/20/97 0:50 | 1,716.5 | 36 |  |  |  |  |  |
| 1/25/97 4:20 | 1,716.4 | 36 |  |  |  |  |  |
| 1/25/97 17:20 | 1,716.4 | 36 |  |  |  |  |  |
| 2/4/97 13:10 | 1,716.4 | 36 |  |  |  |  |  |
| 2/7/97 4:40 | 1,716.4 | 36 |  |  |  |  |  |
| 2/11/97 4:20 | 1,716.4 | 36 |  |  |  |  |  |
| 2/14/97 0:10 | 1,716.3 | 36 |  |  |  |  |  |
| 2/15/97 0:15 | 1,716.4 | 36 |  |  |  |  |  |
| 2/16/97 0:10 | 1,716.3 | 36 |  |  |  |  |  |
| 2/19/97 0:20 | 1,716.3 | 36 |  |  |  |  |  |
| 2/22/97 4:30 | 1,716.4 | 36 |  |  |  |  |  |
| 2/25/97 4:00 | 1,716.5 | 36 |  |  |  |  |  |
| 2/27/97 13:30 | 1,716.6 | 36 |  |  |  |  |  |
| 3/3/97 17:15 | 1,716.7 | 36 |  |  |  |  |  |
| 3/7/97 20:25 | 1,716.6 | 36 |  |  |  |  |  |
| 3/9/97 0:45 | 1,716.7 | 37 |  |  |  |  |  |
| 3/12/97 0:50 | 1,716.6 | 37 |  |  |  |  |  |
| 3/12/97 12:30 | 1,716.6 | 37 |  |  |  |  |  |
| 3/14/97 4:45 | 1,716.7 | 36 |  |  |  |  |  |
| 3/17/97 4:10 | 1,716.8 | 37 |  |  |  |  |  |
| 3/19/97 0:45 | 1,716.7 | 37 |  |  |  |  |  |
| 3/20/97 0:20 | 1,716.7 | 37 |  |  |  |  |  |
| 3/20/97 18:30 | 1,716.7 | 37 |  |  |  |  |  |
| 3/21/97 0:20 | 1,716.7 | 37 |  |  |  |  |  |
| 3/22/97 5:00 | 1,716.8 | 37 |  |  |  |  |  |
| 3/23/97 5:15 | 1,716.7 | 37 |  |  |  |  |  |
| 3/23/97 16:20 | 1,716.8 | 37 |  |  |  |  |  |
| 3/24/97 0:55 | 1,716.8 | 37 |  |  |  |  |  |
| 3/25/97 1:00 | 1,716.8 | 37 |  |  |  |  |  |
| 3/26/97 0:20 | 1,716.8 | 37 |  |  |  |  |  |
| 3/26/97 19:00 | 1,716.8 | 37 |  |  |  |  |  |
| 3/27/97 1:45 | 1,716.8 | 37 |  |  |  |  |  |
| 3/28/97 8:45 | 1,716.7 | 37 |  |  |  |  |  |
| 3/29/97 18:00 | 1,716.7 | 37 |  |  |  |  |  |
| 3/30/97 5:00 | 1,716.8 | 37 |  |  |  |  |  |
| 3/30/97 18:30 | 1,716.7 | 37 |  |  |  |  |  |
| 3/31/97 17:00 | 1,716.8 | 37 |  |  |  |  |  |
| 4/1/97 0:25 | 1,716.8 | 37 |  |  |  |  |  |
| 4/1/97 7:35 | 1,716.9 | 37 |  |  |  |  |  |
| 4/1/97 18:15 | 1,716.9 | 38 |  |  |  |  |  |
| 4/2/97 0:15 | 1,717.2 | 38 |  |  |  |  |  |
| 4/2/97 3:00 | 1,717.4 | 38 |  |  |  |  |  |
| 4/2/97 5:15 | 1,717.5 | 38 |  |  |  |  |  |
| 4/2/97 8:30 | 1,717.6 | 38 |  |  |  |  |  |
| 4/2/97 12:45 | 1,718.0 | 38 |  |  |  |  |  |
| 4/2/97 18:00 | 1,718.2 | 38 |  |  |  |  |  |
| 4/3/97 4:00 | 1,718.7 | 38 |  |  |  |  |  |
| 4/3/97 6:40 | 1,718.9 | 39 |  |  |  |  |  |
| 4/3/97 12:25 | 1,719.2 | 39 |  |  |  |  |  |
| 4/3/97 19:00 | 1,719.4 | 39 |  |  |  |  |  |
| 4/4/97 1:30 | 1,719.7 | 39 |  |  |  |  |  |
| 4/4/97 4:45 | 1,719.9 | 39 |  |  |  |  |  |
| 4/4/97 7:15 | 1,720.1 | 39 |  |  |  |  |  |
| 4/4/97 11:30 | 1,720.4 | 39 |  |  |  |  |  |
| 4/4/97 16:05 | 1,720.6 | 39 |  |  |  |  |  |
| 4/4/97 19:35 | 1,720.8 | 39 |  |  |  |  |  |
| 4/4/97 23:00 | 1,720.9 | 39 |  |  |  |  |  |
| 4/5/97 3:00 | 1,720.8 | 38 |  |  |  |  |  |
| 4/5/97 5:05 | 1,721.3 | 38 |  |  |  |  |  |
| 4/5/97 7:10 | 1,721.6 | 38 |  |  |  |  |  |
| 4/5/97 11:41 | 1,721.9 | 38 |  |  |  |  |  |
| 4/5/97 13:55 | 1,722.1 | 38 |  |  |  |  |  |
| 4/5/97 19:20 | 1,722.6 | 38 |  |  |  |  |  |
| 4/5/97 10:20 | 1,722.7 | 38 |  |  |  |  |  |
| 4/6/97 2:30 | 1,723.0 | 38 |  |  |  |  |  |
| 4/6/97 11:15 | 1,723.7 | 38 |  |  |  |  |  |
| 4/8/97 14:00 | 1,723.6 | 38 |  |  |  |  |  |
| 4/9/97 1:30 | 1,723.5 | 38 |  |  |  |  |  |
| 4/9/97 4:30 | 1,723.5 | 38 |  |  |  |  |  |
| 4/9/97 9:30 | 1,723.4 | 38 |  |  |  |  |  |
| 4/9/97 18:30 | 1,723.3 | 38 |  |  |  |  |  |
| 4/10/97 0:15 | 1,723.2 | 38 |  |  |  |  |  |
| 4/10/97 7:35 | 1,723.1 | 38 |  |  |  |  |  |
| 4/10/97 6:30 | 1,723.1 | 38 |  |  |  |  |  |
| 4/10/97 9:30 | 1,723.0 | 38 |  |  |  |  |  |
| 4/10/97 13:00 | 1,723.0 | 38 |  |  |  |  |  |
| 4/11/97 5:30 | 1,722.7 | 38 |  |  |  |  |  |
| 4/11/97 13:00 | 1,722.6 | 38 |  |  |  |  |  |
| 4/11/97 20:20 | 1,722.5 | 38 |  |  |  |  |  |
| 4/12/97 0:01 | 1,722.4 | 38 |  |  |  |  |  |
| 4/12/97 13:15 | 1,722.3 | 38 |  |  |  |  |  |
| 4/13/97 1:20 | 1,722.0 | 38 |  |  |  |  |  |
| 4/13/97 1:20 | 1,721.8 | 38 |  |  |  |  |  |
| 4/13/97 10:40 | 1,721.7 | 38 |  |  |  |  |  |
| 4/14/97 4:00 | 1,721.6 | 38 |  |  |  |  |  |
| 4/14/97 16:10 | 1,721.4 | 38 |  |  |  |  |  |
| 4/14/97 23:30 | 1,721.4 | 38 |  |  |  |  |  |
| 4/15/97 6:40 | 1,721.3 | 38 |  |  |  |  |  |
| 4/15/97 11:20 | 1,721.3 | 38 |  |  |  |  |  |
| 4/15/97 17:15 | 1,721.5 | 38 |  |  |  |  |  |
| 4/16/97 3:20 | 1,721.4 | 38 |  |  |  |  |  |
| 4/16/97 17:30 | 1,721.4 | 38 |  |  |  |  |  |
| 4/17/97 8:00 | 1,721.3 | 37 |  |  |  |  |  |
| 4/18/97 1:00 | 1,721.0 | 37 |  |  |  |  |  |
| 4/19/97 7:50 | 1,720.9 | 38 |  |  |  |  |  |
| 4/20/97 12:00 | 1,720.8 | 38 |  |  |  |  |  |
| 4/21/97 0:01 | 1,720.8 | 38 |  |  |  |  |  |
| 4/22/97 13:30 | 1,720.8 | 38 |  |  |  |  |  |
| 4/22/97 0:10 | 1,720.8 | 38 |  |  |  |  |  |
| 4/23/97 17:30 | 1,720.8 | 40 |  |  |  |  |  |
| 4/23/97 4:00 | 1,720.6 | 39 |  |  |  |  |  |
| 5/2/97 4:45 | 1,720.1 | 44 |  |  |  |  |  |
| 5/3/97 4:45 | 1,720.0 | 43 |  |  |  |  |  |
| 5/4/97 4:40 | 1,720.0 | 45 |  |  |  |  |  |
| 5/4/97 16:15 | 1,719.8 | 46 |  |  |  |  |  |
| 5/5/97 4:40 | 1,719.8 | 46 |  |  |  |  |  |
| 5/5/97 6:40 | 1,719.8 | 46 |  |  |  |  |  |
| 5/7/97 0:15 | 1,719.7 | 50 |  |  |  |  |  |
| 5/9/97 4:15 | 1,719.6 | 48 |  |  |  |  |  |
| 5/10/97 4:00 | 1,719.6 | 49 |  |  |  |  |  |
| 5/12/97 0:50 | 1,719.4 | 51 |  |  |  |  |  |
| 5/12/97 20:45 | 1,719.3 | 50 |  |  |  |  |  |
| 5/16/97 5:00 | 1,719.1 | 50 |  |  |  |  |  |
| 5/17/97 0:45 | 1,718.9 | 53 |  |  |  |  |  |
| 5/18/97 2:25 | 1,718.9 | 53 |  |  |  |  |  |
| 5/19/97 18:25 | 1,718.8 | 50 |  |  |  |  |  |
| 5/21/97 0:40 | 1,718.7 | 52 |  |  |  |  |  |
| 5/23/97 8:30 | 1,718.6 | 54 |  |  |  |  |  |
| 5/24/97 4:00 | 1,718.5 | 55 |  |  |  |  |  |
| 5/25/97 11:50 | 1,718.5 | 55 |  |  |  |  |  |
| 5/26/97 12:10 | 1,718.4 | 54 |  |  |  |  |  |
| 5/28/97 1:00 | 1,718.4 | 54 |  |  |  |  |  |
| 5/30/97 1:05 | 1,718.4 | 54 |  |  |  |  |  |
| 6/2/97 0:30 | 1,718.3 | 59 |  |  |  |  |  |
| 6/3/97 0:25 | 1,718.3 | 60 |  |  |  |  |  |
| 6/6/97 4:20 | 1,718.3 | 64 |  |  |  |  |  |
| 6/9/97 0:40 | 1,718.3 | 66 |  |  |  |  |  |
| 6/11/97 11:30 | 1,718.2 | 65 |  |  |  |  |  |
| 6/14/97 1:00 | 1,718.1 | 68 |  |  |  |  |  |
| 6/18/97 1:00 | 1,717.9 | 68 |  |  |  |  |  |
| 6/20/97 1:15 | 1,717.9 | 68 |  |  |  |  |  |
| 6/20/97 16:00 | 1,717.9 | 68 |  |  |  |  |  |
| 6/22/97 1:05 | 1,717.9 | 69 |  |  |  |  |  |
| 6/23/97 12:15 | 1,717.9 | 70 |  |  |  |  |  |
| 6/26/97 12:20 | 1,717.9 | 71 |  |  |  |  |  |
| 6/28/97 4:30 | 1,717.8 | 73 |  |  |  |  |  |
| 7/5/97 4:20 | 1,717.8 | 64 |  |  |  |  |  |
| 7/6/97 0:40 | 1,717.8 | 68 |  |  |  |  |  |
| 7/7/97 0:30 | 1,717.6 | 64 |  |  |  |  |  |
| 7/10/97 0:01 | 1,717.6 | 66 |  |  |  |  |  |
| 7/14/97 0:30 | 1,717.7 | 74 |  |  |  |  |  |
| 7/18/97 4:45 | 1,717.6 | 73 |  |  |  |  |  |
| 7/21/97 0:15 | 1,717.6 | 76 |  |  |  |  |  |
| 7/25/97 12:45 | 1,717.5 | 74 |  |  |  |  |  |
| 7/28/97 0:35 | 1,717.6 | 76 |  |  |  |  |  |
| 7/28/97 16:30 | 1,717.8 | 75 |  |  |  |  |  |
| 7/29/97 0:20 | 1,717.8 | 75 |  |  |  |  |  |
| 7/29/97 12:25 | 1,717.9 | 74 |  |  |  |  |  |
| 7/30/97 1:15 | 1,717.9 | 73 |  |  |  |  |  |
| 7/31/97 0:05 | 1,717.8 | 73 |  |  |  |  |  |
| 7/31/97 12:15 | 1,717.7 | 71 |  |  |  |  |  |
| 8/1/97 0:50 | 1,717.8 | 71 |  |  |  |  |  |
| 8/4/97 4:00 | 1,717.8 | 74 |  |  |  |  |  |
| 8/6/97 20:15 | 1,717.7 | 74 |  |  |  |  |  |
| 8/11/97 7:40 | 1,717.6 | 69 |  |  |  |  |  |
| 8/14/97 0:30 | 1,717.5 | 68 |  |  |  |  |  |
| 8/15/97 1:35 | 1,717.5 | 68 |  |  |  |  |  |
| 8/16/97 1:20 | 1,717.5 | 68 |  |  |  |  |  |
| 8/19/97 1:30 |  | 66 |  |  |  |  |  |
| 8/21/97 4:15 | 1,717.5 | 66 |  |  |  |  |  |
| 8/23/97 4:30 | 1,717.5 | 68 |  |  |  |  |  |
| 8/24/97 11:45 | 1,717.6 | 68 |  |  |  |  |  |
| 8/26/97 0:30 | 1,717.5 | 72 |  |  |  |  |  |
| 8/31/97 3:30 | 1,717.5 | 73 |  |  |  |  |  |
| 9/2/97 18:45 | 1,717.4 | 71 |  |  |  |  |  |
| 9/4/97 0:01 | 1,717.3 | 69 |  |  |  |  |  |
| 9/7/97 0:40 | 1,717.4 | 68 |  |  |  |  |  |
| 9/13/97 0:01 | 1,717.3 | 66 |  |  |  |  |  |
| 9/15/97 0:01 | 1,717.3 | 67 |  |  |  |  |  |
| 9/19/97 23:30 | 1,717.4 | 66 |  |  |  |  |  |
| 9/22/97 0:10 | 1,717.3 | 65 |  |  |  |  |  |
| 9/25/97 23:15 | 1,717.3 | 64 |  |  |  |  |  |
| 9/28/97 0:10 | 1,717.3 | 63 |  |  |  |  |  |
| 9/30/97 9:30 | 1,717.2 | 58 |  |  |  |  |  |
| 10/3/97 0:20 | 1,717.2 | 58 |  |  |  |  |  |
| 10/6/97 0:45 | 1,717.2 | 60 |  |  |  |  |  |
| 10/10/97 9:30 | 1,717.1 | 58 |  |  |  |  |  |
| 10/13/97 1:00 | 1,717.4 |  |  |  |  |  |  |
| 10/15/97 9:30 | 1,717.2 | 51 |  |  |  |  |  |
| 10/18/97 9:10 | 1,717.3 | 51 |  |  |  |  |  |
| 10/26/97 11:00 | 1,717.3 | 40 |  |  |  |  |  |
| 11/1/97 3:00 | 1,717.3 | 40 |  |  |  |  |  |
| 11/7/97 11:30 | 1,717.3 | 36 |  |  |  |  |  |
| 11/16/97 8:00 | 1,717.4 | 33 |  |  |  |  |  |
| 12/22/97 9:30 | 1,717.5 | 36 |  |  |  |  |  |
| 2/20/98 0:03 | 1,717.5 |  |  |  |  |  |  |
| 2/23/98 0:45 | 1,717.6 |  |  |  |  |  |  |
| 2/24/98 3:00 | 1,717.7 |  |  |  |  |  |  |
| 2/25/98 0:05 | 1,717.8 |  |  |  |  |  |  |
| 2/26/98 0:05 | 1,718.0 |  |  |  |  |  |  |
| 2/27/98 0:07 | 1,718.2 |  |  |  |  |  |  |
| 2/28/98 0:01 | 1,718.5 |  |  |  |  |  |  |
| 3/1/98 0:03 | 1,718.6 |  |  |  |  |  |  |
| 3/2/98 0:03 | 1,718.6 |  |  |  |  |  |  |
| 3/3/98 0:03 | 1,718.6 |  |  |  |  |  |  |
| 3/4/98 0:03 | 1,718.6 |  |  |  |  |  |  |
| 3/6/98 1:15 | 1,718.6 |  |  |  |  |  |  |
| 3/6/98 0:03 | 1,718.6 |  |  |  |  |  |  |
| 3/7/98 0:01 | 1,718.6 |  |  |  |  |  |  |
| 3/7/98 8:50 | 1,718.6 |  |  |  |  |  |  |
| 3/8/98 10:45 | 1,718.6 |  |  |  |  |  |  |
| 3/13/98 23:05 | 1,718.4 |  |  |  |  |  |  |
| 3/17/98 0:05 | 1,718.2 |  |  |  |  |  |  |
| 3/22/98 0:03 | 1,718.1 |  |  |  |  |  |  |
| 3/30/98 0:45 | 1,718.6 | 41 |  |  |  |  |  |
| 4/1/98 9:00 | 1,718.7 | 41 |  |  |  |  |  |
| 4/2/98 7:03 | 1,718.7 | 39 |  |  |  |  |  |
| 4/3/98 9:04 | 1,718.7 | 39 |  |  |  |  |  |
| 4/4/98 0:45 | 1,718.7 | 40 |  |  |  |  |  |
| 4/5/98 17:00 | 1,718.6 | 38 |  |  |  |  |  |
| 4/6/98 11:05 | 1,718.6 | 39 |  |  |  |  |  |
| 4/8/98 4:35 | 1,718.6 | 39 |  |  |  |  |  |
| 4/9/98 0:05 | 1,719.0 | 38 |  |  |  |  |  |
| 4/11/98 11:20 | 1,719.1 | 39 |  |  |  |  |  |
| 4/11/98 17:15 | 1,719.2 |  |  |  |  |  |  |
| 4/13/98 3:05 | 1,719.1 | 45 |  |  |  |  |  |
| 4/13/98 12:08 | 1,719.1 | 45 |  |  |  |  |  |
| 4/14/98 0:02 | 1,719.1 | 47 |  |  |  |  |  |
| 4/14/98 21:00 | 1,719.1 | 48 |  |  |  |  |  |
| 4/15/98 12:01 | 1,719.0 | 46 |  |  |  |  |  |
| 4/19/98 4:00 | 1,718.6 | 47 |  |  |  |  |  |
| 4/26/98 4:03 | 1,718.6 | 47 |  |  |  |  |  |
| 4/21/98 20:00 | 1,718.5 | 50 |  |  |  |  |  |
| 4/24/98 4:03 | 1,718.5 | 49 |  |  |  |  |  |
| 4/25/98 4:30 | 1,718.4 | 53 |  |  |  |  |  |
| 4/26/98 16:02 | 1,718.5 | 52 |  |  |  |  |  |
| 4/27/98 1:00 | 1,718.5 | 52 |  |  |  |  |  |
| 4/28/98 10:03 | 1,718.5 | 52 |  |  |  |  |  |
| 4/29/98 0:45 | 1,718.5 | 52 |  |  |  |  |  |
| 4/30/98 4:35 | 1,718.5 | 52 |  |  |  |  |  |
| 5/1/98 4:20 | 1,718.5 | 53 |  |  |  |  |  |
| 5/3/98 4:45 | 1,718.5 | 55 |  |  |  |  |  |
| 5/3/98 15:00 | 1,718.3 | 56 |  |  |  |  |  |
| 5/5/98 1:00 | 1,718.3 | 59 |  |  |  |  |  |
| 5/9/98 0:05 | 1,718.4 | 57 |  |  |  |  |  |
| 5/10/98 4:00 | 1,718.2 | 57 |  |  |  |  |  |
| 5/10/98 12:00 | 1,718.2 | 57 |  |  |  |  |  |
| 5/12/98 4:03 | 1,718.3 | 58 |  |  |  |  |  |
| 5/14/98 0:45 | 1,718.5 | 59 |  |  |  |  |  |
| 5/14/98 20:15 | 1,718.4 | 62 |  |  |  |  |  |
| 5/15/98 0:40 | 1,718.4 | 62 |  |  |  |  |  |
| 5/16/98 4:02 | 1,718.5 | 61 |  |  |  |  |  |
| 5/16/98 13:00 | 1,718.5 | 61 |  |  |  |  |  |
| 5/17/98 5:00 | 1,718.5 | 62 |  |  |  |  |  |
| 5/18/98 1:03 | 1,718.7 | 63 |  |  |  |  |  |
| 5/19/98 2:00 | 1,718.8 | 66 |  |  |  |  |  |
| 5/20/98 1:04 | 1,718.7 | 66 |  |  |  |  |  |
| 5/20/98 16:01 | 1,718.7 | 65 |  |  |  |  |  |
| 5/22/98 11:05 | 1,718.6 | 63 |  |  |  |  |  |
| 5/23/98 17:00 | 1,718.6 | 62 |  |  |  |  |  |
| 5/25/98 4:02 | 1,718.5 | 60 |  |  |  |  |  |
| 5/27/98 1:15 | 1,718.5 | 61 |  |  |  |  |  |
| 5/27/98 7:00 | 1,718.5 | 60 |  |  |  |  |  |
| 5/27/98 4:00 | 1,718.5 | 67 |  |  |  |  |  |
| 5/29/98 19:00 | 1,718.4 | 67 |  |  |  |  |  |
| 5/29/98 1:00 | 1,718.4 | 66 |  |  |  |  |  |
| 6/2/98 5:00 | 1,718.3 | 64 |  |  |  |  |  |
| 6/3/98 18:00 | 1,718.2 | 61 |  |  |  |  |  |
| 6/4/98 0:03 | 1,718.2 | 61 |  |  |  |  |  |
| 6/5/98 0:15 | 1,718.2 | 60 |  |  |  |  |  |
| 6/6/98 10:15 | 1,718.1 | 60 |  |  |  |  |  |
| 6/8/98 16:00 | 1,718.1 | 58 |  |  |  |  |  |
| 6/9/98 21:00 | 1,718.1 | 58 |  |  |  |  |  |
| 6/13/98 0:45 | 1,718.1 | 63 |  |  |  |  |  |
| 6/15/98 6:55 | 1,718.1 | 63 |  |  |  |  |  |
| 6/16/98 7:00 | 1,718.2 | 63 |  |  |  |  |  |
| 6/18/98 4:45 | 1,718.1 | 64 |  |  |  |  |  |
| 6/18/98 16:00 | 1,718.1 | 65 |  |  |  |  |  |
| 6/19/98 0:45 | 1,718.1 | 66 |  |  |  |  |  |
| 6/24/98 0:03 | 1,718.1 | 69 |  |  |  |  |  |
| 6/26/98 0:03 | 1,718.1 | 68 |  |  |  |  |  |
| 6/27/98 4:45 | 1,717.9 | 70 |  |  |  |  |  |
| 7/1/98 16:03 | 1,718.1 | 73 |  |  |  |  |  |
| 7/6/98 3:55 | 1,718.1 | 72 |  |  |  |  |  |
| 7/6/98 20:01 | 1,718.1 | 72 |  |  |  |  |  |
| 7/8/98 0:01 | 1,718.2 | 74 |  |  |  |  |  |
| 7/10/98 10:04 | 1,718.0 | 73 |  |  |  |  |  |
| 7/11/98 12:02 | 1,718.0 | 72 |  |  |  |  |  |
| 7/11/98 21:00 | 1,718.0 | 74 |  |  |  |  |  |
| 7/19/98 8:00 | 1,717.8 | 76 |  |  |  |  |  |
| 7/20/98 0:01 | 1,717.8 | 76 |  |  |  |  |  |
| 7/24/98 4:01 | 1,717.7 | 75 |  |  |  |  |  |
| 7/27/98 7:00 | 1,717.7 | 75 |  |  |  |  |  |
| 7/31/98 0:01 | 1,717.6 | 72 |  |  |  |  |  |
| 8/2/98 20:00 | 1,717.5 | 72 |  |  |  |  |  |
| 8/4/98 4:00 | 1,717.7 | 69 |  |  |  |  |  |
| 8/4/98 12:45 | 1,717.6 | 70 |  |  |  |  |  |
| 8/5/98 12:01 | 1,717.6 | 70 |  |  |  |  |  |
| 8/7/98 4:00 | 1,717.6 | 70 |  |  |  |  |  |
| 8/10/98 4:03 | 1,717.7 | 73 |  |  |  |  |  |
| 8/13/98 13:03 | 1,717.5 | 75 |  |  |  |  |  |
| 8/17/98 7:01 | 1,717.5 | 73 |  |  |  |  |  |
| 8/19/98 4:02 | 1,717.4 | 74 |  |  |  |  |  |
| 8/22/98 16:15 | 1,717.5 | 74 |  |  |  |  |  |
| 8/24/98 23:55 | 1,717.5 | 76 |  |  |  |  |  |
| 8/29/98 0:05 | 1,717.3 | 75 |  |  |  |  |  |
| 9/1/98 7:00 | 1,717.4 | 72 |  |  |  |  |  |
| 9/4/98 20:00 | 1,717.3 | 71 |  |  |  |  |  |
| 9/8/98 17:05 | 1,717.2 | 69 |  |  |  |  |  |
| 9/12/98 19:03 | 1,717.2 | 69 |  |  |  |  |  |
| 9/14/98 1:00 | 1,717.1 | 70 |  |  |  |  |  |
| 9/15/98 4:03 | 1,717.1 | 70 |  |  |  |  |  |
| 9/17/98 1:00 | 1,717.1 | 60 |  |  |  |  |  |
| 9/21/98 0:03 | 1,717.0 | 65 |  |  |  |  |  |
| 9/27/98 15:03 | 1,717.0 | 62 |  |  |  |  |  |
| 9/29/98 4:03 | 1,717.0 | 55 |  |  |  |  |  |
| 10/3/98 4:03 | 1,716.9 | 55 |  |  |  |  |  |
| 10/4/98 12:03 | 1,716.9 | 54 |  |  |  |  |  |
| 10/5/98 20:05 | 1,717.1 | 55 |  |  |  |  |  |
| 10/6/98 0:03 | 1,717.0 | 53 |  |  |  |  |  |
| 10/6/98 12:03 | 1,717.0 | 53 |  |  |  |  |  |
| 10/7/98 3:05 | 1,717.0 | 52 |  |  |  |  |  |
| 10/9/98 4:02 | 1,717.1 | 52 |  |  |  |  |  |
| 10/10/98 1:25 | 1,717.1 | 54 |  |  |  |  |  |
| 10/13/98 12:20 | 1,717.0 | 50 |  |  |  |  |  |
| 10/15/98 4:00 | 1,717.0 | 50 |  |  |  |  |  |
| 10/15/98 20:00 | 1,717.0 | 50 |  |  |  |  |  |
| 10/16/98 7:00 | 1,717.0 | 51 |  |  |  |  |  |
| 10/17/98 4:35 | 1,717.1 | 51 |  |  |  |  |  |
| 10/18/98 18:45 | 1,717.7 | 48 |  |  |  |  |  |
| 10/19/98 16:00 | 1,717.8 | 48 |  |  |  |  |  |
| 10/20/98 5:00 | 1,717.9 | 47 |  |  |  |  |  |
| 10/22/98 4:03 | 1,717.9 | 48 |  |  |  |  |  |
| 10/23/98 3:35 | 1,718.0 | 48 |  |  |  |  |  |
| 10/23/98 1:02 | 1,718.1 | 48 |  |  |  |  |  |
| 10/24/98 19:55 | 1,718.0 | 50 |  |  |  |  |  |
| 10/25/98 20:05 | 1,717.9 | 50 |  |  |  |  |  |
| 10/26/98 23:45 | 1,718.1 | 51 |  |  |  |  |  |
| 10/29/98 3:03 | 1,718.1 | 51 |  |  |  |  |  |
| 10/31/98 0:03 | 1,718.0 | 50 |  |  |  |  |  |
| 11/1/98 6:40 | 1,718.1 | 49 |  |  |  |  |  |
| 11/3/98 18:00 | 1,718.0 | 45 |  |  |  |  |  |
| 11/7/98 16:00 | 1,718.0 | 40 |  |  |  |  |  |
| 11/13/98 12:03 | 1,718.1 | 35 |  |  |  |  |  |
| 11/14/98 4:05 | 1,718.1 | 35 |  |  |  |  |  |
| 11/16/98 12:03 | 1,718.1 | 35 |  |  |  |  |  |
| 11/20/98 0:15 | 1,718.1 | 36 |  |  |  |  |  |
| 11/28/98 4:00 | 1,718.0 | 35 |  |  |  |  |  |
| 12/1/98 18:00 | 1,718.2 | 35 |  |  |  |  |  |
| 12/4/98 19:00 | 1,718.1 | 37 |  |  |  |  |  |
| 12/6/98 19:00 | 1,718.1 | 36 |  |  |  |  |  |
| 12/11/98 1:03 | 1,718.0 | 33 |  |  |  |  |  |
| 12/15/98 2:02 | 1,718.0 | 34 |  |  |  |  |  |
| 12/17/98 2:45 | 1,718.0 | 33 |  |  |  |  |  |
| 12/20/98 9:30 | 1,718.0 | 32 |  |  |  |  |  |
| 12/28/98 4:02 | 1,717.9 | 33 |  |  |  |  |  |
| 1/1/99 3:55 | 1,717.9 | 33 |  |  |  |  |  |
| 1/4/99 4:03 | 1,717.9 | 33 |  |  |  |  |  |
| 1/7/99 16:02 | 1,717.8 | 33 |  |  |  |  |  |
| 1/12/99 6:03 | 1,717.8 | 33 |  |  |  |  |  |
| 1/17/99 5:04 | 1,717.7 | 34 |  |  |  |  |  |
| 1/19/99 16:22 | 1,718.0 | 34 |  |  |  |  |  |
| 1/21/99 17:16 | 1,718.0 | 34 |  |  |  |  |  |
| 1/26/99 4:01 | 1,717.6 | 34 |  |  |  |  |  |
| 1/29/99 4:15 | 1,717.6 | 34 |  |  |  |  |  |
| 2/4/99 4:02 | 1,717.6 | 34 |  |  |  |  |  |
| 2/6/99 2:05 | 1,717.7 | 34 |  |  |  |  |  |
| 2/8/99 4:45 | 1,717.6 | 34 |  |  |  |  |  |
| 2/9/99 3:03 | 1,717.6 | 34 |  |  |  |  |  |
| 2/10/99 2:03 | 1,717.6 | 34 |  |  |  |  |  |
| 2/13/99 16:00 | 1,717.6 | 35 |  |  |  |  |  |
| 2/17/99 16:01 | 1,717.6 | 36 |  |  |  |  |  |
| 2/18/99 16:15 | 1,717.6 | 36 |  |  |  |  |  |
| 2/19/99 0:03 | 1,717.6 | 36 |  |  |  |  |  |
| 2/20/99 16:25 | 1,717.6 | 37 |  |  |  |  |  |
| 2/23/99 0:05 | 1,717.6 | 37 |  |  |  |  |  |
| 2/24/99 16:03 | 1,717.7 | 37 |  |  |  |  |  |
| 2/26/99 4:15 | 1,717.6 | 38 |  |  |  |  |  |
| 2/27/99 4:15 | 1,717.6 | 38 |  |  |  |  |  |
| 3/2/99 2:15 | 1,717.8 | 38 |  |  |  |  |  |
| 3/4/99 15:15 | 1,717.9 | 39 |  |  |  |  |  |
| 3/9/99 3:05 | 1,718.0 | 39 |  |  |  |  |  |
| 3/11/99 3:35 | 1,717.9 | 39 |  |  |  |  |  |
| 3/15/99 3:03 | 1,717.9 | 39 |  |  |  |  |  |
| 3/19/99 0:05 | 1,718.0 | 41 |  |  |  |  |  |
| 3/20/99 0:04 | 1,718.0 | 41 |  |  |  |  |  |
| 3/20/99 17:04 | 1,718.0 | 41 |  |  |  |  |  |
| 3/21/99 17:03 | 1,718.0 | 42 |  |  |  |  |  |
| 3/25/99 0:45 | 1,718.0 | 42 |  |  |  |  |  |
| 3/27/99 1:05 | 1,718.0 | 43 |  |  |  |  |  |
| 3/27/99 20:01 | 1,718.0 | 42 |  |  |  |  |  |
| 3/30/99 0:55 | 1,718.0 | 39 |  |  |  |  |  |
| 3/30/99 19:05 | 1,717.8 | 40 |  |  |  |  |  |
| 4/2/99 18:05 | 1,718.0 | 41 |  |  |  |  |  |
| 4/3/99 3:02 | 1,718.0 | 41 |  |  |  |  |  |
| 4/4/99 5:25 | 1,718.1 | 41 |  |  |  |  |  |
| 4/5/99 5:15 | 1,718.1 | 42 |  |  |  |  |  |
| 4/6/99 4:24 | 1,718.2 | 42 |  |  |  |  |  |
| 4/8/99 0:35 | 1,718.2 | 45 |  |  |  |  |  |
| 4/12/99 4:00 | 1,718.3 | 44 |  |  |  |  |  |
| 4/13/99 18:01 | 1,718.2 | 46 |  |  |  |  |  |
| 4/14/99 19:04 | 1,718.3 | 46 |  |  |  |  |  |
| 4/16/99 7:02 | 1,718.1 | 46 |  |  |  |  |  |
| 4/25/99 0:02 | 1,718.1 | 48 |  |  |  |  |  |
| 4/27/99 3:03 | 1,718.2 | 48 |  |  |  |  |  |
| 4/28/99 3:05 | 1,718.2 | 49 |  |  |  |  |  |
| 4/29/99 4:04 | 1,718.1 | 51 |  |  |  |  |  |
| 4/29/99 19:00 | 1,718.1 | 52 |  |  |  |  |  |
| 5/1/99 4:03 | 1,718.1 | 54 |  |  |  |  |  |
| 5/1/99 19:25 | 1,718.1 | 55 |  |  |  |  |  |
| 5/2/99 5:01 | 1,718.1 | 54 |  |  |  |  |  |
| 5/3/99 4:35 | 1,718.0 | 54 |  |  |  |  |  |
| 5/5/99 2:00 | 1,718.0 | 57 |  |  |  |  |  |
| 5/8/99 13:00 | 1,718.0 | 52 |  |  |  |  |  |
| 5/12/99 4:01 | 1,718.2 | 55 |  |  |  |  |  |
| 5/12/99 16:05 | 1,718.1 | 56 |  |  |  |  |  |
| 5/15/99 19:03 | 1,718.2 | 58 |  |  |  |  |  |
| 5/16/99 20:03 | 1,718.2 | 59 |  |  |  |  |  |
| 5/18/99 20:00 | 1,718.2 | 56 |  |  |  |  |  |
| 5/25/99 4:25 | 1,718.1 | 59 |  |  |  |  |  |
| 5/29/99 1:23 | 1,718.0 | 66 |  |  |  |  |  |
| 6/7/99 4:15 | 1,717.9 | 67 |  |  |  |  |  |
| 6/10/99 4:01 | 1,718.1 | 71 |  |  |  |  |  |
| 6/13/99 4:04 | 1,718.0 | 72 |  |  |  |  |  |
| 6/20/99 20:03 | 1,717.8 | 64 |  |  |  |  |  |
| 6/22/99 4:15 | 1,717.8 | 67 |  |  |  |  |  |
| 6/26/99 6:05 | 1,717.9 | 72 |  |  |  |  |  |
| 6/27/99 4:01 | 1,717.8 | 71 |  |  |  |  |  |
| 6/29/99 0:04 | 1,717.8 | 69 |  |  |  |  |  |
| 7/2/99 4:15 | 1,717.9 | 69 |  |  |  |  |  |
| 7/11/99 19:03 | 1,717.6 | 72 |  |  |  |  |  |
| 7/12/99 20:00 | 1,717.6 | 74 |  |  |  |  |  |
| 7/15/99 1:00 | 1,717.6 | 73 |  |  |  |  |  |
| 7/18/99 1:00 | 1,717.6 | 73 |  |  |  |  |  |
| 7/20/99 0:03 | 1,717.6 | 72 |  |  |  |  |  |
| 7/23/99 1:03 | 1,717.6 | 76 |  |  |  |  |  |
| 7/25/99 1:15 | 1,717.5 | 75 |  |  |  |  |  |
| 7/26/99 4:03 | 1,717.4 | 80 |  |  |  |  |  |
| 7/31/99 3:05 | 1,717.4 | 78 |  |  |  |  |  |
| 8/3/99 16:03 | 1,717.3 | 75 |  |  |  |  |  |
| 8/4/99 18:03 | 1,717.2 | 76 |  |  |  |  |  |
| 8/6/99 20:45 | 1,717.2 | 75 |  |  |  |  |  |
| 8/7/99 20:45 | 1,717.2 | 75 |  |  |  |  |  |
| 8/13/99 19:03 | 1,717.1 | 72 |  |  |  |  |  |
| 8/22/99 20:00 | 1,717.0 | 73 |  |  |  |  |  |
| 8/24/99 0:18 | 1,716.9 | 71 |  |  |  |  |  |
| 8/29/99 20:00 | 1,716.9 | 71 |  |  |  |  |  |
| 9/1/99 19:01 | 1,716.9 | 71 |  |  |  |  |  |
| 9/3/99 12:45 | 1,716.9 | 71 |  |  |  |  |  |
| 9/3/99 12:45 | 1,716.9 | 70 |  |  |  |  |  |
| 9/4/99 12:00 | 1,717.1 | 70 |  |  |  |  |  |
| 9/4/99 17:15 | 1,717.1 | 70 |  |  |  |  |  |
| 9/5/99 12:04 | 1,717.1 | 67 |  |  |  |  |  |
| 9/5/99 16:03 | 1,716.9 | 68 |  |  |  |  |  |
| 9/6/99 4:03 | 1,717.0 | 67 |  |  |  |  |  |
| 9/8/99 18:03 | 1,717.1 | 65 |  |  |  |  |  |
| 9/9/99 7:45 | 1,717.0 | 64 |  |  |  |  |  |
| 9/11/99 4:00 | 1,716.9 | 62 |  |  |  |  |  |
| 9/11/99 12:00 | 1,716.9 | 62 |  |  |  |  |  |
| 9/12/99 5:00 | 1,717.0 | 62 |  |  |  |  |  |
| 9/15/99 0:45 | 1,716.9 | 58 |  |  |  |  |  |
| 9/16/99 0:45 | 1,716.9 | 58 |  |  |  |  |  |
| 9/17/99 0:00 | 1,716.9 | 59 |  |  |  |  |  |
| 9/18/99 0:15 | 1,716.9 | 59 |  |  |  |  |  |
| 9/20/99 0:15 | 1,716.8 | 61 |  |  |  |  |  |
| 9/21/99 0:00 | 1,716.8 | 61 |  |  |  |  |  |
| 9/22/99 0:15 | 1,716.8 | 59 |  |  |  |  |  |
| 9/23/99 0:45 | 1,716.8 | 58 |  |  |  |  |  |
| 9/26/99 4:00 | 1,716.8 | 59 |  |  |  |  |  |
| 9/29/99 19:55 | 1,716.8 | 58 |  |  |  |  |  |
| 10/1/99 4:00 | 1,716.7 | 54 |  |  |  |  |  |
| 10/1/99 20:15 | 1,716.7 | 52 |  |  |  |  |  |
| 10/14/99 0:45 | 1,716.6 | 51 |  |  |  |  |  |
| 10/16/99 1:00 | 1,716.7 | 51 |  |  |  |  |  |
| 10/17/99 3:00 | 1,716.7 | 49 |  |  |  |  |  |
| 10/21/99 19:04 | 1,716.7 | 47 |  |  |  |  |  |
| 10/23/99 23:03 | 1,716.6 | 45 |  |  |  |  |  |
| 10/25/99 23:45 | 1,716.7 | 44 |  |  |  |  |  |
| 10/27/99 21:15 | 1,716.6 | 45 |  |  |  |  |  |
| 10/30/99 16:03 | 1,716.6 | 45 |  |  |  |  |  |
| 10/31/99 16:03 | 1,716.6 | 45 |  |  |  |  |  |
| 11/4/99 0:01 | 1,716.5 | 42 |  |  |  |  |  |
| 11/5/99 11:02 | 1,716.5 | 41 |  |  |  |  |  |
| 11/8/99 7:15 | 1,716.5 | 41 |  |  |  |  |  |
| 11/10/99 12:55 | 1,716.5 | 45 |  |  |  |  |  |
| 11/12/99 3:03 | 1,716.5 | 45 |  |  |  |  |  |
| 11/14/99 4:01 | 1,716.5 | 45 |  |  |  |  |  |
| 11/15/99 4:15 | 1,716.4 | 45 |  |  |  |  |  |
| 11/15/99 16:15 | 1,716.5 | 43 |  |  |  |  |  |
| 11/20/99 4:02 | 1,716.5 | 39 |  |  |  |  |  |
| 11/23/99 20:01 | 1,716.5 | 38 |  |  |  |  |  |
| 11/25/99 4:00 | 1,716.6 | 37 |  |  |  |  |  |
| 11/25/99 16:30 | 1,716.5 | 36 |  |  |  |  |  |
| 11/26/99 0:00 | 1,716.5 | 37 |  |  |  |  |  |
| 11/26/99 20:00 | 1,716.5 | 36 |  |  |  |  |  |
| 12/4/99 3:45 | 1,716.5 | 36 |  |  |  |  |  |
| 12/5/99 20:00 | 1,716.5 | 35 |  |  |  |  |  |
| 12/8/99 13:05 | 1,716.4 | 37 |  |  |  |  |  |
| 12/10/99 3:03 | 1,716.5 | 36 |  |  |  |  |  |
| 12/12/99 1:00 | 1,716.4 | 37 |  |  |  |  |  |
| 12/13/99 3:45 | 1,716.5 | 37 |  |  |  |  |  |
| 12/15/99 3:04 | 1,716.4 | 36 |  |  |  |  |  |
| 12/17/99 4:03 | 1,716.5 | 32 |  |  |  |  |  |
| 12/23/99 15:45 | 1,716.5 | 33 |  |  |  |  |  |
| 12/26/99 20:15 | 1,716.4 | 36 |  |  |  |  |  |
| 12/27/99 20:55 | 1,716.5 | 36 |  |  |  |  |  |
| 12/28/99 16:00 | 1,716.4 | 34 |  |  |  |  |  |
| 12/29/99 20:00 | 1,716.5 | 35 |  |  |  |  |  |

References

Albrook, P.J., 1975, Electricity, Water, Gas, Steam Heat in Watertown, South Dakota:		139p.

Barari, Assad, 1971, Hydrology of Lake Kampeska: South Dakota Geological Survey

Report of Investigations No. 103, 84 p.

J. T. Banner & Associates, Inc., 1965, Water Supply Facilities and Proposed 			Improvements for city of Watertown, South Dakota, three volumes.

Little, J. R., 1998, Upper Big Sioux River Hydrology: Lake Kampeska Water Project		 District, 25 p.

Madison, K. R., 1994, Diagnostic Feasibility Study, Lake Kampeska, Codington County,		South Dakota: South Dakota Department of Environment and Natural Resources,		107p.

Spuhler, W., Lytle, W. F., and Moe, D., 1971, Climate of South Dakota: South Dakota		 State University, Agriculture Experiment Station, Bulletin 582, 30 p.

U.S. Army Corps of Engineers, Omaha District, Missouri River Division, 1994, Flood		 Control for Watertown and Vicinity, South Dakota, three volumes.

U.S. Geological Survey, Water Resources Data, South Dakota, annual reports, Water		Year 1997-98.

U.S. National Oceanic and Atmospheric Administration, 1985, Climate for South		Dakota, 1951-80.

Watertown Public Opinion, Weekly and Daily Newspapers, 1893, 1897, 1937, 1943.